3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310779

# Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Qualcomm Incorporated

Title:	Open issues for Discovery and Relay (re)selection

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses the open issues on discovery and Relay UE (re)selection.

# Discussion

### 2.1 Open issues on discovery and Relay (re)selection

Issue 1: Whether UE can trigger Relay selection when detecting direct link PC5-RLF

The principle is same as Relay UE reselection in which case we allow UE to trigger Relay reselection when directing RLF. So it should be reasonable to allow UE triggering Relay selection when detecting direct link PC5-RLF.

Proposal 1: The UE can trigger Relay selection when detecting direct link PC5-RLF.

Issue 2: Whether AS criterion is needed for switching from indirect link to direct link

A technical issue that ping-pong Relay UE (re)selection could happen if no AS criterion since RAN2 agreed that Relay selection could be triggered if the direct link quality is below a threshold and the link quality for the candidate Relay UE is above the threshold. Then it is reasonable to introduce AS criterion for switching from indirect link to direct link.

Proposal 2: AS criterion is needed for switching from indirect link to direct link.

### 2.2 Discovery and Relay (re)selection configuration

Discovery resource pool can be provided using dedicated configuration, but existing resource pool configuration is enough, and no new parameters to be introduced for discovery resource pool configuration.

Proposal 3: Reuse existing discovery resource pool configuration in dedicated signalling, i.e. no specific resource pool is configured for U2U discovery.

Existing parameter SL-ReselectionConfig-r17 is used to configured for U2N relay UE (re)selection threshold. For the link quality evaluation, there is no difference between U2N relay and U2U relay reselection, then it is proposed to reuse the existing parameter for U2N relay as U2U relay reselection configuration.

Proposal 4: Reuse existing U2N relay (re)selection configuration for U2U relay (re)selection.

Proposal 5: If no (re)selection configuration is provided in dedicated signalling, use the configuration broadcasted in SIB12; if out coverage of the interested frequency, use preconfiguration.

For The motivation for the discovery transmission SL threshold is to avoid the unnecessary discovery transmission when the Relay UE is not able to be a candidate Relay UE. So it should be reasonable to set the discovery transmission threshold to the same as Relay (re)selection threshold. And then the existing sl-RSRP-Thresh parameters in SIB12 can be reused for discovery transmission SL threshold.

Proposal 6:  For Model B response transmission in target Remote UE, the link quality threshold is same as Relay UE selection threshold.

Proposal 7: The same threshold is configured for Model A discovery announcement, Model B solicitation message and DCR message forwarding in U2U relay UE.

### 2.3 AS-NAS module for discovery message transmission

For the part of Relay UE and target Remote UE determining whether to transmit discovery message, there could be two alternatives:

Alt1: AS layer checks condition after receiving discovery message from upper layer. As you proposed in the CR, upper layer is not informed whether the sidelink condition is met and upper layer just blindly decodes and re-generates discovery message, and forwards to AS layer. And then AS layer will check whether to transmit discovery.

Alt2: AS layer checks condition before delivering discovery message to upper layer, as we proposed. In this alternative, when receiving discovery message, AS layer firstly checks whether transmission condition and deliver the message to upper layer whether.

Both alternatives could work, but Alt 1 wastes Relay UE and target Remote UE processing resource on decoding and re-generating discovery message which may not be transmitted in AS layer. So Alt 2 is more reasonable from our perspective.

Proposal 8: AS layer checks whether link quality is above the threshold before delivering discovery message to upper layer.

# Conclusion

This contribution discusses RAN2 part for UE-to-UE relay discovery and (re-) selection and provides the following proposals.

Open issues,

Proposal 1: The UE can trigger Relay selection when detecting direct link PC5-RLF.

Proposal 2: AS criterion is needed for switching from indirect link to direct link.

Configuration,

Proposal 3: Reuse existing discovery resource pool configuration in dedicated signalling, i.e. no specific resource pool is configured for U2U discovery.

Proposal 4: Reuse existing U2N relay (re)selection configuration for U2U relay (re)selection.

Proposal 5: If no (re)selection configuration is provided in dedicated signalling, use the configuration broadcasted in SIB12; if out coverage of the interested frequency, use preconfiguration.

Proposal 6:  For Model B response transmission in target Remote UE, the link quality threshold is same as Relay UE selection threshold.

Proposal 7: The same threshold is configured for Model A discovery announcement, Model B solicitation message and DCR message forwarding in U2U relay UE.

AS-NAS modeling,

Proposal 8: AS layer checks whether link quality is above the threshold before delivering discovery message to upper layer.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements