3GPP TSG-RAN WG2 Meeting #123bis	R2-2310794

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reselection and Access control for NES

WID/SID:	Netw_Energy_NR-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN2 is starting WI on NW energy saving (RP-223540) with one of the goals:

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

The TR 38.864 v18.0.0 captures this on the objective:

### 6.5.1	Cell selection/reselection

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

The definition of NES cell will be discussed in the WI phase.

The NW should be able to configure NES-capable UEs to prioritize/down-prioritize a specific NES cell or NES cells on a specific frequency. It is left to the WI phase whether the existing mechanism for cell (re)selection is sufficient according to the NES techniques specified.

From RAN2 perspective, legacy UEs and NES-capable UEs can be handled via cell selection/reselection techniques in the presence of NES cells.

In this paper we discuss the objective.

In RAN2#121 we reached following agreement:

Agreements:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

and in RAN2#122:

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

and in RAN2#123:

Agreements

-	One single bit in SIB1 is introduced for controlling all “NES-capable UEs” to access a cell.  FFS what “NES capable UE” bit means.  The NES UE always follows the NES bit used for barring, if present.  If not present the UE shall follow legacy barring.

-	No new cell baring techniques for non-NES UEs will be specified.

-	No new cell re-selection techniques will be considered in this Rel-18

# 2 	Motivation

Whenever the gNB (or one of the network entities it controls, such as a cell, a DU, a CU) operates in a network energy saving (NES) mode such as a dormant state, this entails that some of its hardware components are switched off or kept in a sleep mode to obtain network energy reduction. As a result, the capability of the gNB to provide services to the end users may be temporarily reduced until the energy saving mode is exited. Accordingly, there can be a trade-off between the energy to be saved and performance to be provided. For example, the provided performance during the NES mode (e.g.  in terms of bit rate, latency, number of served UEs, amount of data served, etc.) can be limited or degraded.

Observation 1: Cell operating in a NES mode will have reduced performance (e.g. in terms of bit rate, latency, number of served UEs, amount of data served, etc.)

To enable increased performance or revert to the normal performance level, the network may want to exit the NES mode. However, this comes at the price of an increased energy consumption.

Without any NES WI improvements, the main method for achieving energy saving is to turn off cells where capacity is not needed. This assumes that the operator has sufficient capacity on other frequency/NW layers. Such a behavior will be quite slow and mostly one would turn off cells when there is less demand for capacity, e.g., during the night. We consider that it is imperative to enable a fast way to activate NES modes in order to achieve energy savings when there is possibility to do so even if only lasting for short moments. This is similar to UE DRX when one only applies full capacity when there is need. So, the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signaling) without upper layer signaling (e.g. RRC signaling). This may cause issues how the upper layer signaling can control UEs – especially UEs in IDLE/INACTIVE.

Observation 2: NES WI should enable fast methods to turn on/off NES modes and also turning on/off the cell.

Thus, we see motivation to allow/disallow legacy UEs to camp on NES cells due to the limited capacity that NES activated cells can accommodate. Of course, this would mostly be applicable in a 5G network that has capacity to handle legacy UEs on other frequency layers too. So, a network could choose to control for example that some UEs will not camp at all on NES cells or then they could allow camping but limit the accesses to the system while operating in NES mode.

# 3	Barring

UE behaviour regarding in the draft 38.304 CR is captured currently as follows:

### 5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is ignored by UEs supporting NTN while cellBarredNTN is included in SIB1.

-	cellBarredNES (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs. This field is only applicable to NES-capable UEs.

-	cellBarredNTN (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is ignored if the UE does not support NTN connectivity.

-	cellBarredRedCap1Rx (IE type: "barred" or "not barred")
OMITTED PARTS

When cell broadcasts any CAG-IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not "true" for future use:

-	All NPN-capable UEs shall treat this cell as candidate during the cell selection and cell reselection procedures, other UEs shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for other use, and either cell does not broadcast any CAG-IDs or NIDs or does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for future use,

-	The UE shall treat this cell as if cell status is "barred".

When cellBarredNES is indicated as "barred",

-	The NES-capable UE shall treat this cell as if cell status is "barred".

When cellBarredNES is indicated as "not barred",

-	The NES-capable UE shall treat this cell as if cell status is "not barred".

When cellBarredNES is not broadcast in the cell,

-	The NES-capable UE shall follow the MIB cellBarred indication.

When cellBarredNTN is not broadcast in this cell, 
OMITTED PARTS

The text “When cellBarredNES is not broadcast in the cell,-	The NES-capable UE shall follow the MIB cellBarred indication” is really confusing text as now we write something that UE needs to obey specification as written and UE needs to do something it is configured with. This part of text should be removed altogether.

Proposal 1: Remove “When cellBarredNES is not broadcast in the cell,-	The NES-capable UE shall follow the MIB cellBarred indication”

As the cellBarred in MIB is used to bar legacy UEs, another mechanism needs to be introduced so that the NW bars also NES-capable UEs in case it intends to shut down the cell completely. Using a NES specific new cellBarredNES in SIB1 has been agreed in RAN2#123 . However, this requires that a SIB update procedure takes place, which in turn requires sending paging indicating SI update notification for a modification period, as well as then sending updated SIB for the next modification period. Barring the cell for the NES UEs with this process is not very power efficient from a NES point of view, while it also introduces delay for the NW to turn off the cell.

In order to speed up the cell reselection, we propose to indicate, e.g., in short message DCI (addressed to P-RNTI) to initiate cell reselection for all the UEs camping on the cell. After this indication, the NW can ensure that no UE camps on the cell and can turn off the cell without waiting the SI update procedure to complete.

Upon reception of the short message indicating NW initiated cell reselection, a UE that is camped on the cell considers the cell as barred and performs cell re-selection procedure.

Proposal 2: Introduce an indication not requiring SI update procedure to indicate the cell is being shut down and to trigger NES-capable UE to bar the cell and perform cell reselection

## 2.2 	Access control

As we can also have mixed deployments, then it becomes more complex to control UEs (NES capable or legacy UE) to camp on some cells on frequency. Especially NES capable UEs should be allowed to camp on NES cells but this could be problematic if large of amount of NES capable cells arrive to the NES utilizing cell simultaneously. Network would need to have control on how those UEs access the cell.

Currently, the network can configure limitations to the initial access using the UAC framework. The UAC is however defined to mitigate network congestion. It seems quite likely that when network is operating in NES mode it cannot handle efficiently many UEs and thus it becomes critical that network can effectively prevent new access in the cell. The UAC information is transmitted in system information (SI), and thus requires SI modification for any changes to be made to the access barring. This implies that currently the network cannot adjust the barring information dynamically and requires SI re-acquisition. And as the NES mode of gNB/network could change frequently (e.g. with PHY/MAC signaling) without upper layer signaling (e.g. RRC signaling), a slow update of access barring or cell barring can become difficult to manage.

Proposal 3: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.

# 3	Paging

## 3.1	Paging in subset of beams

RAN2 received LS R3-232084 regarding restricting paging in limited area where they state regarding possibly RAN2 impacts:

RAN3 is currently working on enhancements to restrict paging in a limited area as described in TR 38.864 clause 6.5.4, and has made the following agreements for RAN-initiated paging:

a)	It is gNB’s implementation to decide to which Ues the paging enhancement technique is applied.

b)	Introduce the recommended paging SSB list in the F1AP PAGING message sent from the gNB-CU to the gNB-DU.

c) Introduce the recommended paging SSB list in the F1AP UE CONTEXT RELEASE COMPLETE message sent from the gNB-DU to the gNB-CU.

According to above agreements, paging for Ues in RRC INACTIVE state may be restricted to a limited number of beams within a cell. RAN3 would respectfully like to ask RAN2 the following question:

Q1: Whether RAN3’s agreements have any impact on, or could be impacted by, TS 38. 304 clause 7 Paging, e.g., related to the following paragraph.

TS 38.304 clause 7.1:

“In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.”

In Incheon RAN2 meeting we have following outcome:

R2-2304627	LS on the enhancements to restricting paging in a limited area (R3-232084; contact: Nokia)	RAN3	LS in	Rel-18	Netw_Energy_NR-Core	To:RAN2, SA2

-	Nokia thinks that we should respond that there is no RAN2 impact.  Huawei agrees but thinks we should wait for SA2 respond.

-	Xiaomi is wondering if this solution works at all.  Qualcomm thinks that there are impact to RAN2.

=>	We will for SA2 to respond to RAN3

R2-2306067	Work plan for NR network energy savings	Huawei, HiSilicon	Work Plan	Rel-18	Netw_Energy_NR

=>	Noted

And now SA2 has provided their response in S2-2307984 with essentially following response:

Q2: RAN3 would respectfully like to ask SA2 if there are any issues from CN perspective with this mechanism.

Answer: SA2 does not see any showstoppers from CN perspective for the enhancements to restrict paging in a limited area for UEs in RRC IDLE. SA2 requests RAN3 to inform SA2 when the related CRs are agreed in order to proceed with alignment CRs in specifications under SA2 control.

SA2 would also like to validate the following assumption: If the initial paging attempt using the potential new parameter of "List of recommended beams" fails, how paging escalation happen will be controlled by the AMF based on existing procedures.

and ask the following questions:

Q1: SA2 understands that the beam-based paging is most efficient towards low mobility and static UEs. How is the gNB expected to determine whether the UE is static or not across multiple RRC connections from the same UE considering TS 33.501 contains requirements on 5G-S-TMSI reallocation?

Q2: SA2 would like to ask about the validity condition of the potential “List of recommended beams” container that is stored in the AMF. What is the AMF condition to delete the stored "List of recommended beams"?

Then considering also 38.304- UE assumes that UE may try to receive paging basically from any beam – naturally UE won’t be trying to receive from any beam it won’t be able to receive with sufficient performance. So, in order to limit paging to a subset of beams, the NW would need to have reasonable understanding which beams UEs are able to receive. Or alternatively, the NW can start paging in a subset of beams but if the UE does not respond to paging, it would need to start sending paging in wider area e.g. every beam or even multiple gNB etc. It is our understanding though that we cannot change UE behaviour - at least not legacy UE behaviour. So at least part of the population of UEs will always assume paging will be same in each of the beams. But on the other hand we don’t see issue from UE point of view if paging content is different as long as NW ensures at some point NW pages in proper beam(s) so that UE can receive the paging.

Observation 3: UEs may continue to assume same paging will be sent on each beam

Such a behaviour will likely add some delay in the paging provision for the UE, if the initial guess of beams where UE is located is not correct. If the NW starts paging in subset of beams and if the UE does not respond to paging, the NW would need to start sending paging in wider area, e.g., every beam, and this would cause delay (at least length of paging cycle). Also, the amount of signaling is increasing, because NW needs to send paging again, if the UE is not responding in the first attempt. Also, it is not really RAN2 realm to assess how this kind of repetition is feasible form RAN3/SA2 point of view.

Proposal 4: Respond to RAN3 that RAN2 sees no need to impact the clause in 7.1 of 38.304 regarding paging in subset of beams but highlight to them that this may cause UEs to miss paging if wrong subset of beams is selected by the network.

## 3.2	Avoiding paging sending in the POs during cell DTX/DRX

According to the RAN2#121 agreements, there should be no impact to paging for legacy and RRC Idle/Inactive UEs because of cell DTX/DRX. Although no explicit agreement has been done on supporting the paging reception in RRC Connected during cell DTX/DRX, it can be assumed that the network has to transmit the short message (that may contain ETWS and CMAS notifications) in the paging DCI for UEs in any RRC state, so also in RRC Connected.

Currently, the UE in the RRC Connected state has some freedom to monitor for the short message in any paging occasion (at least) once every defaultPagingCycle. Therefore, the RRC Connected UE can then also assume that the short message will continue to be available in any Paging Occasion independently of the cell DTX state.

As discussed earlier in this contribution, we see it beneficial to sometimes prevent legacy UE / non NES UE camping on a cell. In such a scenario, it would be desirable for the cell to omit the transmission of the short message during the POs which occur in cell DTX non-active time to further increase the energy saving potential. Thus, we would like to propose RAN2 to discuss whether it would be feasible for NES UEs in RRC_CONNECTED mode only to try to monitor paging in POs which fall in to Cell DTX active time.

Proposal 5: Make it possible for network to avoid sending paging/short messages in POs outside Cell DTX/DRX.

# 4	Conclusion

Observation 1: Cell operating in a NES mode will have reduced performance (e.g. in terms of bit rate, latency, number of served UEs, amount of data served, etc.)

Observation 2: NES WI should enable fast methods to turn on/off NES modes and also turning on/off the cell.

Proposal 1: Remove “When cellBarredNES is not broadcast in the cell,-	The NES-capable UE shall follow the MIB cellBarred indication”

Proposal 2: Introduce an indication not requiring SI update procedure to indicate the cell is being shut down and to trigger NES-capable UE to bar the cell and perform cell reselection

Proposal 3: RAN2 should study how the network can control accesses in the NES utilizing gNB effectively.

Observation 3: UEs may continue to assume same paging will be sent on each beam

Proposal 4: Respond to RAN3 that RAN2 sees no need to impact the clause in 7.1 of 38.304 regarding paging in subset of beams but highlight to them that this may cause UEs to miss paging if wrong subset of beams is selected by the network.

Proposal 5: Make it possible for network to avoid sending paging/short messages in POs outside Cell DTX/DRX.