3GPP TSG-RAN2 Meeting #123bis	R2-2310809

Xiamen, China, 9th - 13th October 2023

Agenda item:	7.20.2

Source:	Qualcomm Incorporated

Title:	Discussion on multi-DCI multi-TRP with two TAs

WID/SID:	NR_MIMO_evo_DL_UL-Core – Release 18

Document for:	Discussion and Decision

# Introduction

RAN2 #123 meeting made several agreements related to the multi-DCI multi-TRP with two TAs.

At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells. At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell). Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

In RAN2 #122 meeting, the multi-DCI multi-TRP with two TAs was discussed and the following agreement was made.

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

In this paper, we would like to further discuss the remaining issues of multi-DCI multi-TRP with two TAs.

# Discussion

According to RAN2 #123 meeting agreement, two PTAGs will be modeled as working assumption. If SpCell is configured with multi-DCI multi-TRP with two TAs, the two TAGs in SpCell are PTAGs. In case the two TATs associated with the two PTAGs are both expired, the UE behavior should be the same as the legacy when TAT associated with the PTAG is expired.

Proposal 1: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TATs associated to the two PTAGs are both expired, UE behavior is the same as the legacy when TAT associated with PTAG is expired.

For the SCell configured with two TAGs, the two TAGs could be two STAG or PTAG + STAG. In case the two TATs associated with two STAGs are both expired, then for all serving cell belonging to these two STAGs, the UE behavior should be the same as the legacy when TAT associated with the STAG is expired. In case the TATs associated with PTAG + STAG in one SCell, assuming the TAT of the other PTAG is still running (otherwise, the assumption of P1 is applied), for such SCell, i.e., including PTAG + STAG, the UE behavior should be the same as the legacy when TAT associated with STAG is expired and the behavior is applied to two TAGs with TAT expired of all serving cells which includes these two TAGs.

Proposal 2: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TATs associated to the two TAGs in SCell are both expired, UE should follow the behavior as legacy TAT associated with STAG is expired and the behavior is applied to the two TAGs with TAT expired of all serving cells including both these two TAGs.

Then the other case is that the TAT associated to one TAG is expired and while the other TAT of the other TAG in one serving cell is still running. For this case, RAN2 #123 meeting agreed to not perform any uplink transmission to the TAG with TAT expired, and to maintain NTA of this TAG. There should be no impact to the TAG with TAT running.

Regarding flushing all HARQ buffers of the serving cells in the legacy UE behaviors when TAT associated with TAG is expired, it may not be needed in the two TA cases. Since there are still one TAT associated to one TAG in one serving cell running, UE can retransmit the buffered data via the workable uplink (TAT running). It is beneficial for UE to keep uplink data rate without interruption in one serving cell and to further improve the reliability. Otherwise, the data stored in the uplink in the TAG whose TAT is expired has to be dropped hence causing data loss. However, whether UE can support the data to be scheduled via another uplink TRP depends on UE capability.

Rel-16 multi-DCI multi-TRP has defined a capability, supportRetx-Diff-CoresetPool-Multi-DCI-TR. If this capability is reported, UE does not support retransmission scheduled by PDCCH received in a different CORESETPoolIndex compared to the CORESETPoolIndex of the initial transmission. The description of this capability states that UE is not expected to receive, for the same HARQ process ID, DCI from a different CORESETPoolIndex that schedule the retransmission, and this applies to the both PDSCH and PUSCH transmission. Therefore, if UE does not report this capability, the HARQ buffers can not be flushed.

Proposal 3: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TAT for one TAG is expired and while the other TAT of the other TAG is still running for a serving cell, for all serving cells including this TAG with TAT expired and in which the TAT of the other TAG is still running, whether the HARQ buffers associated to this TAG with TAT expired can be flushed or not depends on the UE capability.

Then for those other legacy UE behaviors of TAT expiry, they can be applied to the TAG with TAT expired for serving cells which includes this TAG with TAT expired and in which the TAT of the other TAG is still running.

Proposal 4: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TAT for one TAG is expired and while the other TAT of the other TAG is still running for a serving cell, for all serving cells including this TAG with TAT expired and in which the TAT of the other TAG is still running, the legacy UE behaviors of TAT expiry including (1) notify RRC to release PUCCH; (2) notify RRC to release SRS; (3) clear any configured downlink assignments and configured uplink grants; (4) clear any PUSCH resource for semi-persistent CSI reporting are applied.

There could be another case that SpCell is configured with single TRP and one PTAG is modelled in SpCell as legacy, and SCell is configured with multi-DCI multi-TRP with two TAs, and two TAGs are modelled in SCell. In this case, if the TAT associated with PTAG is expired, and even one TAT associated to the TAG is expired and while the other TAT associated to the other TAG in SCell is still running, UE should follow the legacy behavior of TAT associated with PTAG is expired.

Proposal 5: If SpCell is configured with single TRP and SCell is configured with multi-DCI multi-TRP with two TAs, and two TAGs are modelled in SCell, if the TAT associated with PTAG is expired, and even one TAT associated to the TAG is expired and while the other TAT associated to the other TAG in SCell is still running, UE should follow the legacy behavior of TAT associated with PTAG is expired.

How to acquire the TA for the second TAG of one serving cell should be studied. In current NR, TA of STAG is updated by the PDCCH ordered CFRA for the SCell in CA case, and network will require UE to initiate RACH by PDCCH order for TA acquisition of the target SCell.

For multi-DCI multi-TRP with two TAs, the second TAG update could be similar to the method of TA update for the SCell in CA case. For the RRC connected UE, if second TRP of one serving cell is configured by network, network schedules the UE by PDCCH order to trigger CFRA and UE acquires the TA for the synchronization with the second TRP.

Proposal 6: The initial TA update for the second TAG is initiated by network using PDCCH order triggered CFRA procedure.

For inter-cell multi-DCI multi-TRP, RAN1 has achieved following conclusion.

Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

According to RAN1 agreement, it can be observed that additional RACH configuration shall be configured for the cell with additional PCI and UE can perform RACH triggered by PDCCH order for the corresponding configured additional PCI.

Observation 1: For inter-cell multi-DCI multi-TRP, UE performs RACH triggered by PDCCH order associated with the configured additional PCI.

Since the RACH resource triggered by PDCCH order is associated with the configured additional PCI, UE acquires the updated TA from the triggered RACH procedure and there is no ambiguity for UE to know the TA is updated for which TAG. Thus, for inter-cell multi-DCI multi-TRP case, we have following proposal.

Proposal 7: For inter-cell multi-DCI multi-TRP, the TA is updated for the TAG from the triggered RACH which is associated to the RACH resource configuration, i.e., from serving cell or associated with the additional PCI.

For intra-cell multi-DCI multi-TRP in SpCell, if UE is configured with two TAGs and maintains two TAs, the legacy UE-initiated CBRA can be triggered due to certain events, such as when DL or UL data arrival and meanwhile the UL is un-sync, or BFR triggering and so on. In the legacy UE-initiated CBRA, UE evaluates SSB and selects RACH occasions associated with the qualified SSB. Since SSB is not configured to associate with the two TRPs for intra-cell case, UE does not know which TRP the RACH towards. For Msg2/MSGB reception, network should indicate the TA to be applied to which TAG for intra-cell case. Therefore, for intra-cell case, RAN2 should study the enhancement on the TA update to be applied for which TAG.

Proposal 8: RAN2 to study the enhancement on the TA update to be applied for which TAG for intra-cell multi-DCI multi-TRP with two TA case.

In RAN1 #112bis-e meeting, the following conclusion was made.

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated

It should be noted that the absolute TAC MAC CE is not supported in PDCCH order triggered RACH in current spec. So, the intention of this RAN1 agreement should be for the UE-initiated RACH. When UE initiates 2-step RACH, network should indicate the TAG ID via absolute TA command MAC CE to inform UE the TA update in MAC CE applied for which TAG.

From RAN2 perspective, it simply reuses one reserve bit in absolute TA command MAC CE indicating which TAG the absolute TA command is applied.

Observation 2: RAN1 has agreed that for two TA for multi-DCI multi-TRP, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2.

Proposal 9: One reserve bit in absolute TA command MAC CE is used to indicate which TAG the absolute TA command is applied.

For 4-step RACH, RAN1 #114 meeting made the following agreement.

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.

Based on the RAN1 agreement, for intra-cell multi-DCI multi-TRP with two TAs, at least for PDCCH order CFRA, the TAG index (either the first TAG ID or the second TAG ID for the serving cell) is indicated in RAR along with the updated TA command. Further, RAN2 #123 meeting made following agreement as baseline.

The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

Based on baseline RAN2 agreement, for intra-cell case, the same mechanism agreed for PDCCH order CFRA can be reused for CBRA case. We prefer to have a unified solution for both PDCCH order CFRA and UE-initiated CBRA, and it seems no technical issue to introduce the TAG indication in RAR. Thus, we have following proposal.

Proposal 10: For the intra-cell multi-DCI multi-TRP with two TAs, the reserve bit in RAR is enhanced to indicate which TAG (the first TAG or the second TAG) the TA in RAR is applied.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For inter-cell multi-DCI multi-TRP, UE performs RACH triggered by PDCCH order associated with the configured additional PCI.

Observation 2: RAN1 has agreed that for two TA for multi-DCI multi-TRP, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2.

Proposal 1: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TATs associated to the two PTAGs are both expired, UE behavior is the same as the legacy when TAT associated with PTAG is expired.

Proposal 2: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TATs associated to the two TAGs in SCell are both expired, UE should follow the behavior as legacy TAT associated with STAG is expired and the behavior is applied to the two TAGs with TAT expired of all serving cells including both these two TAGs.

Proposal 3: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TAT for one TAG is expired and while the other TAT of the other TAG is still running for a serving cell, for all serving cells including this TAG with TAT expired and in which the TAT of the other TAG is still running, the HARQ buffers associated to this TAG with TAT expired may not be flushed given the UE capability.

Proposal 4: If SpCell is configured with multi-DCI multi-TRP with two TAs, and two PTAGs are modelled in SpCell, when TAT for one TAG is expired and while the other TAT of the other TAG is still running for a serving cell, for all serving cells including this TAG with TAT expired and in which the TAT of the other TAG is still running, the legacy UE behaviors of TAT expiry including (1) notify RRC to release PUCCH; (2) notify RRC to release SRS; (3) clear any configured downlink assignments and configured uplink grants; (4) clear any PUSCH resource for semi-persistent CSI reporting are applied.

Proposal 5: If SpCell is configured with single TRP and SCell is configured with multi-DCI multi-TRP with two TAs, and two TAGs are modelled in SCell, if the TAT associated with PTAG is expired, and even one TAT associated to the TAG is expired and while the other TAT associated to the other TAG in SCell is still running, UE should follow the legacy behavior of TAT associated with PTAG is expired.

Proposal 6: The initial TA update for the second TAG is initiated by network using PDCCH order triggered CFRA procedure.

Proposal 7: For inter-cell multi-DCI multi-TRP, the TA is updated for the TAG from the triggered RACH which is associated to the RACH resource configuration, i.e., from serving cell or associated with the additional PCI.

Proposal 8: RAN2 to study the enhancement on the TA update to be applied for which TAG for intra-cell multi-DCI multi-TRP with two TA case.

Proposal 9: One reserve bit in absolute TA command MAC CE is used to indicate which TAG the absolute TA command is applied.

Proposal 10: For the intra-cell multi-DCI multi-TRP with two TAs, the reserve bit in RAR is enhanced to indicate which TAG (the first TAG or the second TAG) the TA in RAR is applied.

# References

# Annex