3GPP TSG-RAN2 Meeting #123bis	R2-2310810

Xiamen, China, 9th - 13th October 2023

Agenda item:	7.18.2

Source:	Qualcomm Incorporated

Title:	Discussion on NCD-SSB for MT-SDT

WID/SID:	NR_MT_SDT-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In this paper, we would like to discuss the issue of NCD-SSB for MT-SDT.

# Discussion

In Rel-17 MO-SDT, a RedCap UE can trigger MO-SDT when RedCap-specific initial downlink BWP includes CD-SSB, or when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured. The reason RAN2 made this agreement was because RedCap UE has to measure the RSRP of DL RS to validate the use of each CG-SDT occasion, and for the subsequent data transfer phase of RA-SDT, RedCap UE has to perform link maintenance such as time synchronization using the DL RS. Therefore, either CD-SSB or NCD-SSB included in the RedCap-specific initial downlink BWP is one of the conditions for triggering MO-SDT.

For Rel-18 MT-SDT, RedCap UE uses RACH or CG-SDT resource to initiate resume procedure in response to RAN paging message. In MT-SDT, similar requirement is applied to RedCap UE, i.e., monitor DL RS for validation of each CG-SDT occasion and perform link maintenance using the DL RS in the RA-SDT subsequent data phase. The requirement for a MT-SDT on the DL RS configuration should be same as the conditions of MO-SDT. Therefore, we believe this condition of MO-SDT should be used to that of MT-SDT, i.e., a RedCap UE can trigger MT-SDT when ReCap-specific initial downlink BWP includes no CG-SSB but with NCD-SSB configured.

For the NCD-SSB resource, the ncd-SSB-RedCapInitialBWP-SDT introduced for Rel-17 MO-SDT can be extend to MT-SDT. Network should configure ncd-SSB-RedCapInitialBWP-SDT if a RedCap UE is configured with SDT in the RedCap-specific initial DL BWP not associated with CD-SSB.

Proposal 1: The conditions for a RedCap UE triggering MT-SDT should include when RedCap-specific initial BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured.

In Rel-18, eRedCap is studied. Based on the eRedCap capability framework, the specification for a UE supporting RedCap UE also apply for a UE supporting the eRedCap unless stated otherwise. eRedCap UE will use the RedCap specific initial BWP and no more eRedCap specific initial BWP is introduced. Given eRedCap UE may have lower capabilities and eRedCap UE also uses the RedCap specific initial BWP, the similar logic on the usage of NCD-SSB for SDT configured in RedCap specific initial BWP should be applied to eRedCap so that eRedCap UE can perform SDT in the RedCap specific initial BWP with at least one type of SSB.

Proposal 2: The condition for a RedCap UE triggering SDT about RedCap-specific initial BWP including either CD-SSB or NCD-SSB should be applied for eRedCap UE.

Regarding the existing UE capability, ncd-SSB-ForRedCapInitialBWP-SDT-r17, it indicates that whether RedCap UE supports using RedCap-specific initial DL BWP associated with NCD-SSB for SDT. Otherwise (if absent), RedCap UE only supports SDT in an initial DL BWP that include the CD-SSB. Considering the discussions for P1, we believe such capability is not only applied for the MO-SDT but should be extended to be applied for MT-SDT, i.e., mt-SDT-r18 or mt-CG-SDT-r18. Thus, we have following proposal.

Proposal 3: The Rel-17 ncd-SSB-ForRedCapInitialBWP-SDT-r17 UE capability should be extended to apply for MT-SDT.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The conditions for a RedCap UE triggering MT-SDT should include when RedCap-specific initial BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured.

Proposal 2: The condition for a RedCap UE triggering SDT about RedCap-specific initial BWP including either CD-SSB or NCD-SSB should be applied for eRedCap UE.

Proposal 3: The Rel-17 ncd-SSB-ForRedCapInitialBWP-SDT-r17 UE capability should be extended to apply for MT-SDT.

# References

# Appendix