3GPP RAN WG2 Meeting #123bis	R2-2310844

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.7.4.2

Source:	InterDigital

Title:	NTN mobility enhancements for RRC_CONNECTED

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Work will consider existing methods from NR TN and Rel-17 NR NTN as baseline for further enhancements. The following agreements have been made regarding handover enhancements in Rel-18 NTN. FFS topics which are still open are highlighted in yellow.

From agreements in RAN2#121 [1], the following aspect is still open:

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

From agreements in RAN2#121bis-e [2], the following aspect is still open:

- FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell.

From agreements in RAN2#122 [3], the following aspect is still open:

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

From agreements in RAN2#123 [6], the following aspect is still open:

- The MAC entity applies the N_TA (value 0 or same as source cell) configured in the RACH-less HO command for the PTAG. FFS on when timerAlignmentTimer associated with this TAG starts

Furthermore, the following issues related to RACH-less HO have been identified by the work item Rapporteur as needing to be addressed:

- Optionally, Let’s check time-based CHO combining with RACH-less

- MAC aspect : Whether preallocated grant can be configured without ntn-RSRP-ThresholdSSB

Note: aspects related to same PCI switch can be found in a companion contribution [4]

# Mobility enhancements: Remaining open issues

## Candidate cell reference location derivation for location-based CHO in earth-moving cell

Location-based measurement initiation is supported for earth-moving cells re-using the measurement rules from the quasi-earth fixed cells (defined in Rel-17). One difference is the Serving Cell reference location moves continuously over earth. To compensate, the UE roughly derives the reference location trajectory based on the serving satellite ephemeris and epochTime.

Location-based CHO requires the UE to monitor its distance to both the Serving Cell and Candidate Cell reference locations. For earth-moving cells a similar solution as above is used to derive the Serving Cell reference location, however it is currently FFS whether the same mechanism can be used for the candidate cell’s reference location. The three components needed to support reference location derivation are: 1) the reference location at epochTime; 2) epochTime; and 3) the satellite ephemeris. Assuming this is information is available and up-to-date for a candidate cell, it seems the same procedure can be used for candidate cell reference location derivation.

Proposal 1: 	For location-based CHO for earth-moving cells, re-use the procedure from cell reselection to derive the candidate cell’s reference location as the cell moves.

## RACH-less HO failure handling

Based on RAN1 requirements from [5], at the time of RACH-less HO execution the UE has valid assistance information and will have performed necessary pre-compensation procedures to access the target cell. In this case RACH failure would more likely be due to radio-link issues rather than time/frequency pre-compensation.

If the radio-link issues are severe, falling back to RACH-based HO would not do much, and could increase recovery delay due to the large RTT. Legacy RACH-less HO already supports failure recovery via the RRC-Reestablishment procedure, which is also agreed as part of the NR procedure. This seems sufficient to resolve RACH-less HO failure without introducing additional delay by first attempting RACH.

Proposal 2: 	Upon RACH-less HO failure, UE does not fallback to RACH-based HO.

## Enhancements to RACH-less HO completion confirmation

In RACH-less HO, after transmitting the RRCReconfigurationComplete in the initial UL transmission the UE monitors for the UE Contention Resolution Identity MAC CE to confirm HO success, ignoring the contents if received. This is the same procedure used in LTE and applies to both dynamic and preallocated grant case.

Since the UE ignores the MAC CE contents, it is essentially a wasted transmission. One possibility is to use any general PDCCH/PSDCH addressed to the UE’s C-RNTI as confirmation. Since both transmissions are addressed to the UE’s C-RNTI, it would essentially accomplish the same thing, but allow the network to transmit meaningful data. It should be noted that this is under the assumption that the network has DL data available. In the case that no data is available, the network can simply re-use the existing solution of transmitting a Contention Resolution MAC CE.

Proposal 3: 	NW can send PDCCH/PDSCH addressed to C-RNTI to confirm RACH-less HO completion instead of UE Contention Resolution MAC CE (e.g., if DL data is available).

## Time alignment timer handling for RACH-less

In LTE the timeAlignmentTimer associated with PTAG is started once the MAC entity applies the N_TA configured in the RACH-less HO command. An additional step is included in NTN since prior to starting timeAlignmentTimer, the UE must first achieve UL/DL synchronization (e.g., acquire SIB19, perform time/frequency pre-compensation etc.) and start T430.

In the current specification, upon completion of UL synch RRC sends an indication to MAC which triggers the MAC entity to allow UL transmission on the Serving Cell. The same indication can also be used by the UE in the RACH-less HO case to know when to start the associated timeAlignmentTimer.

Proposal 4: 	UE starts the PTAG timeAlignmentTimer upon indication from upper layers (i.e., RRC) that UL synchronization is obtained for the target cell during RACH-less HO.

## Time-based CHO combined with RACH-less HO

In RAN2#121bis-e it was agreed that combination of RACH-less HO with time-based CHO can be considered for NTN. A benefit is, if the RACH-less HO handover is restricted to within a time window T1 to T1+duration, then the network can limit the amount of time if needs to reserve resources on the target cell for the initial transmission (i.e., the preallocated grant can be considered valid only within the CHO time window).

However, this relies on the time window being configured with a reasonably short duration, otherwise the network may waste significant resources supported a preallocated grant for a long CHO time window. If the initial transmission is via dynamic grant, then additional discussion is needed to determine when/how to provide the grant.

Although there is some benefit to combining RACH-less HO with CHO, it seems difficult to conclude considering limited time remaining in the release. It is suggested that this feature be postponed to Rel-19, and possibly be addressed by another work item (e.g., feMobility).

Proposal 5: 	Combination of RACH-less HO with time-based CHO is not supported in Rel-18 NTN.

## Configuring pre-allocated UL grant without a threshold

To support SSB selection for initial transmission after RACH-less HO, an RSRP threshold ntn-RSRP-ThresholdSSB is configured for preallocated uplink grant. The selection procedure is similar to CG-SDT, where if there is no suitable SSB above the threshold the configured grant is considered as not valid.

An open issue from the running CR review is whether preallocated grant can be configured without an associated RSRP threshold (i.e., the UE may select any SSB associated with the preallocated grant regardless of quality). To avoid increased probability of transmission failure by selecting a poor or sub-optimal SSB, it is better the preallocated UL grant always be configured with an RSRP threshold. This is in-line with CG-SDT design (which is agreed as the baseline for preallocated grant).

Proposal 6: 	Preallocated UL grant must be configured with an associated RSRP threshold.

# Conclusion

In this contribution the following observations and proposals were made concerning RRC_CONNECTED measurement, mobility, and service continuity enhancements in Rel-18 NR NTN:

Proposal 1: 	For location-based CHO for earth-moving cells, re-use the procedure from cell reselection to derive the candidate cell’s reference location as the cell moves.

Proposal 2: 	Upon RACH-less HO failure, UE does not fallback to RACH-based HO.

Proposal 3: 	NW can send PDCCH/PDSCH addressed to C-RNTI to confirm RACH-less HO completion instead of UE Contention Resolution MAC CE (e.g., if DL data is available).

Proposal 4: 	UE starts the PTAG timeAlignmentTimer upon indication from upper layers (i.e., RRC) that UL synchronization is obtained for the target cell during RACH-less HO.

Proposal 5: 	Combination of RACH-less HO with time-based CHO is not supported in Rel-18 NTN.

Proposal 6: 	Preallocated UL grant must be configured with an associated RSRP threshold.

# References

R2-2302401 – RAN2#121 Meeting Report

R2-2306553 – RAN2#121bis-e Meeting Report

R2-2307001 – RAN2#122 Meeting Report

R2-2310845 – Satellite switching without PCI change – InterDigital

R2-2300020 - Reply LS on RACH-less handover in NTN –  RAN1

Draft_R2_123_meeting_report_v1