3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310899

# Xiamen, China, October 9th – 13th October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | DraftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the number of beam updates in the NCR Access Link Beam Indication MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NetConRepeater |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has agreed in RAN2 #121-bis-e that RAN2 will not support update of partial beam indexes in NCR Access Link Beam Indication MAC CE. In CR 38.321 (R2-2309052), for a resource set with ID Resource set ID, the beams with indices Beam index IDi, index i ranging from 0 to N-1, can be updated. However, N is not further described nor its value set. Based on the RAN2 agreement, N is equal to the number of the beam indications in the resource set with ID Resource set ID. This should be reflected in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 6.1.3.u - Clarified that N, the number of updated beams in the MAC CE is equal to the number of the beam indications in the resource set with ID Resource set ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, it would not be possible to know the number of updated beam indication in the NCR Access Link Beam Indication MAC CE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the agreed CR for NCR in R2-2309052 with original track changes as accepted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 6.3.2	Radio resource control information elements

*** Unchanged parts are omitted ***

#### 6.1.3.u	NCR Access Link Beam Indication MAC CE

The NCR Access Link Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields (Figure 6.1.3.u-1):

-	Resource set ID: This field is used to indicate one of forwarding semi-persistent resource lists signalled in NCR-SemiPersistentFwdResourceSet (as specified in TS 38.331 [5]). The field contains a list ID (comprising all 5 bits);

-	A/D: If the value of this field is set to 1, the forwarding resource list indicated in Resource list ID field is being activated. If the value of this field is set to 0, the forwarding resource list indicated in Resource list ID field is being deactivated;

- 	C: If the value of this field is set to 1, the Beam index IDi field is present. If the value of this field is set to 0, the Beam index IDi field is absent. This field can be set to 1 only if MAC CE is used for activation, i.e., when the A/D field is set to 1. If MAC CE is used for deactivation, this field is set to 0;

- 	Beam index IDi: This field indicates the updated beam index for forwarding resources within the list indicated by the Resource list ID field. Beam index ID0 indicates the beam index for the first forwarding resource within the list; Beam index ID1 indicates the beam index for the second forwarding resource within the list and so on. The length of the field is 6 bits; the number of updated beams, N, is equal to the number of the beam indications in the semi-persistent resource set with ID Resource set ID;

-	R: Reserved bit, set to 0.

END OF CHANGES