3GPP TSG-RAN WG2 Meeting #123bis	R2-2310916

Xiamen, China, October 9 – 13, 2023

Agenda item:	7.2.5

Source: 	Qualcomm Incorporated

Title: 	Configuration Enhancements for DL-PRS Aggregation

Document for:		Discussion and Decision

# 1.	Introduction

The Work Item on "Expanded and Improved NR Positioning" includes the following objective [1]:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

The impacts of the RAN1 agreements on RAN2 specifications have been discussed/summarized in the email discussion "[Post123][402][POS]" [2].

However, the details for the on-demand DL-PRS for bandwidth aggregation are up to RAN2 and are discussed in this contribution:

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

The RAN1 outcome/agreements are captured in TS 38.214 [3]:

When the UE is expected to perform joint measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameter [linkage], between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of QCL, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceSymbolOffset, dl-prs-MutingBitRepetitionFactor, dl-PRS-CyclicPrefix, comb size, power per subcarrier, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE may assume that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE assumes phase continuity on the DL PRS resources; otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation. The UE may be configured to measure and report, subject to UE capability, [up to XX] joint DL RSTD measurement(s) per pair of dl-PRS-ID, from aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may be configured to measure and report, subject to UE capability, [up to YY] joint UE Rx-Tx time difference measurement(s) from aggregated DL PRS resources across two or three DL PRS positioning frequency layers. The UE may be requested via higher layer parameter [positioning frequency layer aggregation indication] to perform the joint DL RSTD measurement(s) or the joint UE Rx-Tx time difference measurement(s) across two or three DL PRS positioning frequency layers. The UE may report via higher layer parameter [positioning frequency layer aggregation information] which indicates if bandwidth aggregation is performed and which two or three DL PRS positioning frequency layers are used for the joint DL RSTD measurement(s) and the joint UE Rx-Tx time difference measurement(s). In a measurement report, the UE may report PRS resource set IDs across the two or three DL PRS positioning frequency layers used to perform the joint DL RSTD measurement or the joint UE Rx-Tx time difference measurement.

# 2.	NR-DL-PRS-AssistanceData for DL-PRS aggregation

According to the RAN1 agreements summarized in [2], the UE may be configured with "linkage" information which indicates the DL-PRS Resource Sets across DL-PRS positioning frequency layers available for aggregation. A DL-PRS Resource Set in the IE NR-DL-PRS-AssistanceData can be uniquely identified by its dl-PRS-ID and NR-DL-PRS-ResourceSetID, requiring 8+3=11 bits. Alternatively, a DL-PRS Resource Set in the IE NR-DL-PRS-AssistanceData can be identified by its PFL-Index, TRP-Index, and Resource Set Index in the IE NR-DL-PRS-AssistanceData, requiring 2+6+1=9 bits. Although, there is no significant difference in the number of required bits, an indexing scheme would be consistent, for example, with the DL-PRS Assistance Data used in IEs method-ProvideAssistanceData (with 'method' = NR-DL-TDOA, NR-DL-AoD, NR-Multi-RTT). The "linked" DL-PRS Resource Sets can be indicated directly in the IE NR-DL-PRS-AssistanceData as shown below.

In [4], RAN1 requested to capture the condition of the same TRP in RAN2 specifications for PRS bandwidth aggregation:

"For PRS bandwidth aggregation, RAN1’s agreement is that the linked PRS resource sets from two or three PFLs should be from the same TRP. RAN1 kindly requests RAN2 to capture the condition of the same TRP in RAN2 specifications for PRS bandwidth aggregation." [4]

which is proposed to be caputure in a Table Note below.

#### – NR-DL-PRS-AssistanceData The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data. […] -- ASN1START NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { nr-DL-PRS-ReferenceInfo-r16 DL-PRS-ID-Info-r16, nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16, nr-SSB-Config-r16 SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF NR-SSB-Config-r16 OPTIONAL, -- Need ON ..., [[ nr-DL-PRS-AggregationInfo-r18 NR-DL-PRS-AggregationInfo-r18 OPTIONAL -- Need ON ]] } NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE { nr-DL-PRS-PositioningFrequencyLayer-r16 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16, ... } NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-DL-PRS-SFN0-Offset-r16 NR-DL-PRS-SFN0-Offset-r16, nr-DL-PRS-ExpectedRSTD-r16 INTEGER (-3841..3841), nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 INTEGER (0..246), nr-DL-PRS-Info-r16 NR-DL-PRS-Info-r16, ..., [[ prs-OnlyTP-r16 ENUMERATED { true } OPTIONAL -- Need ON ]], [[ nr-DL-PRS-ExpectedAoD-or-AoA-r17 NR-DL-PRS-ExpectedAoD-or-AoA-r17 OPTIONAL -- Need ON ]] } NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE { dl-PRS-SubcarrierSpacing-r16 ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...}, dl-PRS-ResourceBandwidth-r16 INTEGER (1..63), dl-PRS-StartPRB-r16 INTEGER (0..2176), dl-PRS-PointA-r16 ARFCN-ValueNR-r15, dl-PRS-CombSizeN-r16 ENUMERATED {n2, n4, n6, n12, ...}, dl-PRS-CyclicPrefix-r16 ENUMERATED {normal, extended, ...}, ... } NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE { sfn-Offset-r16 INTEGER (0..1023), integerSubframeOffset-r16 INTEGER (0..9), ... } NR-DL-PRS-ExpectedAoD-or-AoA-r17 ::= CHOICE { expectedAoD-r17 SEQUENCE { expectedDL-AzimuthAoD-r17 INTEGER (0..359), expectedDL-AzimuthAoD-Unc-r17 INTEGER (0..60) OPTIONAL, -- Need OP expectedDL-ZenithAoD-r17 INTEGER (0..180), expectedDL-ZenithAoD-Unc-r17 INTEGER (0..30) OPTIONAL -- Need OP }, expectedAoA-r17 SEQUENCE { expectedDL-AzimuthAoA-r17 INTEGER (0..359), expectedDL-AzimuthAoA-Unc-r17 INTEGER (0..60) OPTIONAL, -- Need OP expectedDL-ZenithAoA-r17 INTEGER (0..180), expectedDL-ZenithAoA-Unc-r17 INTEGER (0..30) OPTIONAL -- Need OP } } NR-DL-PRS-AggregationInfo-r18 ::= SEQUENCE (SIZE (1..256) OF NR-DL-PRS-AggregationIndexList-r18 NR-DL-PRS-AggregationIndexList-r18 ::= SEQUENCE (SIZE (2..3) OF NR-DL-PRS-AggregationElement-r18 NR-DL-PRS-AggregationElement-r18 ::= SEQUENCE { nr-DL-PRS-FrequencyLayerIndex-r18 INTEGER (0..nrMaxFreqLayers-1-r16), nr-DL-PRS-TRP-Index-r18 INTEGER (0..nrMaxTRPsPerFreq-1-r16), nr-DL-PRS-ResourceSetIndex-r18 INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16) } -- ASN1STOP NR-DL-PRS-AssistanceData field descriptions. nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.. nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.. nr-SSB-Config This field specifies the SSB configuration of the TRPs.. nr-DL-PRS-AggregationInfo This field specifies the DL-PRS Resource Sets across DL-PRS Positioning Frequency Layers available for DL-PRS bandwidth aggregation. The 2 or 3 DL-PRS Resource Sets indicated by IE NR-DL-PRS-AggregationIndexList are linked for bandwidth aggregation. - nr-DL-PRS-FrequencyLayerIndex: This field indicates the frequency layer provided in nr-DL-PRS-AssistanceDataList. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on. - nr-DL-PRS-TRP-Index: This field indicates the TRP/DL-PRS ID provided in nr-DL-PRS-AssistanceDataPerFreq. Value 0 corresponds to the first TRP/DL-PRS ID provided in nr-DL-PRS-AssistanceDataPerFreq, value 1 to the second TRP/DL-PRS ID in nr-DL-PRS-AssistanceDataPerFreq, and so on. - nr-DL-PRS-ResourceSetIndex: This field indicates the DL-PRS Resource Set in nr-DL-PRS-ResourceSetList in IE NR-DL-PRS-Info. Value 0 corresponds to the first DL-PRS Resource Set provided in nr-DL-PRS-ResourceSetList, value 1 to the second DL-PRS Resource Set in nr-DL-PRS-ResourceSetList. NOTE: The linked DL-PRS Resource Sets from two or three Positioning Frequency Layers are from the same TRP.. […].

Proposal 1: 	The "linkage information" for DL-PRS bandwidth aggregation between 2 or 3 DL-PRS Resource Sets across PFLs is provided in IE NR-DL-PRS-AssistanceData. The DL-PRS Resource Sets in this "linkage information" are addressed by their PFL-SEQUENCE-Index, TRP- SEQUENCE-Index and DL-PRS Resource Set-SEQUENCE-Index in the IE NR-DL-PRS-AssistanceData.

# 3.	On-demand DL-PRS for DL-PRS bandwidth aggregation

## 3.1	Available On-Demand DL-PRS Configurations

The IEs method-ProvideAssistanceData (with 'method' = NR-DL-TDOA, NR-DL-AoD, NR-Multi-RTT) may provide up to 8 DL-PRS configuration information a UE may request on demand (IE NR-On-Demand-DL-PRS-Configurations-r17).

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand.

However, the IE NR-On-Demand-DL-PRS-Configurations can include only some high-level parameter characterizing a DL-PRS, as illustrated in Figure 3.1-1.

Figure 3.1-1: Available on-demand DL-PRS parameter.

For example, there is no PFL information included in the IE NR-On-Demand-DL-PRS-Configurations. All 8 available DL-PRS configurations may for example belong to a single PFL but may have difference choices for e.g., periodicity, comb-size, etc. However, for DL-PRS aggregation, up to 3 available DL-PRS configurations would have to be from different PFLs (of the same TRP). In addition, all the parameters in the IE NR-On-Demand-DL-PRS-Configurations – except for dl-PRS-ResourceRepetitionFactor and dl-PRS-ResourceBandwidth – must be the same for DL-PRS aggregation (see section 1 above). A UE may infer from multiple DL-PRS configurations with the same values for dl-PRS-CombSizeN, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, and dl-PRS-QCL-Info that the corresponding configurations could be aggregated. However, the same configuration parameter may not necessarily mean that the corresponding configurations can be aggregated, and therefore, additional information in the IE NR-On-Demand-DL-PRS-Configurations is required to indicate to the UE the availability of aggregated DL-PRS.

The IE NR-On-Demand-DL-PRS-Configurations can include "linkage" information on which DL-PRS Configuration IDs can be aggregated. For example:

#### – NR-On-Demand-DL-PRS-Configurations The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be requested by the target device on-demand. -- ASN1START NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ..., [[ on-demand-dl-prs-aggregation-list-r18 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Aggregation-Info-r18 OPTIONAL ]] } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... } DL-PRS-Configuration-ID-r17 ::= SEQUENCE { nr-dl-prs-configuration-id-r17 INTEGER (1..maxOD-DL-PRS-Configs-r17), ... } On-Demand-DL-PRS-Aggregation-Info-r18 ::= SEQUENCE (SIZE (2..3)) OF DL-PRS-Configuration-ID-r17 -- ASN1STOP NR-DL-PRS-On-Demand-Configurations field descriptions. dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.. nr-DL-PRS-PositioningFrequencyLayer This field, together with nr-DL-PRS-Info, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-PositioningFrequencyLayer are applicable: dl-PRS-ResourceBandwidth, dl-PRS-CombSizeN. The target device shall ignore the remaining fields in IE NR-DL-PRS-PositioningFrequencyLayer.. nr-DL-PRS-Info This field, together with nr-DL-PRS-PositioningFrequencyLayer, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-Info are applicable: DL-PRS periodicity in dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols, comb-size in dl-PRS-CombSizeN-AndReOffset, dl-PRS-QCL-Info. The target device shall ignore the remaining fields in IE NR-DL-PRS-Info.. on-demand-dl-prs-aggregation-list This field indicates the 2 or 3 DL-PRS-Configuration-ID's whose corresponding On-Demand-DL-PRS-Configuration's are available for DL-PRS aggregation..

The above essentially groups the DL-PRS-Configuration-ID's into up to 8 sets, each including up to 3 DL-PRS configurations available for aggregation.

The corresponding UE request would then only require an index to the on-demand-dl-prs-aggregation-list field, possibly in a preferred order:

#### – NR-On-Demand-DL-PRS-Request The IE NR-On-Demand-DL-PRS-Request is used by the target device to request on-demand DL-PRS from a location server. -- ASN1START NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE { dl-prs-StartTime-and-Duration-r17 DL-PRS-StartTime-and-Duration-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-r17 NR-On-Demand-DL-PRS-Information-r17 OPTIONAL, dl-prs-configuration-id-PrefList-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF DL-PRS-Configuration-ID-r17 OPTIONAL, ..., [[ dl-prs-aggregation-id-PrefList-r18 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF INTEGER (1..maxOD-DL-PRS-Configs-r17) OPTIONAL ]] } DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE { dl-prs-start-time-r17 INTEGER (1..1024) OPTIONAL, dl-prs-duration-r17 SEQUENCE { seconds-r17 INTEGER (0..59) OPTIONAL, minutes-r17 INTEGER (0..59) OPTIONAL, hours-r17 INTEGER (0..23) OPTIONAL, ... } OPTIONAL, ... } -- ASN1STOP NR-On-Demand-DL-PRS-Request field descriptions. dl-prs-StartTime-and-Duration This field specifies the requested start time and duration for the on-demand DL-PRS and comprises the following subfields: - dl-prs-start-time specifies the desired start time for the requested DL-PRS. It indicates the time in seconds from the time the IE NR-On-Demand-DL-PRS-Request was received. - dl-prs-duration specifies the desired duration of the requested DL-PRS. The desired duration is the sum of the seconds, minutes, hours fields. If this field is included, at least one of the seconds, minutes, hours fields shall be present.. nr-on-demand-DL-PRS-Information This field specifies the on-demand DL-PRS configuration information requested by the target device. NOTE: If the network provided predefined on-demand DL-PRS configurations (NR-On-Demand-DL-PRS-Configurations), the target device can only request explicit parameters (nr-on-demand-DL-PRS-Information) within the scope of those configurations.. dl-prs-configuration-id-PrefList This field specifies the on-demand DL-PRS configuration associated with DL-PRS-Configuration-ID in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first DL-PRS-Configuration-ID in the list is the most preferred configuration, the second DL-PRS-Configuration-ID the second most preferred, etc.. dl-prs-aggregation-id-PrefList This field specifies the on-demand DL-PRS aggregated configuration associated with on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first integer value in the list is the most preferred aggregated configuration, the second integer value in the list is the second most preferred, etc. The integer value corresponds to the entry in the field on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations..

Proposal 2:	The available on-demand DL-PRS configurations supporting DL-PRS bandwidth aggregation are indicated via a group of 2 or 3 DL-PRS-Configuration-ID's in IE NR-On-Demand-DL-PRS-Configurations. Up to 8 such groups can be indicated in IE NR-On-Demand-DL-PRS-Configurations.

Proposal 3:	The IE NR-On-Demand-DL-PRS-Request can include a list of preferred aggregated DL-PRS configurations in the order of preference, where each aggregated DL-PRS configuration is addressed by its SEQUENCE-index in the IE NR-On-Demand-DL-PRS-Configurations.

## 3.2	Explicit On-demand DL-PRS Request Parameter

The IE NR-On-Demand-DL-PRS-Request allows a UE to request a DL-PRS configuration form a list of available DL-PRS configurations (as discussed in section 3.1 above) or allows a UE to request a DL-PRS configuration with specific DL-PRS parameter. The available parameters are summarized in Figure 3.2-1. The UE can request the set of parameter for up to 4 PFLs.

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-On-Demand-DL-PRS-PerFreqLayer-r17 NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE { dl-prs-FrequencyRangeReq-r17 ENUMERATED { fr1, fr2, ...}, dl-prs-ResourceSetPeriodicityReq-r17 ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40, p64, p80, p160, p320, p640, p1280, p2560, p5120, p10240, p20480, p40960, p81920, ..., p128-v1760, p256-v1760, p512-v1760} OPTIONAL, dl-prs-ResourceBandwidthReq-r17 INTEGER (1..63) OPTIONAL, dl-prs-ResourceRepetitionFactorReq-r17 ENUMERATED {n2, n4, n6, n8, n16, n32, ...} OPTIONAL, dl-prs-NumSymbolsReq-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-CombSizeN-Req-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-QCL-InformationReqTRPlist-r17 DL-PRS-QCL-InformationReqTRPlist-r17 OPTIONAL, ... }

Figure 3.2-1: Available NR-On-Demand-DL-PRS-Information parameter.

As discussed in section 3.1 above, all the parameters in the IE NR-On-Demand-DL-PRS-PerFreqLayer – except for dl-PRS-ResourceRepetitionFactorReq and dl-PRS-ResourceBandwidthReq – must be the same for DL-PRS aggregation. Similar to the discussion in section 3.1 above, a UE could request DL-PRS for up to 3 PFLs with the same parameters in IE NR-On-Demand-DL-PRS-PerFreqLayer and the LMF could infer that in this case aggregated PFLs are requested. However, the same parameter request for multiple PFLs does not necessarily mean that aggregated PFLs are requested (e.g., a UE may request 2 PFLs with same periodicity etc. but with different bandwidth for example.).

Therefore, the solution could be the same as for the available on-demand DL-PRS configurations in section 3.1. The UE may request multiple PFLs with explicit parameter and additional information on which PFLs are requested to be aggregated. Since all parameter in IE NR-On-Demand-DL-PRS-PerFreqLayer are OPTIONAL present (except the dl-prs-FrequencyRangeReq), the requested DL-PRS parameter in the multiple PFLs would be the same and need to be included only in the first PFL. The second (or third) PFL would need to include only the dl-prs-ResourceBandwidthReq (and dl-prs-FrequencyRangeReq). This would be backwards compatible since the new aggregation information is on top of the legacy information (i.e., a legacy LMF would still interpret the on-demand PRS request correctly but would not provide aggregated DL-PRS).

For example:

#### – NR-On-Demand-DL-PRS-Request The IE NR-On-Demand-DL-PRS-Request is used by the target device to request on-demand DL-PRS from a location server. -- ASN1START NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE { dl-prs-StartTime-and-Duration-r17 DL-PRS-StartTime-and-Duration-r17 OPTIONAL, nr-on-demand-DL-PRS-Information-r17 NR-On-Demand-DL-PRS-Information-r17 OPTIONAL, dl-prs-configuration-id-PrefList-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF DL-PRS-Configuration-ID-r17 OPTIONAL, ..., [[ dl-prs-aggregation-id-PrefList-r18 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF INTEGER (1..maxOD-DL-PRS-Configs-r17) OPTIONAL, nr-on-demand-DL-PRS-Aggregation-ReqList-r18 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF NR-On-Demand-DL-PRS-Aggregation-ReqElement OPTIONAL ]] } DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE { dl-prs-start-time-r17 INTEGER (1..1024) OPTIONAL, dl-prs-duration-r17 SEQUENCE { seconds-r17 INTEGER (0..59) OPTIONAL, minutes-r17 INTEGER (0..59) OPTIONAL, hours-r17 INTEGER (0..23) OPTIONAL, ... } OPTIONAL, ... } NR-On-Demand-DL-PRS-Aggregation-ReqElement ::= SEQUENCE (SIZE (2..3)) OF INTEGER (1..nrMaxFreqLayers-r16) OPTIONAL -- ASN1STOP NR-On-Demand-DL-PRS-Request field descriptions. dl-prs-StartTime-and-Duration This field specifies the requested start time and duration for the on-demand DL-PRS and comprises the following subfields: - dl-prs-start-time specifies the desired start time for the requested DL-PRS. It indicates the time in seconds from the time the IE NR-On-Demand-DL-PRS-Request was received. - dl-prs-duration specifies the desired duration of the requested DL-PRS. The desired duration is the sum of the seconds, minutes, hours fields. If this field is included, at least one of the seconds, minutes, hours fields shall be present.. nr-on-demand-DL-PRS-Information This field specifies the on-demand DL-PRS configuration information requested by the target device. NOTE: If the network provided predefined on-demand DL-PRS configurations (NR-On-Demand-DL-PRS-Configurations), the target device can only request explicit parameters (nr-on-demand-DL-PRS-Information) within the scope of those configurations.. dl-prs-configuration-id-PrefList This field specifies the on-demand DL-PRS configuration associated with DL-PRS-Configuration-ID in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first DL-PRS-Configuration-ID in the list is the most preferred configuration, the second DL-PRS-Configuration-ID the second most preferred, etc.. dl-prs-aggregation-id-PrefList This field specifies the on-demand DL-PRS aggregated configuration associated with on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first integer value in the list is the most preferred aggregated configuration, the second integer value in the list is the second most preferred, etc. The integer value corresponds to the entry in the field on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations.. nr-on-demand-DL-PRS-Aggregation-ReqList This field specifies the aggregated on-demand DL-PRS configuration information requested by the target device in the order of preference. The first NR-On-Demand-DL-PRS-Aggregation-ReqElement in the list is the most preferred aggregated configuration, the second element in the list is the second most preferred, etc. The integer value in NR-On-Demand-DL-PRS-Aggregation-ReqElement corresponds to the entry in the IE NR-On-Demand-DL-PRS-Information..

Proposal 4:	The requested aggregated PFLs can be indicated by its SEQUENCE-index in the IE NR-On-Demand-DL-PRS-Information in IE NR-On-Demand-DL-PRS-Request. The UE can include a list of preferred aggregated DL-PRS configurations in the order of preference.

# 4.	Summary

In this contribution, we discussed the required LPP changes to support DL-PRS bandwidth aggregation. The following proposals were made.

NR-DL-PRS-AssistanceData for DL-PRS aggregation:

Proposal 1: 	The "linkage information" for DL-PRS bandwidth aggregation between 2 or 3 DL-PRS Resource Sets across PFLs is provided in IE NR-DL-PRS-AssistanceData. The DL-PRS Resource Sets in this "linkage information" are addressed by their PFL-SEQUENCE-Index, TRP- SEQUENCE-Index and DL-PRS Resource Set-SEQUENCE-Index in the IE NR-DL-PRS-AssistanceData.

Available On-Demand DL-PRS Configurations:

Proposal 2:	The available on-demand DL-PRS configurations supporting DL-PRS bandwidth aggregation are indicated via a group of 2 or 3 DL-PRS-Configuration-ID's in IE NR-On-Demand-DL-PRS-Configurations. Up to 8 such groups can be indicated in IE NR-On-Demand-DL-PRS-Configurations.

Proposal 3:	The IE NR-On-Demand-DL-PRS-Request can include a list of preferred aggregated DL-PRS configurations in the order of preference, where each aggregated DL-PRS configuration is addressed by its SEQUENCE-index in the IE NR-On-Demand-DL-PRS-Configurations.

Explicit On-demand DL-PRS Request Parameter

Proposal 4:	The requested aggregated PFLs can be indicated by its SEQUENCE-index in the IE NR-On-Demand-DL-PRS-Information in IE NR-On-Demand-DL-PRS-Request. The UE can include a list of preferred aggregated DL-PRS configurations in the order of preference.

# References

[1]	RP-232670, "Revised WID on Expanded and Improved NR Positioning".

[2]	R2-2310998, "[Post123][402][POS] RAN2 impact of RAN1-led positioning objectives (Nokia)".

[3]	R1-2308719, "CR to38.214: Introduction of specification support for Expanded and Improved NR Positioning".

[4]	R2-2309428, "LS on TRP ID for positioning with bandwidth aggregation", RAN1.