3GPP TSG RAN WG2#123bis	R2-2310917

Xiamen, China, 9th - 13th October 2023

Agenda item:	7.17.4

Source:	China Telecom

Title:	MUSIM gap priority handlling

WID/SID:	NR_DualTxRx_MUSIM - Release 18

Document for:	Discussion and Decision

1	Introduction.

- Agreements (in RAN4#108) - Introduce signalling to allow UE to request to use “keep solution” collision handling mechanism for requested aperiodic and periodic MUSIM gaps and network to grant UE the use of “keep solution”. The same request applies for all MUSIM gaps altogether (i.e. one bit indication). Signalling design is up to RAN2.

- Agreements (in RAN2#121bis) - Introduce 1 optional per-UE capability bit (without xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17. - Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI.

In this contribution, we discuss the open issues with the signaling of MUSIM gap collision handling mechanism and the prohibit timer for MUSIM gap priority based on RAN4 LS (R4-2314449) and agreements in RAN2#121bis.

2	Discussion

2.1 Signaling of MUSIM gap collision handling mechanism

“keep solution” is a collision handling solution related to MUSIM gap priority preference, so if UE is allowed to request for MUSIM gap priority preference via UAI, it should also be allowed to request for “keep solution”, no additional indication is needed.

Proposal1: No additional indication is needed to allow UE requests for “keep solution”.

Since all of the MUSIM gaps and gap priority are configured by network, i.e. UE and network are synchronized in their knowledge of gaps, it is only necessary to introduce 1 bit for both sides to synchronize the collision handling solution.

Proposal2: Introduce 1 bit in UAI to indicate UE’s preference on using “keep solution”, and it can be written like TP in Annex A.

Proposal3: Introduce 1 bit in RRCReconfiguration message for network to grant UE the use of “keep solution”, and it can be written like TP in Annex B.

2.2 Prohibit timer for MUSIM gap priority
In Rel-17, UE can report MUSIM gap preference via UAI if it is allowed by network. In Rel-18, it is agreed to provide priority preference for the MUSIM gap. One approach is that MUSIM gap preference reports together with MUSIM gap preference. Another approach is that MUSIM gap priority preference reports after network has configured UE with MUSIM gap pattern and UE has obtained gapId, so that UE can report MUSIM gap priority preference with the related gap. Obviously, the former approach is simpler and more timely. If the MUSIM gap priority preference reports together with MUSIM gap preference, the prohibit timer musim-GapProhibitTimer agreed in Rel-17 MUSIM can be reused for MUSIM gap priority preference reporting.

Proposal 4: the prohibit timer musim-GapProhibitTimer agreed in Rel-17 MUSIM can be reused for MUSIM gap priority preference reporting.

3	Conclusion

Proposal1: No additional indication is needed to allow UE requests for “keep solution”.

Proposal2: Introduce 1 bit in UAI to indicate UE’s preference on using “keep solution”, and it can be written like TP in Annex A.

Proposal3: Introduce 1 bit in RRCReconfiguration message for network to grant UE the use of “keep solution”, and it can be written like TP in Annex B.

Proposal 4: the prohibit timer musim-GapProhibitTimer agreed in Rel-17 MUSIM can be reused for MUSIM gap priority preference reporting.

References

- RP-220955 Revised WID:  Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, RAN meeting #95e, Release 18.

- R4-2314449, LS on Dual Tx/Rx Multi-SIM gaps

Annex A. Text Proposal

Please note that this TP is based on the running CR in R2-2309307

UEAssistanceInformation message

UEAssistanceInformation-v18xy-IEs ::= SEQUENCE {

musim-Assistance-v18xy                  MUSIM-Assistance-v18xy                  OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                           OPTIONAL

}

MUSIM-Assistance-v18xy ::=              SEQUENCE {

musim-GapPriorityPreferenceList-r18           MUSIM-GapPriorityPreferenceList-r18     OPTIONAL,

musim-CapRestriction-r18                      MUSIM-CapRestriction-r18     OPTIONAL,

musim-keepGaps-r18                            MUSIM-KeepGaps-r18 OPTIONAL

MUSIM-GapPriorityPreferenceList-r18 ::= SEQUENCE (SIZE (1..3)) OF GapPriority-r17

MUSIM-KeepGaps-r18 ::= ENUMERATED {true}

Annex B. Text Proposal

Please note that this TP is based on the running CR in R2-2309307

RRCReconfiguration message

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

MUSIM-GapConfig information element

-- ASN1START

-- TAG-MUSIM-GAPCONFIG-START

MUSIM-GapConfig-r17 ::=                  SEQUENCE {

musim-GapToReleaseList-r17       SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17               OPTIONAL, -- Need N

musim-GapToAddModList-r17        SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17             OPTIONAL, -- Need N

musim-AperiodicGap-r17           MUSIM-GapInfo-r17                OPTIONAL, -- Need N

...,

[[

musim-GapPriorityToAddModList-r18        SEQUENCE (SIZE (1..3)) OF GapPriority-r17             OPTIONAL,

musim-KeepGaps-r18          ENUMERATED{true}       OPTIONAL

]]

}

MUSIM-Gap-r17 ::=          SEQUENCE {

musim-GapId-r17                        MUSIM-GapId-r17,

musim-GapInfo-r17                      MUSIM-GapInfo-r17

}

Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap.

-- TAG-MUSIM-GAPCONFIG-STOP

-- ASN1STOP

MUSIM-GapConfig field descriptions

musim-GapPriorityToAddModList Indicate the priority of MUSIM periodic gap. If the network includes musim-GapPriorityToAddModList-r18, it includes the same number of entries, and listed in the same order, as in musim-GapToAddModList-r17.

Musim-KeepGaps-r18 Indicates the UE is allowed to use “keep solution”