3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310922

Xiamen, China, 9th – 13th October, 2023

Agenda Item:	7.5.4.2

Source:	MediaTek Inc.

Title:	On PSI and discard

Document for:	Discussion and decision

# 1 Introduction

We have had extensive discussions on the use of PSI to discard data due to congestion, with limited progress thus far. This paper reflects on the state of the current discussions in RAN2 and proposes a way forward for this discussion.

# 2 Discussion

We have discussed several flavours of PSI based discard [1] such as:

- Timer-based discard, with different timer values configured for different PSI levels

- Threshold-based discard, dropping PDU sets with a PSI below a threshold

- Buffer-based discard, dropping PDU sets of a PSI when amount of buffered data exceeds a threshold

Excerpt 1 - SA4's feedback on PDU set dependencies

Clarify what, if any, dependency there is between PDU Sets that may carry, e.g. different frames/”slices”.

Again, referring to the response in question 1, no single comprehensive answer can be provided. In some cases, there are no dependencies, in other cases dependencies exist. Referring to the examples, spatial and or temporal prediction of slices/frames across NAL units in the case of the video coding typically applies.  In motion-compensated predicted video decoding, some frames/slices refer to other frames (typically entire frames and not restricted to slices) based on the video encoding configuration but also based on dynamic operational decisions. As consequence, a PDU Set may “depend” on previously received PDU Sets. However, such dependencies do not necessarily result in discarding dependent information units, but the user experience may be degraded.

SA4 would like to point out, that due to its heavy-compression and spatial-temporal prediction, any packet losses in video generally result in degradation of the user-perceived quality of experience. Hence, video applications generally (i) benefit, (ii) are more efficient and (iii) can be simplified, if the network minimizes video packet losses. Nevertheless, a video decoder in particular in a low-latency application needs to include mechanisms to handle packet losses and delayed packets, such as frequent resynchronization and error concealment. In those cases, the operation of the receiver/network may vary. For example, the handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application. However, typically, video applications prefer reducing the encoding bitrate in order to minimize congestion-related packet losses. If the application and the 5GS have agreed to a QoS flow establishment, then the network is obviously expected to support the delivery of PDUs according to the QoS requirements of the application.

In the same discussion, it was clear that most companies agree that discard of data should be carefully performed, so as to not negatively affect the service. This was also clear from SA4’s LS (relevant sections highlighted in Excerpt 1 below) [2]. PDU sets are dependent on earlier PDU sets, so any dropping of data can have significant negative consequences to the service. As indicated by SA4, discarded PDU sets can result in the application becoming complicated and inefficient, leading to a degraded end-user experience.

Observation 1: Unless carefully designed, data discard can result in the video application becoming complicated and inefficient, leading to a degraded end-user experience.

While Observation 1 is familiar to all companies in RAN2, where the discussions digress is on what PDU sets of different PSI levels actually mean. Would always dropping packets of low importance be better for the end-user experience (e.g. threshold based discard), or should some of them be transmitted (e.g. timer/buffer based discard)? Without having a clear understanding of what sets apart packets of different PSI levels and their cross-dependencies (if any), introducing a mechanism of PSI based discard is premature and could lead to a worse end-user experience than if we did nothing at all.

Observation 2: RAN2 does not have clear understanding of the impact of PSI based discard on the video application.

Given that SA4 are the experts on this topic and have introduced the concept of PSI, RAN2 should first get clarification from SA4 on the differences between PDU sets of different PSI levels, their cross-dependencies, and the impact of discarding data based on PSI in the presence of congestion. Once RAN2 understand this better, we can make an informed choice on the correct way forward. Of course, by the time this is clarified by SA4, we may not have time left in Rel-18 to introduce such a mechanism, but this can be done in the Rel-19 timeframe instead. We should not introduce a mechanism without understanding its impact on the end-user experience.

Proposal 1: RAN2 does not introduce a PSI based discard mechanism until its impact on the video application is understood.

Proposal 2: RAN2 sends an LS to SA4 to clarify the differences between PDU sets of different PSI levels, their cross-dependencies (if any) and the consequences to the video application of dropping packets of low PSI levels in the presence of congestion.

# 3 Conclusion

In this contribution we have the following observations:

Observation 1: Unless carefully designed, data discard can result in the video application becoming complicated and inefficient, leading to a degraded end-user experience.

Observation 2: RAN2 does not have clear understanding of the impact of PSI based discard on the video application.

Based on the observations above, we propose:

Proposal 1: RAN2 does not introduce any mechanism of PSI based discard until its impact on the video application is understood.

Proposal 2: RAN2 sends an LS to SA4 to clarify the differences between PDU sets of different PSI levels, their cross-dependencies (if any) and the consequences to the video application of dropping packets of low PSI levels in the presence of congestion.

# 4 References

- R2-2309003 - Report of [AT123][205][XR] Options for PSI-based discard (Ericsson)

- R2-2206969 - LS Reply on QoS support with PDU Set granularity (SA4)