3GPP TSG-RAN WG2 #123bis	Tdoc R2-2301923

Xiamen, China, October 9th- 13th, 2023

Agenda Item:	7.8.3

Source:	Ericsson

Title:	UAV Flight Path Reporting

Document for:	Discussion, Decision

# Introduction

The WID for supporting UAV in NR [1] contains an objective on enhancements to measurement reports, including flight path reporting.

The following agreement was made in the last meeting.

Agreements RAN2#123 - No delta signaling for updated flight path will be supported - Confirm that same indication is used for both initial and updated flightpath plan - If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication. - When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met. No prohibit timer will be introduced.

In addition, as per the chair’s guidance on providing a list of open issues, the rapp has provided the following on the flight path plan:

Which messages should be used to (re)configure the distance/time threshold for Flight path update (no explicit agreement on these so far)

In this paper we address the issues as listed above and remaining open issues for flight path reporting.

# Flight path reporting

## Open Issues

### From-the-List

As mentioned in the introduction, the open issue is about which messages should be used to (re)configure the distance/time threshold for the flight path update. In addition to the RRCReconfiguration message, the other options are the RRCSetup and RRCResume message. For the RRCSetup message, the assumption of the spec rapporteur is that this is the initial link establishment procedure (from RRC_IDLE), and the UE is unlikely to have already reported a flight path, hence, an update cannot happen. In which case, the same logic should be applicable to the RRCResume message because the RRCResumeRequest is triggered from the UE in an RRC_INACTIVE state. Although there are chances that the UE has triggered a flight path before going into the RRC_INACTIVE/IDLE state, the handling of the flight path reporting RRC_INACTIVE/IDLE has not been discussed or in scope of this release.

Handling of the flight path report in RRC_INACTIVE/IDLE is not in scope of this release.

Hence, the RRCReconfiguration message is sufficient to (re)configure the distance/time threshold.

- The (re)configuration of the distance/time threshold is sent only in the RRCReconfiguration message.

### Not-From-the-List

The following is the current formulation of the flight path information reporting in the TS 38.300 running CR:

“Following an indication from the Aerial UE that flight path information is available, NG-RAN can request the Aerial UE to report flight path information consisting of a number of waypoints defined as 3D locations, as defined in TS 37.355 [43]”

However, the network can also poll the UE (via the UEInformationRequest message) at any time to obtain the flight path information independent of the flight path availability indication. The corresponding TP is as shown below.

- NG-RAN can request the UE to obtain the flight path information independent of the (flight path) availability indication. Adopt the TP for 38.300.

## Remaining Open Issues

### Distance and time threshold

Although RAN2 has agreed that, if any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication. It remains open how the threshold should be specified (e.g., in what format). For the distance threshold, in our understanding, there would be a hypothetical 3D sphere around each waypoint with a radius controlled by the distance threshold. If the location of a waypoint is changed to outside the associated sphere, the UE sends a flight path update report. Since there can be different types of UAVs with different sizes, speeds, executing different types of missions, the distance threshold can vary a lot in absolute value. In other words, the distance threshold for flight path update can be very dependent on the UAV type, its planned flightpath, as well as the location accuracy that the network wants to have. Not to mention other types of aerial UEs such as air taxis or even helicopters.

Distance threshold between the different type of UAVs can vary a lot when expressed in absolute values. It can depend on the UAV type, planned flightpath, location accuracy.

On the other hand, it is desirable to have a representation of the distance threshold that is applicable to all kinds of aerial UEs. Therefore, in our view, it is most efficient if the network configures the UE with a reference distance (e.g., based on the parameters mentioned above) and the distance threshold is signaled to the UE as a percentage of this reference distance. That is, if the distance between a new location and a previous location of the same waypoint has changed more than a certain percentage of the reference distance, a flight path update is triggered. The reference distance can be calculated by the network based on the different parameters as described above and up to implementation.

- The distance threshold is configured as a percentage value of a reference distance, which is also configured by the network.

The associated TP for 38.331 is as provided below.

### Time Stamp

In our view, besides the baseline contents like in Rel-15, other alternatives or additions can be specified to improve the usability of the time and location information included in the flight path report. For example, instead of reporting a single timestamp, the UE can report a time window which indicates a period that the UAV is expected to be in or near a certain location, e.g., for a mission such as inspecting a construction site or part of a power grid.

Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

The time window can be expressed using a validity timer to allow for flexibility of signalling. Similarly, instead of using 3-D waypoints, one can consider the notion of area or volume or 3-D zone.

- To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

### Waypoint Information

Additionally, some other types of information related to the flight path and operating condition of the UAV can also be reported, either in the flight path report or in a separate UE report. For example, information about default/emergency landing spots could be used by the network to secure good connectivity to those spots.

This indication about default/emergency landing spots is related to the final waypoint indication. It would be beneficial for the network to understand the initial and final waypoints to get a better idea of the flight path. In which case, for default/emergency landing spots, the UAV UE can just indicate a single (final) waypoint in the flight path update.

- Introduce the notion of an initial and final waypoint in the flight path report.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Handling of the flight path report in RRC_INACTIVE/IDLE is not in scope of this release.

Observation 2	Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The (re)configuration of the distance/time threshold is sent only in the RRCReconfiguration message.

Proposal 2	NG-RAN can request the UE to obtain the flight path information independent of the (flight path) availability indication. Adopt the related TP for 38.300.

Proposal 3	To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

Proposal 4	Introduce the notion of an initial and final waypoint in the flight path report.

# References

RP-223545, WID update: NR support for UAV (Uncrewed Aerial Vehicles), RAN#98, Dec 2022

# TP for 38.300

============= START OF CHANGES ===============

<Unmodified parts omitted>

### 16.X.6	Flight Path Information Reporting for Aerial UE Communication

NG-RAN can request the Aerial UE to report flight path information. Optionally, NG-RAN can also request the Aerial UE to report the flight path information based on a flight path availability indication from the Aerial UE. The flight path information consists of a number of waypoints defined as 3D locations, as defined in TS 37.355 [43]. Aerial UE reports up to configured number of waypoints if flight path information is available at the UE. The report can also contain a time stamp per waypoint if configured by the NG-RAN and if available at the UE. The flight path information can be also provided by the source gNB to the target gNB during handover. If configured by the NG-RAN and if the associated time- or distance-based condition (see x and y in [12]) for indication reporting are met for any of the waypoints, the Aerial UE indicates the availability of the updated flight path information.

<Unmodified parts omitted>

============= END OF CHANGES ===============

# TP for 38.331

TP for Distance threshold. This TP is based on the latest version of the RRC running CR.

============= START OF CHANGES ===============

<Unmodified parts omitted>

#### –	UAV-Config

The IE UAV-Config provides configuration parameters for UAV UE.

UAV-Config information element

-- ASN1START

-- TAG-UAV-CONFIG-START

UAV-Config-r18 ::= SEQUENCE {

flightPathUpdateDistanceThr-r18   CHOICE {

release    NULL,

setup      ::= SEQUENCE {

distanceReference		FFS,

distanceThreshold		INTEGER (1..100)

}

}                                         OPTIONAL, -- Need M

flightPathUpdateTimeThr-r18       CHOICE {

release    NULL,

setup      FFS

}                                          OPTIONAL -- Need M

}

-- TAG-UAV-CONFIG-STOP

-- ASN1STOP

UAV-Config field descriptions

flightPathUpdateDistanceThr Distance threshold for triggering flight path update indication.

flightPathUpdateTimeThr Time threshold for triggering flight path update indication.

distanceReference Reference distance for determining the distance threshold for flight path update trigger.

distanceThreshold Threshold value used together with the reference distance specified in distanceReference for computing distance threshold for flight path update trigger. 1 means 1 percent, 2 means two percent, and so on.

<Unmodified parts omitted>

============= END OF CHANGES ===============

# Annex

List of agreements from the previous meetings:

Agreements RAN2#119-bis-e: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells).

Agreements RAN2#120:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

Agreements RAN2#121:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Agreements RAN2#121bis:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling