3GPP TSG-RAN WG2 #123bis	R2-2310925

Xiamen, China, 9th October – 13th October 2023

Agenda Item:	7.9.2

Source:	Ericsson

Title:	Discussion on Relay (re)selection and Discovery

Document for:	Discussion, Decision

# 1 Introduction

In the WID [1], relay (re-)selection is one common objective for L2 and L3 U2U relays. As per the chair’s guidance on providing a list of open issues [1], the rapp has provided the following related to discovery/relay selection for U2U relays, captured as the editor note:

Editor Note: FFS whether separate thresholds are configured for NR sidelink U2U Relay UE

Editor Note: FFS whether separate thresholds are configured for NR sidelink U2U Remote UE

Editor NOTE: FFS whether reuse the U2N relay (re)selection parameters to U2U relay (re)selection.

Editor NOTE: FFS communication or discovery resource pool for DCR message with integrated discovery

Editor Note: FFS whether/how to capture if the SL-RSRP/SD-RSRP measurement of the peer NR sidelink U2U Remote UE is not available.

Editor NOTE: FFS whether this section needs to update for U2U relay (re)selection evaluation or not

Editor NOTE: For U2U Relay UE and Target Remote UE, it is FFS whether AS layer check discovery transmission condition before delivering discovery message to upper layer or after receiving discovery from upper layer, and FFS whether upper layer needs to know if AS condition is met for each discovery message.

Editor Note: FFS the remote UE in previous agreement “When the remote UE receives PC5-RLF indication from the U2U relay UE, it would inform upper layers and rely on upper layers to trigger relay reselection (or not).” applies to both source and target remote UEs or not, applies to both L2 and L3 U2U relay or not.

We address the issues as listed above and remaining open issues for the relay (re)selection and discovery for U2U relays.

# 2 Discussion

## 2.1 Open Issues

### 2.1.1 From-the-List

For the FFS on whether separate thresholds are configured for NR sidelink U2U relay UE in the case of integrated, model-A and mode-B discovery, we believe that a single threshold is sufficient to cover all the cases. This is because independent of the cases, the UE action is to forward either the DCR message, the announcement message, or the solicitation message. All messages should be handled in the same way, and the condition only depends on the link quality between the relay UE and remote UEs i.e., source or destination independent of the discovery procedure.

Single threshold is configured for NR sidelink U2U Relay UE.

- On the FFS on separate thresholds are configured for NR sidelink U2U Remote UE, the trigger threshold for relay selection and reselection should be the same i.e., the threshold to trigger relay selection over the direct link between the source/destination UE should be the same as the trigger for relay reselection over the first/second hop between the source/destination UE and U2U relay UE. Effectively, the threshold is based on the signal strength between two entities.

Single threshold is configured for NR sidelink U2U Remote UE.

- For the FFS about using the discovery resource pool for DCR message when using the integrated discovery procedure, we believe the DCR message can also be transmitted in the discovery resource pool. After performing the integrating discovery procedure, the UEs can continue to perform the PC5 link establishment over the shared resource pools.

DCR messages with integrated discovery can be transmitted over the discovery resource pools.

- On the FFS relating to the whether/how to capture if the SL-RSRP/SD-RSRP measurement of the peer NR sidelink U2U Remote UE is unavailable, this does not seem like a valid scenario. The source remote UE when initiating communication over SL will always perform the discovery procedure to find the destination remote UE. In which case, the source remote UE will be able to measure the SD-RSRP of the corresponding messages. Then, it is up to the U2U Remote UE to choose the appropriate link i.e., the direct or indirect link.

Need not capture the case when the SL-RSRP/SD-RSRP measurement of the peer NR U2U Remote UE is unavailable.

### 2.1.2 Not-From-the-List

- In addition to the list of open issues from the rapp, the following issues are yet to be concluded upon.

- In the case when the source U2U Remote UE triggers a relay reselection procedure and finds both the direct link and indirect link to the destination U2U Remote UE, it is up to UE implementation which path is chosen.

The choice of when both a direct and indirect path is available during the U2U relay reselection procedure is up to UE implementation.

## 2.2 Remaining Open Issues

### 2.2.1 Coexistence between U2N and U2U

Proposal 11: RAN2 to agree that the same dedicated discovery resource pool (defined in Rel-17), if configured, can be used for non-relay discovery, U2N relay discovery and/or U2U relay discovery as baseline. Can be revisited if any impact on co-existence between U2N/U2U.

Proposal 12: RAN2 to further discuss whether to support co-existence between U2N relays and U2U relays and the potential specification impact if any.

In the above proposal is from the previous meeting, the impacts of allowing the U2U relay discovery and U2N relay discovery in the same dedicated discovery pool are yet to be studied. On the UE-side, it would require a prioritization and differentiation at the AS-layer between U2U/U2N relays. Similarly, from the network’s perspective, if the same/different UEs support U2N/U2U relaying at least would need to perform prioritization between transmissions.

Coexistence would require further study in terms of differentiation and prioritization of services in the AS-layer.

Such co-existence mechanisms between U2N relays and U2U relays is out of scope of the WI and should not be pursued.

RAN2 does not pursue the co-existence between U2N relays and U2U relays in this release as it is not scope of the work item.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Coexistence would require further study in terms of differentiation and prioritization of services in the AS-layer.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Single threshold is configured for NR sidelink U2U Relay UE.

Proposal 2	Single threshold is configured for NR sidelink U2U Remote UE.

Proposal 3	DCR messages with integrated discovery can be transmitted over the discovery resource pools.

Proposal 4	Need not capture the case when the SL-RSRP/SD-RSRP measurement of the peer NR U2U Remote UE is unavailable.

Proposal 5	The choice of when both a direct and indirect path is available during the U2U relay reselection procedure is up to UE implementation.

Proposal 6	RAN2 does not pursue the co-existence between U2N relays and U2U relays in this release as it is not scope of the work item.

# 4 References

- R2-2309755, Report of [Post123][Relay] Remaining Open Issues (LG), RAN2#123-bis, Xiamen, China