3GPP TSG-RAN WG2 #123bis	R2-2310927

Xiamen, China, 9th October – 13th October 2023

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on Inter-gNB Service Continuity

Document for:	Discussion, Decision

# 1	Introduction

As per the chair’s guidance on providing a list of open issues, the rapp has provided the following related to the service continuity aspects [2], captured as editor’s note:

Inter-gNB service continuity cases missing in Stage 2 specification

The order of RRCReconfiguration of Relay UE and Remote UE and the spec impact on supporting the relay UE being in RRC_IDLE or RRC_INACTIVE for switching from indirect to indirect path

We address the issues as listed above and remaining open issues for the service continuity of L2 U2N relays

# 2	Discussion

## 2.1 Open Issues

For the order of the RRCReconfiguration of relay UE and remote UE captured in the FFS in the 38.300 running CR [1] in section 16.12.6.x, we think it should be up to network implementation and there need not be a restriction in the order. There is no advantage in swapping the orders.

It can be left to network implementation when the remote UE or relay UE is (re-)configured. Remove the related EN.

## 2.2 Remaining Open Issues

### 2.2.1 Triggering the U2N Relay UE to CONN State

The triggering of the U2N relay UE into the CONN state is relevant for the d2i and i2i scenarios.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios as a baseline for single-path relay.  Other mechanisms are not excluded if an issue is found with the baseline.

- The above agreement was made in the last meeting with the addition of the highlighted text. There was also some discussion about using a paging-based solution in addition to the trigger from the remote UE to transit the U2N relay UE in IDLE/INACTIVE to the CONN state.

- Although we foresee some potential benefits from the paging-based solution especially for inter-gNB path switch scenarios, in terms of reducing the latency of the overall path switch procedure by potentially transitioning the target U2N relay UE to the CONN state before remote UE initiates the PC5 communication. The following is our analysis:

- For the target U2N relay UE in the INACTIVE state, the paging-based solution is already feasible based on Rel-17 specifications and with no changes required to the legacy paging mechanism. That is, if the gNB can retrieve the target U2N relay UE’s context, it can page (using RAN-paging) the target U2N relay UE to initiate the random-access procedure. This can also be done before the path switch command is sent to the remote UE and as a result, potentially, the target U2N relay UE is already in the CONNECTED state when the remote UE initiates PC5 communication for relaying.

- For the target U2N relay UE in the IDLE state, additional specification work is required to support this state, and this would involve cross-WG interactions (SA2/RAN3). Given the current workload, such solutions should be down prioritized.

For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

  - In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

  - In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1 It can be left to network implementation when the remote UE or relay UE is (re-)configured. Remove the related EN.

Proposal 2 For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

- In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

- In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

# 4 References

[1] R2-2309339, Introduction of NR Sidelink relay enhancements, RAN2#123, Toulouse, France.

[2] R2-2309755, Report of [Post123][Relay] Remaining Open Issues (LG), RAN2#123-bis, Xiamen, China