3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310929

Xiamen, China, October 9-13, 2023

Agenda Item:	7.5.3

Souce:	MediaTek Inc.

Title:	Remaining issues for C-DRX in XR

Document for:	Discussion and decision

# 1 Introduction

In previous RAN2 meetings, the following agreements were reached for the XR power savings [1]:

- The maximum value of the counter (NSFN) is 2^16 – 1 = 65535. - Network sets DRX reference SFN (drx-ReferenceSFN) to either 0 or 512, in the same way as in Rel-16 IIoT. - Use the following option: both the counter NSFN and the DRX reference SFN drx-ReferenceSFN are added to the DRX formula. NSFN is initialized to 0.

In this contribution, we discuss the remaining issues for XR power savings.

# 2 Discussion

## 2.1 Rational number DRX cycles

In the previous RAN2 meeting, we agreed to introduce DRX cycles expressed as rational numbers. New rational number DRX cycles can be easily defined by introducing two new parameters (per DRX cycle) Dividend and Divisor in DRX-Config in the RRC specification TS 38.331. This will make it very easy to define new rational number DRX cycles with Dividend and Divisor parameters and has the added advantage of being future proof. The corresponding rational number DRX cycle can then be computed as:

drx-NonIntegerShortCycle = drx-ShortCycleDividend / drx-ShortCycleDivisor drx-NonIntegerLongCycle = drx-LongCycleDividend / drx-LongCycleDivisor

Proposal 1: Implement rational number DRX cycles by introducing Dividend and Divisor parameters in DRX-Config in TS 38.331.

Modulo operation in DRX formulae is currently performed with integer base (integer DRX cycles). When non-integer base (rational DRX cycle) is used, the possibility of rounding errors has been raised as a concern in some contributions [2].

To overcome the rounding errors, the modulo operation can be replaced by the equivalent function as below:

A modulo B =A - B ×floor AB

There have also been suggestions that replacing the modulo function with the above equivalent may sometimes still result in rounding errors [2]. Such errors only occur if the equivalent modulo function is implemented incorrectly, for example if a fixed-point division operation is performed before the multiplication operation. It should be noted that the formula does not make any assumptions on fixed point division or on the order of the operation. However, if incorrect implementation is a concern, one option is to define the exact order of the implementation of the equivalent modulo operation by using parentheses. For example, the following formula can be included in the specifications:

A modulo BC =A - B × floor A ×CBC

Additionally, the result of the modulo function should be rounded to the nearest integer by using the floor function in the DRX formulas.

The resulting changes to the TS 38.321 specification can be illustrated as follows:

1> if the Short DRX cycle is used for a DRX group, and floor [[(SFN × 10) + subframe number] modulo (drx-ShortCycle)] = floor [(drx-StartOffset) modulo (drx-ShortCycle)]: 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and floor [[(SFN × 10) + subframe number] modulo (drx-LongCycle)] = drx-StartOffset: <Legacy text omitted> If the drx-ShortCycle or the drx-LongCycle are configured as rational numbers, the modulo operation is defined as: A modulo BC =A - B × floor A ×CBC

Table 1 TP for supporting rational number DRX cycles in DRX formulas in TS 38.321

Proposal 2: Adopt the TP in Table 1 for the DRX formulas in TS 38.321 to support rational number DRX cycles, by using the floor operation and defining the alternative modulo operation as in Table 1 in DRX formulae.

# 3 Conclusion

In this contribution, we discuss the remaining issues for XR power savings, and have the following proposals:

Proposal 1: Implement rational number DRX cycles by introducing Dividend and Divisor parameters in DRX-Config in TS 38.331.

Proposal 2: Adopt the TP in Table 1 for the DRX formulas in TS 38.321 to support rational number DRX cycles, by using the floor operation and defining the alternative modulo operation as in Table 1 in DRX formulae.

# 4 References

- R2-2308962 Report from session on LTE legacy, XR, QoE and Multi-SIM, Vice Chairman (Nokia)

- R2-2305830 Discussion on XR-specific power saving, Ericsson, RAN2#122,