3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310931

Xiamen, China, 9 October – 13 October 2023               (revision of R2-2308702)

Agenda item:	7.8.2

Source: 	Huawei, HiSilicon

Title: 	Measurement report enhancement for NR UAV

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #123, the following agreements have been reached regarding measurement report enhancement for NR UAVs:

Agreements:

Measurement reporting

- New event type is introduced to implement both height-dependent MR configuration and combination of events

- The new event type includes only one of H1 or H2 thresholds.   A hysteresis for each threshold will be introduced.  A single time to trigger for the event will be introduced. Rapporteur will implement the solution and we can come back next meeting if there are technical issues.

- Event combination of H1/H2 with A3, A4, A5 will be support

- When the UE moves to a new height region with a different numberOfTriggeringCells value for one configured event, it is up to UE implementation whether the UE keeps the old measurement report or cellstriggeredlist

- At least 300m above ground level will be supported.  FFS if higher ranges can/need to be considered.

- Signalling should allow the network to simultaneously configure height-based event thresholds for the whole range of possible UE heights

As we can see, there are some FFS that remain to be analyzed. This contribution is a revision of R2-2308702. Compared to the previous version we analyzed a few more technical aspects due to be resolved in RAN2 (i.e. FFSs).

- Discussion

RAN2 has agreed that at least 300m above ground level will be supported for the NR UAV. Some companies said that 300m is the LTE requirement for UAVs, and NR may require a higher altitude. We agree. Now, UAM (urban air mobility) attracts more and more attention, and it will require communication via the 3GPP system [1]. RAN2 needs to consider providing communication for UAM in a similar way to UAV. The height range of UAM is 150 m–1000 m. Thus, considering the UAM, the height range for UAV can be extended to 1km.

Proposal 1: Considering the UAM, the height range for UAV can be extended to 1km.

Regarding the new event type, RAN2 has agreed that a new event type is introduced to implement both height-dependent MR configuration and combination of events. RAN2 also agreed that the new event type includes only one of H1 or H2 thresholds. If the NW wants to configure different configurations at different height ranges, some companies say that the NW can configure multiple events for it.

There is an issue if the NW does this. For instance, it may trigger the UAV to report redundant MR to the NW in certain cases. Specifically, the UAV may trigger multiple MRs if the NW (re-)configures multiple events for the UAV when it is flying at a certain height.

For example, we assume the height of the UAV is 350 m, and the NW configures three new events (event H1 + event A4) for the UAV, where the thresholds of event H1 are 100m, 200 m, and 300m, respectively. At this time, three new events fulfill the height threshold of event H1 (for simplicity, we assume the event A4 thresholds are also fulfilled for these three events). According to the definition of event H1, i.e., if the aerial UE height becomes higher than the threshold, three MRs will be triggered because the 350m is higher than all the three configured new event thresholds. This will cause signalling overhead because these MRs are the same.

Another example is if the UAV is configured with multiple height-dependent “numberoftriggeringcells”, e.g., (100m, 3 cells), (200m, 4 cells), and (300m, 2 cells). The UAV will be confused when it receives this configuration at 350m. What “numberoftriggeringcells” should be considered? It can be 2, 3, and 4 cells based on the definition of event H1/H2 because 350 m is higher than all of them.

We can see that this issue exists in multiple events H1/H2, H1/H2 + A3/A4/A5, and H1/H2 + “numberoftriggeringcells”.

Observation: If the NW configures multiple event H1/H2 or multiple height-dependent configurations/ measurement events, the UE may trigger multiple MRs or apply multiple configurations.

In order to reduce the signalling overhead and determine what configuration should be applied, RAN2 should define the UE behaviour when multiple height-dependent MR configurations or events are configured on the UAV. We propose that if the NW configures multiple event H1/H2 or multiple height-dependent configurations/ measurement events, the UAV should choose the one of them whose distance is the smallest between the altitude of the UAV and the configured height threshold to trigger or the application of the corresponding event or of the MR configuration.

We can see the example mentioned above again: the height of the UAV is 350 m, and the NW configures three new events (event H1 + event A4) for the UAV, where the thresholds of event H1 are 100m, 200 m, and 300m, respectively. At this time, the UAV triggers the MR based on the event H1 with a 300m threshold (for simplicity, we assume the event A4 thresholds are also fulfilled) because this threshold is the nearest one to the UAV height.

Proposal 2: If the NW configures multiple event H1/H2 or multiple height-dependent configurations/ measurement events, the UAV should choose the one of them whose distance is the smallest between the altitude of the UAV and the configured height threshold to trigger or the application of the corresponding event or of the MR configuration.

Regarding the numberOfTriggeringCells, there is an open issue, i.e., if a height-dependent numberOfTriggeringCells is applied, how does the UE handle the old measurement results (e.g., the cell in cellTriggeredList) when the UE applies a new numberOfTriggeringCells when it reaches a new height range. This issue is also present in height-dependent SSB-ToMeasure, and it was agreed in the last meeting that how to handle the old samples is up to UE implementation.

The purpose of SSB-ToMeasure and numberOfTriggeringCells are different.

The SSB-ToMeasure is used to configure a pattern of SSBs. Once the SSB-ToMeasure has changed, the UE will change the measured objective. The old samples may be useless if the measured objective is totally different. But the old samples may be partially useless if the measured objective changes partially. Thus, how to handle the old samples can be left to the UE implementation.

On the contrary, the numberOfTriggeringCells is used to indicate the number of cells detected that are required to fulfil an event for a measurement report to be triggered. Once the numberOfTriggeringCells is changed, the old measured cells are still useful for the network. If the UE discards the old measured cells, the late measurement report will be a more serious issue. Thus, RAN2 should consider that the UE should keep the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

Proposal 3: RAN2 should consider that the UE keeps the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

- Conclusion

Proposal 1: Considering the UAM, the height range for UAV can be extended to 1km.

Observation: If the NW configures multiple event H1/H2 or multiple height-dependent configurations/ measurement events, the UE may trigger multiple MRs or apply multiple configurations.

Proposal 2: If the NW configures multiple event H1/H2 or multiple height-dependent configurations/ measurement events, the UAV should choose the one of them whose distance is the smallest between the altitude of the UAV and the configured height threshold to trigger or the application of the corresponding event or of the MR configuration.

Proposal 3: RAN2 should consider that the UE keeps the old measurement results (e.g., the cell in cellTriggeredList) when the UE moves to a new height region with a different numberOfTriggeringCells value.

- Reference

[1] https://www.easa.europa.eu/en/what-is-uam