3GPP TSG-RAN WG2 Meeting #123-bis	 R2-2310936

# Xiamen, China, 9 October – 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0965 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of capabilities for Rel-18 NR Support for UAV WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV-Core |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of capabilities for Rel-18 NR Support for UAV WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add capability bit indications for height-based measurement reporting, height-dependent measurement configurations, triggering based on number of cells, flight path plan report/update and dedicated resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Features of Rel-18 NR Support for UAV WI not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.9, 4.2.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38331 CR draftCR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 2	References

<<<<<<<<<<<skipped>>>>>>>>>>>>>

[x]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

END OF CHANGE

START OF CHANGE

### 4.2.9	MeasAndMobParameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| height-meas-r18 This field defines whether the UE supports height-based measurement reporting as specified in TS 38.331 [9]. It is mandatory to support this feature for UEs which have Aerial UE subscription as defined in TS 23.256 [x]. | UE | Yes | No | No |
| height- dependent-configurations- r18 This field defines whether the UE supports height-dependent measurement configurations as specified in TS 38.331 [9]. It is optional to support this feature for UEs which have Aerial UE subscription as defined in TS 23.256 [x]. | UE | No | No | No |
| multipleCellsMeasExtension-r18 This field defines whether the UE supports measurement reporting triggered based on a number of cells. | UE | No | No | No |
| flightPathPlan-r18 This field defines whether the UE supports reporting of the flight path plan and its update through the procedure defined in TS 38.331 [9]. | UE | No | No | No |

Editor’s note: The height-meas-r18 is used for event Hx and the height-dependent-configurations-r18 is used for event Ax + Hx, height-dependent SSB-ToMeasure and height-dependent NumberOfTriggeringCells.

NEXT CHANGE

#### 4.2.16.1	Sidelink Parameters in NR

##### 4.2.16.1.1	Sidelink General Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| sl-A2X-Service-r18 This field defines whether the UE supports dedicated resource pool for A2X service as specified in TS 38.331 [9]. | UE | No | No | No |

END OF CHANGE