3GPP TSG-RAN WG2 #123-bis	Tdoc R2-2310953

Xiamen, China, October 09 – 13, 2023

Agenda Item:	7.25.2

Source:	Ericsson, ZTE Corporation

Title:	RAN2 updates for DSS

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#121, the following changes were endorsed for DSS:

R18 DSS

Not treated, just resubmission of CRs for information

R2-2301321	Running 38.306 CR for R18 DSS	ZTE Corporation, Ericsson	draftCR	Rel-18	38.306	17.3.0	B	NR_DSS_enh-Core

R2-2301405	Running 38.331 CR for R18 DSS	Ericsson, ZTE Corporation	CR	Rel-18	38.331	17.3.0	3888	-	B	NR_DSS_enh-Core

This contribution discusses how to handle the further changes required for DSS.

# 2	Discussion

Since RAN2#121, further aspects for DSS were introduce by RAN1. A new RRC parameter was agreed as indicated in the LS on Rel-18 higher-layer parameter list (R2-2307021):

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh-Core | Support for two overlapping CRS rate matching patterns |  |  |  |  | lte-CRS-PatternList3 | New |
| NR_DSS_enh-Core | Support for two overlapping CRS rate matching patterns |  |  |  |  | lte-CRS-PatternList4 | New |
| NR_DSS_enh-Core | Reception of NR PDCCH candidates overlapping with LTE CRS REs |  |  |  |  | pdcchCandidateReception-WithCRSOverlap |  |

Additionally, UE capabilities were also agreed as indicated in the LS on UE feature list for Rel-18 NR (R1-2306223):

| Features | Index | Feature group |
| --- | --- | --- |
| 52. NR_DSS_enh | 52-1 | Reception of NR PDCCH candidates overlapping with LTE CRS REs |
| 52. NR_DSS_enh | 52-1a | Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns |
| 52. NR_DSS_enh | 52-1b | NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier |

For the RRC parameters, the 38.331 CR previously endorsed already contained lte-CRS-PatternList3 and lte-CRS-PatternList4, but the parameter pdcchCandidateReception-WithCRSOverlap still needs to be reflected on the 38.331 CR.

Add pdcchCandidateReception-With-CRS-Overlap in ServingCellConfig.

On UE capabilities, the 38.306 CR previously endorsed only contains updates to overlapRateMatchingEUTRA-CRS-r16 capability, as discussed based on a previous RAN1 LS R2-2209314. For the further aspects on UE feature list for Rel-18 NR, we understand that those will be handled in the Rel-18 common Mega CRs, note that the latest version of those CRs submitted to RAN#123 already contained updates for DSS capabilities (R2-2308086, R2-2308087).

Therefore, we suggest to agree on the contents of the 38.306 CR as previously endorsed in RAN#121 (R2-2301321), and handle further discussion on UE capabilities for DSS in the Rel-18 common Mega CRs.

Update the overlapRateMatchingEUTRA-CRS-r16 capability based on RAN1 LS R2-2209314. Further UE capability updates for DSS to be discussed in the Rel-18 common Mega CRs.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add pdcchCandidateReception-With-CRS-Overlap in ServingCellConfig.

Proposal 2	Update the overlapRateMatchingEUTRA-CRS-r16 capability based on RAN1 LS R2-2209314. Further UE capability updates for DSS to be discussed in the Rel-18 common Mega CRs.