3GPP TSG-RAN WG2 #123bis	Tdoc R2-2310965

Xiamen, China, Oct. 9th – 13th, 2023

Agenda Item:	7.17.3

Source:	Ericsson

Title:	On some restricted capabilities for Rel-18 MUSIM UE

Document for:	Discussion

# Introduction

In this document, we discuss signalling proposal for some restricted capabilities for Rel-18 MUSIM UE:

-	Restricted frequencies

-	Restricted number of serving cells (CCs)

-	Restricted max number of MIMO layers

-	Restrictions and need for measurement gaps

# 2	Discussion

## 2.1 Restricted Frequencies, CC:s and MIMO layers

“Proposal 2		[10/13] UE can indicate impacted band(s) in a BC for the proactive reporting, detailled signalling is FFS.”

In the email discussion “[Post123][234][MUSIM] UE preferred frequency”, companies expressed their preferences on indicating “impacted frequencies” in the proactive approach, i.e. how the UE indicates to a Nw A that some frequencies are problematic to use with Nw A due to UE simultaneous communication with a Nw B (and Nw A should avoid to configure the UE to use those frequencies). Many companies supported a solution closely linked to the supported bands or band combinations in UE capabilities, and the following draft proposal was the result:

We fear that using some kind of delta-signalling strictly linked to the supportedBandCombination lists in UE capabilities tend to get very complex (both to complete in rel-18 time frame, and for UEs and Networks to implement), given the existing non-trivial band combination structures. In the email discussion, as well as in previous RAN2 discussions, we (as well as some other companies) have instead favoured solution that uses similar mechanisms as already used in the UAI framework. These mechanisms are not strictly coupled to the UE capability structures and parameters.

In the following, we describe the main components of this solution:

- Candidate serving frequency list

Similar to IDC feature, the network indicates a list of candidate serving frequencies (in RRCReconfiguration -> otherConfig) defined e.g. as below:

CandidateServingFreqList-r18 ::= SEQUENCE (SIZE (1..maxFreqMUSIM-r18)) OF

CandidateServingFreq-r18

CandidateServingFreq-r18 ::=     SEQUENCE {

candidateCenterFreq-r18          ARFCN-ValueNR,

candidateBandwidth-r18           ENUMERATED {khz200, khz400, khz600, khz800, mhz1, mhz2, mhz3,

mhz4, mhz5, mhz6, mhz8, mhz10, mhz20, mhz30, mhz40,

mhz50, mhz60, mhz80, mhz100, mhz200, mhz300,

mhz400}                        OPTIONAL -- Need R

}

- Restricted serving frequency list

In UIA message, the UE indicates the set of candidate frequencies that are problematic for the UE, e.g. by indicating the list indexes of the restricted frequencies in candidate frequency list received from Nw:

RestrictedServingFreqList-r18 ::= SEQUENCE (SIZE (1..maxFreqMUSIM-r18)) OF

RestrictedServingFreqIndex

RestrictedServingFreqIndex ::= INTEGER (0.. maxFreqMUSIM-1-r18)

To prevent that UE gets into “problems”, the network avoids configuring the UE with serving cells on the UE-indicated restricted frequencies.

Rel-18 MUSIM UE indicates restricted frequencies to be avoided for serving cells based on Nw-provided candidate serving frequency list.

Furthermore, mechanisms are needed such that Rel-18 MUSIM UE can indicate that its HW resources are restricted:

- Restrict the number of serving cells (CCs) in CA/DC

Existing UAI framework allows the UE to indicate reduced max number of CCs in UL and DL (for overheating and power-saving purposes).

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

Although this is rather coarse granularity that does not consider intra vs inter-band CA/DC configurations or (for intra-band case) contiguous vs non-contiguous CA configurations, we assume this should be sufficient for the Rel-18 MUSIM case, possibly complemented with a preference indication to prevent DC configurations.

Rel-18 MUSIM UE indicates preference on reduced max number of CCs in UL/DL

- Restrict the number of MIMO layers

On the number of MIMO layers, the UIA framework already have the option for UE to indicate preferred number of MIMO layers per UL/DL and per FR1/FR2-1/FR2-2. Reusing this granularity for the Rel-18 MUSIM use case might mean that the MIMO capabilities that UE can offer to a Nw-A when also connected to a Nw-B are not utilized to the full extend, still we assume this is a simple and sufficient solution for the Rel-18 MUSIM use case.

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

:

reducedMaxMIMO-LayersFR2-2          SEQUENCE {

reducedMIMO-LayersFR2-2-DL          MIMO-LayersDL,

reducedMIMO-LayersFR2-2-UL          MIMO-LayersUL

} OPTIONAL

Rel-18 MUSIM UE indicates preference on max number of MIMO layers per FRx and per UL/DL

## 2.2 Measurement gaps for Rel-18 MUSIM UE

Some UEs with full capabilities are able to measure on other frequencies without need for measurement gaps. UE capability independentGapConfig indicates that UE can measure FR2 without need for gaps. But with restricted capabilities due to MUSIM operation, this might no longer be possible, and UE now requires measurement gaps. This capability restriction should be indicated to the network, e.g. with a simple bit/flag (e.g. independentGapConfigRestricted) in UAI message.

The UE indicates that support of independentGapConfig is restricted in UAI message.

In Rel-16, the NeedForGaps feature was introduced to allow the UE to indicate its need for measurement gaps depending on the current UE configuration. This feature is based on similar framework as the UAI signalling (i.e. otherConfg in RRCReconfiguration and UEAssistanceInformation message). Based on the configuration signalled to UE in RRCReconfiguration message, UE indicates its need for gaps in RRCReconfigurationComplete. RAN2 should agree to use this feature to allow UE to indicate its need for measurement gaps, and not introduce a new similar mechanism based on UAI signalling specifically for the MUSIM case.

Rel-18 MUSIM UE that supports NeedForGap feature can use this to indicate changes in need for gap caused by Rel-18 MUSIM use cases.

For the case the UE-indicated capability restrictions in UAI message does not lead to any change of the current configuration (i.e. does not trigger the Nw to send an RRCReconfiguration message), UE will not be able to indicate its changed need for gaps in RRCReconfigurationComplete. Preferably, an indication in UAI message (same as discussed above) can trigger the network to send an RRCReconfiguration.

An indication in UAI message indicates a change in UE’s needForGaps caused by MUSIM operation, and Nw can send an RRCReconfiguration message to allow the UE to indicate its new needForGaps in the RRCReconfigurationComplete.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Rel-18 MUSIM UE indicates restricted frequencies to be avoided for serving cells based on Nw-provided candidate serving frequency list.

Proposal 2	Rel-18 MUSIM UE indicates preference on reduced max number of CCs in UL/DL

Proposal 3	Rel-18 MUSIM UE indicates preference on max number of MIMO layers per FRx and per UL/DL

Proposal 4	The UE indicates that support of independentGapConfig is restricted in UAI message.

Proposal 5	Rel-18 MUSIM UE that supports NeedForGap feature can use this to indicate changes in need for gap caused by Rel-18 MUSIM use cases.

Proposal 6	An indication in UAI message indicates a change in UE’s needForGaps caused by MUSIM operation, and Nw can send an RRCReconfiguration message to allow the UE to indicate its new needForGaps in the RRCReconfigurationComplete.