3GPP TSG-RAN WG2 Meeting #123 bis	R2-2310975

Xiamen, China, October 9-13, 2023

Agenda item:	    7.21.3

Source: 		Qualcomm Incorporated

Title: 	Discussion of PHR enhancements for coverage enhancements

Document for:	Discussion

# Introduction

In the coverage enhancement WID (R2-221858), the following enhancements were studied by RAN1 and RAN4

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

As the WI nears completion, RAN1 and RAN4 have sent LSs to RAN2 asking them to develop the framework necessary to accommodate the RAN1/RAN4 coverage enhancements in the RAN2 specs. In this contribution, we assess the latest LSs from those WGs and we outline the work needed by RAN2.

# Increasing UE power high limit for CA and DC

RAN4 has sent an LS [1] to RAN1 during RAN4 #107 with the following content:

With regard to enhanced information exchange between the UE and gNB to improve scheduling and network performance when using higher power CA/DC, RAN4 would like to provide the following recommendation and guidance as a follow-up to our earlier Reply LS in R4-2303701 from RAN4#106: - enable UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when configured duty cycle is exceed - The occasion of the report should be limited to when configured duty cycle is exceeded. - can be combined with full-power MIMO transmission capability reporting corresponding to the current power class - not to introduce P-MPR report since this is closely related to SAR implementation, which is sensitive to UE design - RAN4 stops the discussion on reporting prediction with specific evaluation periods and durations in Rel-18. - RAN4 does not consider EHR feasible.

RAN1 have investigated that LS and replied with the following to RAN4 and RAN2 [2]

RAN1 thanks RAN4 for the Reply LS on enhancements to realize increasing UE power high limit for CA and DC in R4-2310500. RAN1 discussed about the recommendations and guidance included in R4-2310500 and agreed on the following observations. Concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when configured duty cycle is exceeded: - RAN1 understands it as related to a PHR reporting enhancement by means of which Power class fallback ΔPPowerClass is reported by UE with aperiodic PHR as discussed in R4-2303560, i.e., the WF brought to RAN1’s attention by RAN4 with R4-2303701, Reply LS on enhancements to realize increasing UE power high limit for CA and DC. - The duty cycle exceedance is referred to by RAN4 as “occasion of the report”. RAN1 understands that this expression refers to the event that triggers the aperiodic PHR report, and not to the actual UL resource to send the MAC-CE carrying the report, which would be still subject to UL resource availability as per RAN1 specification. - RAN1 does not see a RAN1 impact for this enhancement. Furthermore, RAN1 agreed on respectfully ask to RAN4 the following questions: - Q1: It is RAN1 understanding that ΔPPowerClass can be triggered by the cases when the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain duty cycle as specified in Clause 6.2.4 of TS 38 101-1. Could RAN4 clarify whether all these cases can trigger ΔPPowerClass reporting in PHR MAC CE? - Q2: In case of duty cycle exceedance, and resulting ΔPPowerClass reporting as per recommendation in R4-2310500, is a further ΔPPowerClass reporting also allowed when UE returns to advertised PC power capabilities? - Q3: Could RAN4 confirm the correctness of RAN1’s understanding as per observation b) concerning the recommendation of enabling UE report on the ΔPPowerClass to indicate which power class requirements that the UE is referring to only when duty cycle is exceeded? - Q4: Could RAN4 clarify the meaning of the recommendation related to the combination of the ΔPPowerClass report with full-power MIMO transmission capability reporting corresponding to the current power class

In the meantime, RAN4 has further sent an LS to RAN1 and RAN2 clarifying the following [3]:

This LS is a supplemental LS for an LS of R4-2310500 approved in RAN4#107 which shared a recommendation and guidance with regard to enhanced information exchange between the UE and gNB to improve scheduling and network performance. Although R4-2310500 explicitly stated that the occasion of reporting ΔPPowerClass should be limited to when configured duty cycle is exceeded, it was not only what RAN4 intended to state. RAN4’s intention is reporting ΔPPowerClass should be limited to occasions when maximum transmission power changes originating from a duty cycle mechanism. Hence, the exchange of ΔPPowerClass is allowed for when maximum transmission power falls as well as it rises. In summary, the main bullet and the 1st sub-bullet in the LS are corrected as follows: - enable UE report on ΔPPowerClass to indicate which power class requirements that the UE is referring to where only ΔPPowerClass (power reduced) resulting from duty cycle exceedance or ΔPPowerClass (power return) resulting from duty cycle reduction. - The occasion of the report should be limited to either when the scheduled duty cycle exceeds the UE maximum duty cycle capability or reduces to equal to or below the UE maximum duty cycle capability after exceedance. It is also noted that RAN4 agreed that full-power MIMO transmission capability reporting corresponding to the applicable power class requirements is the only feature that can be combined with ΔPPowerClass at this writing.

## PHR Reporting Background

Power Headroom reporting is configured by RRC by configuring a few parameters (periodic timer, prohibit timer, reporting format, etc.) as part of the cell group configuration at the MAC entity. The MAC spec. specifies the triggers for reporting PHR as well as the PHR MAC CE formats.

## Enhancing PHR reporting

In issue 5 in [4], RAN4 has decided to study how PHR reporting should be enhanced for FR1 carriers to indicate the event of a power class fallback. RAN4 has indicated in [1] that it recommends “enabling UE to report ΔPPowerClass with aperiodic PHR” as part of power domain enhancements. The background is that when the scheduled UL duty cycle exceeds the maximum UE duty cycle capability the UE may perform a power class change. In this case, the NW may misinterpret the PHR in the absence of information about the power class change event.

It is noteworthy that RAN4 have added some straightforward recommendations with respect ΔPPowerClass reporting that RAN2 can take as a baseline. In particular, there are two items which RAN2 can immediately address

Observation 1: RAN4 has recommended the following enhancements with respect to PHR reporting.

- Triggering of a new PHR report upon a power class change event at the UE.

- Indicating the ΔPPowerClass to the network

Within this, it is suggested for RAN2 to discuss the following proposals.

Proposal 1: A new PHR report can be triggered by the event of UE power class change.

Proposal 2: RAN2 to discuss how ΔPPowerClass can be included in the PHR MAC CE.

Given that RAN2 may be interested in a limited enhancements for the PHR MAC CE framework without necessarily introducing a new MAC CE format, a starting point may be reusing the two MPE bits used for reporting P-MPR. Given that P-MPR reporting is configured only for FR2 and the UE power class fallback is only relevant for FR1, we may use the two bits MPE field to report ΔPPowerClass.

Observation 2: MPE field in MAC CE is only used for FR2, the MAC spec. indicates it is otherwise reserved whereas UE power class change event happens in FR1.

Proposal 3: As a baseline, RAN2 to discuss reusing the MPE field (2 bits) in FR1 to report ΔPPowerClass.

Furthermore, RAN4 has also indicated that full-power MIMO transmission capability reporting corresponding to the current power class can also be indicated to the NW during PHR reporting. Given that the notion of full-power MIMO transmission capability is somewhat ambiguous as indicated by Q4 in [2] from RAN1 to RAN4, RAN2 can wait for some RAN4 clarification or strong recommendation before deciding on whether or how it can be included it in PHR reporting framework.

Proposal 4: RAN2 to wait for further RAN1/RAN4 guidance before discussing whether and how full-power MIMO transmission capability should be included in PHR.

# Dynamic Waveform switching

In the last meeting, RAN2 discussed the LS from RAN1 [5] on dynamic waveform switching (DWS) asking RAN2 to change the specifications to accommodate the following agreements.

For reporting of power headroom information for assumed PUSCH using target waveform different from waveform of actual PUSCH, support the following: - Power headroom information for assumed PUSCH is based on an actual PUSCH transmission. - In case of no actual PUSCH transmission on a serving cell, power headroom information for assumed PUSCH is not supported. - DWS field needs to be configured for at least one DCI format for the BWP of the actual PUSCH, otherwise power headroom information for assumed PUSCH is not supported. - If actual PUSCH transmission is with DFT-S-OFDM waveform, UE computes power headroom information of an assumed PUSCH with CP-OFDM waveform. If actual PUSCH transmission is with CP-OFDM waveform, UE computes power headroom information of an assumed PUSCH with DFT-S-OFDM waveform. - All parameters that are used for the calculation of PCMAX,f,c(i), except waveform, are the same between assumed PUSCH and actual PUSCH. - In case assumed PUSCH transmission is not supported for the parameters that are used for the calculation of PCMAX,f,c(i), power headroom information for assumed PUSCH is not computed or reported. - Power headroom information for assumed PUSCH contains: - PCMAX,f,c(i) of assumed PUSCH - Accounting for applicable MPR, A-MPR and P-MPR for the assumed PUSCH. - If UE reports power headroom information for assumed PUSCH in a PUSCH transmission, legacy PHR is also reported in the same PUSCH transmission. - No consensus in RAN1 if the following applies or not: if UE reports legacy PHR in a PUSCH transmission, power headroom information for assumed PUSCH is also reported. - Note: RAN endorsed the following at RAN#100: “RAN2 will not work on PHR triggering procedure for dynamic waveform switching in Rel-18 UL Coverage enh WI” [RP-231498]. Send LS to RAN2 to inform above agreement.

RAN2 has briefly discussed that in RAN2 #123 with the following agreement reached:

Some RAN2 work is essential to enable this. RAN2 thinks the plenary agreement allows further RAN2 work on this. So, we can discuss this at next meeting. (But this doesn’t necessarily mean that we will be able to finish this work without further RAN1 input as there are still FFSs in RAN1)

During the online discussion, there was some debate about whether RAN2 has all the information or whether there are still open issues in RAN1 that need to be resolved. After some discussions with our RAN1 colleagues, we think that RAN2 has sufficient information to work on this. As a baseline, the PHR reporting in the MAC spec would need to capture the above RAN1 agreements on the reporting of an assumed PUSCH using target waveform different from waveform of actual PUSCH. No new PHR formats or triggers are anticipated.

Proposal 5: RAN2 to update the PHR reporting procedure in the MAC spec to cover the case when reporting an assumed PUSCH using target waveform different from the waveform of the actual PUSCH. No further RAN2 impact is expected.

# Conclusion

Observation 1: RAN4 has recommended the following enhancements with respect to PHR reporting.

- Triggering of a new PHR report upon a power class change event at the UE.

- Indicating the ΔPPowerClass to the network

Proposal 1: A new PHR report can be triggered by the event of UE power class change.

Proposal 2: RAN2 to discuss how ΔPPowerClass can be included in the PHR MAC CE.

Observation 2: MPE field in MAC CE is only used for FR2, the MAC spec. indicates it is otherwise reserved whereas UE power class change event happens in FR1.

Proposal 3: As a baseline, RAN2 to discuss reusing the MPE field (2 bits) in FR1 to report ΔPPowerClass.

Proposal 4: RAN2 to wait for further RAN1/RAN4 guidance before discussing whether and how full-power MIMO transmission capability should be included in PHR.

Proposal 5: RAN2 to update the PHR reporting procedure in the MAC spec to cover the case when reporting an assumed PUSCH using target waveform different from the waveform of the actual PUSCH. No further RAN2 impact is expected.

# References

[1] R4-2310500 Reply LS on enhancements to realize increasing UE power high limit for CA and DC

[2] R2-2309420 Reply LS on enhancements to realize increasing UE power high limit for CA and DC

[3] R2-2309468 LS on enhancements to realize increasing UE power high limit for CA and DC (R4-2314728; contact: Nokia

[4] R4-2303560 WF on coverage enhancement part 1

[5] R2-2309256	LS on Details of power headroom information for assumed PUSCH (R1-2308477; contact: InterDigital)