3GPP TSG-RAN WG2 Meeting #123bis	R2-2310994

# Xiamen, China, 09 – 13 October 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4367 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On-demand SI request corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | requested-SI-List is a bitmap that must point ONLY to SI messages that are on-demand SI messages (i.e., notBroadcasting status) appearing in schedulingInfoList. The current description says first bit corresponds to first listed SI message, second bit corresponds to second listed SI message, and so on, but the first listed and second listed SI messages could also be a broadcast SI message. A similar problem is also seen in the field description of requestedPosSI-List, although the scheduling list involved in this case is posSchedulingInfoList. Note that for the msg1-based on-demand SI request where the si-RequestResources field is used to point to the SI messages, this ambiguity does not exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the field description for requested-SI-List and requestedPosSI-List to say that the bits in these bitmaps point to SI messages for which the broadcast status is set to notBroadcasting. “Implementation of this CR by a Release 15 UE will not cause compatibility issues” Impact analysis Impacted functionality: Msg3-based on-demand SI request. Inter-operability: If the network is implemented according to the CR and the UE is not or vice versa, the network and the UE may have a different view of where the on-demand SI messages are present in a scheduling information list and the UE request for an on-demand SI message may fail. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network and UE may have a different view of where the on-demand SI messages are present in a scheduling information list and the UE request for an on-demand SI message may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 6.2.2	Message definitions

#### –	RRCSystemInfoRequest

The RRCSystemInfoRequest message is used to request SI message(s) required by the UE as specified in clause 5.2.2.3.3 and 5.2.2.3.3a.

Signalling radio bearer: SRB0

RLC-SAP: TM

Logical channel: CCCH

Direction: UE to Network

RRCSystemInfoRequest message

-- ASN1START

-- TAG-RRCSYSTEMINFOREQUEST-START

RRCSystemInfoRequest ::=            SEQUENCE {

criticalExtensions                  CHOICE {

rrcSystemInfoRequest                RRCSystemInfoRequest-IEs,

criticalExtensionsFuture-r16        CHOICE {

rrcPosSystemInfoRequest-r16         RRC-PosSystemInfoRequest-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

}

RRCSystemInfoRequest-IEs ::=    SEQUENCE {

requested-SI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                               BIT STRING (SIZE (12))

}

RRC-PosSystemInfoRequest-r16-IEs ::=  SEQUENCE {

requestedPosSI-List                   BIT STRING (SIZE (maxSI-Message)),  --32bits

spare                                 BIT STRING (SIZE (11))

}

-- TAG-RRCSYSTEMINFOREQUEST-STOP

-- ASN1STOP

RRCSystemInfoRequest-IEs field descriptions

requested-SI-List A bitmap of requested SI messages. According to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1, first/leftmost bit in bitmap corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, the second bit in bitmap corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, and so on.

requestedPosSI-List A bitmap of requested SI messages. According to the order of entry in the list of SI messages configured by posSchedulingInfoList in posSI-SchedulingInfo in SIB1, first/leftmost bit in bitmap corresponds to the first SI message in posSchedulingInfoList for which posSI-BroadcastStatus is set to notBroadcasting, the second bit in bitmap corresponds to the second listed SI message in posSchedulingInfoList for which posSI-BroadcastStatus is set to notBroadcasting, and so on.

End of Changes