3GPP TSG-RAN WG2 Meeting #123bis	R2-2311005

Xiamen, China, 9 - 13 October 2023

Agenda Item:	7.20.4

Source:	Huawei, HiSilicon

Title:	Overlapping UL grants handling for STxMP

Document for:	Discussion and decision

# 1. Introduction

In the Rel-18 MIMO WID [1], one of the objectives with RAN2 impact is STxMP (Simultaneous Transmission across Multiple Panels), as shown in the following description.

6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission • The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation • For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we discuss the potential impacts on RAN2 about overlapping UL grants handling for STxMP UEs.

# 2. Discussion

## 2.1 Rel-15 overlapping UL grants handling

In Rel-15, the UE only uses a configured grant (CG) occasion when the CG PUSCH duration does not overlap with the dynamic grant (DG), which means the UE will not use the CG occasion when it overlaps with DG. This is specified as shown in the TS 38.321 v15.13.0:

TS 38.321 v15.13.0 ## 5.4 UL-SCH data transfer ### 5.4.1 UL Grant reception […] For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall: 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if the configuredGrantTimer for the corresponding HARQ process is not running: 3> consider the NDI bit for the corresponding HARQ process to have been toggled; 3> deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

Observation 1: In Rel-15, the UE will not use the CG when it overlaps with DG.

In Rel-15, there is only one activated CG resource in a BWP, so there is no need to specify the case when CG overlaps with CG.

## 2.2 Rel-16 overlapping UL grants handling

In Rel-16, there are more than one activated CG resources in a BWP, so how to handle the CGs overlapping was discussed. There is a note saying that the UE chooses one configured grant by UE implementation, as shown in the extracted specification below.

TS 38.321 v16.12.0 ## 5.4 UL-SCH data transfer ### 5.4.1 UL Grant reception […] NOTE 7: If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

Observation 2: In Rel-16, when lch-basedPrioritization is not configured, if there are overlapping CGs, the UE selects one CG by UE implementation.

The RRC parameter lch-basedPrioritization is introduced in intra-UE prioritization of Rel-16 Industrial IoT. It specifies how the UE deals with two overlapping configured grants and with a configured grant and a dynamic grant overlapping with each other. In Rel-16, the UE is not required to use two UL grant resources simultaneously. Therefore, when the above cases happen, the UE deprioritizes one of the UL grant according to the priority rule. The priority value of an uplink grant is determined by the highest priority among priorities of the LCHs with data available that are multiplexed or can be multiplexed in the MAC PDU.

Observation 3: In Rel-16, when lch-basedPrioritization is configured, if two UL grants (CG and CG or CG and DG) have time domain overlap, the UE deprioritizes one UL grant with lower priority and does not use this deprioritized UL grant resource.

## 2.3 Rel-18 STxMP overlapping UL grants handling

In Rel-18 MIMO, according to the WID [1] and RAN1 agreements, STxMP is supported for PUSCH and PUSCH simultaneous transmission, and PUCCH and PUCCH simultaneous transmission. Regarding PUSCH and PUSCH simultaneous transmission, from RAN2 perspective, the UE needs to be able to use two UL grants which have time domain overlap.

Observation 4: In Rel-18 STxMP, the UE is able to use two UL grants when they have time domain overlap, instead of using only one of the UL grant, which is different from the Rel-15/16 behaviour.

There are mDCI STxMP and sDCI STxMP. In mDCI STxMP, there could be two UL grants for two TRPs. With the current specification, the UE will only use one of the UL grants, which is not the intention of STxMP. Therefore, for mDCI STxMP, RAN2 need to discuss how to specify the UE behaviour. Regarding sDCI STxMP scenario, the current specification seems fine since there is only one UL grant. RAN2 can discuss mDCI STxMP first.

In mDCI STxMP, when there are two overlapping UL grants associated with two panels/TRPs, the UE uses both UL grants simultaneously.

Another issue is whether the intra-UE prioritization (lch-basedPrioritization) can be configured together with Rel-18 STxMP. If they can be configured together, the UE prioritizes one UL grant and de-prioritizes others only when the UL grants are associated with the same panels/TRPs. For instance, if CG1 and CG2 are both associated with panel1/TRP1, then the UE needs to prioritize one CG. If CG1 and CG2 are associated with different panels/TRPs, the UE can use both UL grants.

Discuss whether intra-UE prioritization (lch-basedPrioritization) can be configured together with STxMP. If yes, then the UE prioritizes one UL grant and de-prioritizes others only when the UL grants are associated with the same panel/TRP.

# 3. Conclusion

Based on the discussion in this contribution, we have the following proposals:

- In mDCI STxMP, when there are two overlapping UL grants associated with two panels/TRPs, the UE uses both UL grants simultaneously.

- Discuss whether intra-UE prioritization (lch-basedPrioritization) can be configured together with STxMP. If yes, then the UE prioritizes one UL grant and de-prioritizes others only when the UL grants are associated with the same panel/TRP.

# 4. Reference

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung