| 3GPP TSG RAN WG2 Meeting #123bis |  | R2-2311017 |
| --- | --- | --- |
| Xiamen, China, October 9th – 13th, 2023 |  |  |
| Agenda item: | 7.9.2 |  |
| Source: | Fraunhofer IIS, Fraunhofer HHI |  |
| Title: | Discussion on remaining issues on U2U Relaying |  |
| Document for: | Discussion and Decision |  |

# Introduction

In RAN2#123 the following agreements were captured in the preliminary chairman notes [1]:

Agreements: UE in RRC_CONNECTED state can obtain UE-to-UE relay discovery parameters in dedicated discovery configuration. For integrated discovery DCA message, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE. For Model B, the relay UE forwards the solicitation message only if the PC5 RSRP between the relay UE and the source remote UE is above a threshold. For Model B, no AS criterion is needed for the relay UE to forward the response message to the source Remote UE. E2E SL-SRB and E2E SL-DRB use different index(es). Fixed index (i.e., 0/1/2/3) are defined for E2E SL-SRB 0/1/2/3 respectively. Use specified PC5 RLC Channel configuration on each hop for E2E SL-SRB 0/1/2/3. The TX Remote UE derives the PDCP and SDAP configuration for e2e SL-DRB and provides the portion of the configuration related to RX to the RX Remote UE using E2E PC5-RRC message (similar to legacy PC5 configuration). The TX Remote UE derives the first hop configuration (e.g. PC5 relay RLC Channel configuration) for SL-DRB and provides to the relay UE the portion of the configuration related to RX on the first hop (i.e., Rx by the relay UE), using per-hop PC5-RRC message (similar to legacy PC5 configuration). The two conclusions above do not exclude the derivation involving information from gNB/preconfiguration/specified configuration. Split PDB is sent to the source (TX) Remote UE from the Relay UE. It is left to Relay UE implementation on how to split the PDB. The Relay UE derives the second hop configuration (e.g. PC5 relay RLC Channel configuration) for each SL-DRB. It is FFS how the Relay UE derives second hop configuration for SL-DRB. Same as L3 based U2U relay, the QoS split should be per e2e QoS flow, and RAN2 expect that the source UE will inform the Relay UE QoS flow(s) and corresponding QoS profiles. FFS if this requires AS signalling or can be done in upper layers. At least PDB is sent from the source UE to the relay UE for splitting. The source UE sends to the Relay UE all the QoS profiles for the e2e QoS flows. At least for single-hop relay, use local ID instead of L2 ID as UE ID in SRAP header. At least for single-hop U2U relay, two local IDs are included in SRAP header to identify source and target Remote UE respectively. FFS impact on SRAP header. For single-hop U2U relay, the local ID for a particular UE is the same on both hops. New specified per-hop configurations are used for E2E SL-SRB 0/1/2/3 respectively. FFS how they will be implemented in specs (e.g., if the configurations are identical the tables might be merged for different SL-SRBs).

Still, there have been open isues that have also been capured during the post-meeting e-mail discussion on “Remaining open issues”.

This document presents the view on selected open issues.

# SRAP Design

In R2#121 [2] it was agreed to include two local IDs in the SRAP header at least for the single-hop U2U relay use case. The local ID is supposed to be assigned by the relay UE and propagated to the remote UEs via RRC messages.

Local ID can be set via PC5 RRC signalling.

It is important that any solution specified that involves local ID and/or SRAP header definitions is forward compatible to support multi-hop in the future. Therefore, the local ID shall be defined in a way that I can also be used for multi-hop U2U and potentially multi-hop U2N relaying.

The local ID should be defined in a way that is forward compatible with multi-hop, also for multi-hop U2N relaying.

If the relay UE is IC and in CONNECTED mode or the relay is in IC but the remote UEs are OOC, the local ID assignment could also be done by the gNodeB via the relay UE. This could also mitigate any ID collisions that can happen in case a remote UE is connected to two relays and two different remote UEs. The relay UE could potentially assign the same local ID as the neighbour relay.

Local ID can be assigned by the gNodeB via the relay UE for the in-coverage and partial coverage use case, where the relay UE has a Uu connection.

RAN 2 should discuss whether this behaviour is suitable and consider implications for multi-hop scenarios.

RAN 2 to discuss the assignment of local ID by a gNodeB.

Because the multi-hop scenario is out-of-scope for this WI, discussion regarding multi-hop procedures should be postponed until a Rel-19 WI is eventually started.

# Relay Re-Selection Triggers

During the last RAN 2 meetings there have been discussions about how U2U remote UEs act on PC5-RLF detection or trigger a relay/path re-selection based on link quality. It was agreed in RAN2#121 that each of the remote UEs can “trigger relay re-selection based at least on current hop quality”.

The hop quality can be mainly based on the PC5-RSRP measurements. In RAN2#120 there was already an FFS in the agreements whether and how the second hop should be considered.

The quality of the link between the two remote UEs via the relay UE can only be as good as the weakest link in the relayed connection, thus it is a good practice to at least consider the link quality of the second hop as equally important as the first hop for the overall connection.

Remote UEs shall perform (relay) re-selection upon receiving information from the relay about the PC5 link quality of the second hop being below a threshold.

The link quality can be based on the RSRP measurements alone or being enhanced with additional measurements, e.g. CBR, QoS parameters. It is worth discussing whether other triggers (event-based) based in information from the relay UE can lead to a re-selection as well, e.g. system properties of the relay. Give the time constraints in Rel-18 this could also be a potential Re-19 topic.

Consider other re-selection triggers based on system information from the relay UE. This could be a Rel-19 topic, if aggreed in RAN#102.

# Message Definitions

In R2-2309755 [3] a number of open issues were listed, one of them pointing out a FFS for the QoS flow and corresponding QoS profile information, related to QoS split, from the source UE to the relay UE. The question is whether AS signalling is required or whether this can be done in upper layers.

We prefer to use AS signalling to reduce the amount of signalling. The setup can be done during the initial setup of the data flow.

AS signalling is used for setting up a QoS split in an e2e QoS flow.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1:	Local ID can be set via PC5 RRC signalling.

Observation 2:	Local ID can be assigned by the gNodeB via the relay UE for the in-coverage and partial coverage use case, where the relay UE has a Uu connection.

Proposal 1:	The local ID should be defined in a way that is forward compatible with multi-hop, also for multi-hop U2N relaying.

Proposal 2:	RAN 2 to discuss the assignment of local ID by a gNodeB.

Proposal 3:	Remote UEs shall perform (relay) re-selection upon receiving information from the relay about the PC5 link quality of the second hop being below a threshold.

Proposal 4:	Consider other re-selection triggers based on system information from the relay UE. This could be a Rel-19 topic, if aggreed in RAN#102.

Proposal 5:	AS signalling is used for setting up a QoS split in an e2e QoS flow.

# References

[1] Chairman Notes RAN2#123, RAN2-123-Positioning-Relay-2023-08-25-1320.docx

[2] R2-2307001 - Report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea

[3] R2-2309755 - Report of [Post123][Relay] Remaining open issues (LG)