3GPP TSG-RAN WG2 #123-bis	Tdoc R2-2311030

Xiamen, China, 2023-10-09 – 2023-10-13

Agenda Item:	7.5.4.1

Source:	Ericsson

Title:	Discussion on BSR enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss BSR enhancements related to the new BSR table to be introduced and the delay reporting.

In the RAN2#123 meeting the following agreement was made for BSR table:

- RAN2 needs to discuss the BSR table definition in the next meeting based on company inputs.

For delay reporting the following was agreed:

- Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG.

- When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time.

- RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE.

- Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

- Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

In this contribution we discuss how to proceed on these topics based on above agreements.

# 2	Discussion on BSR enhancements for XR

## 2.1	Buffer reporting

### 2.1.1	Exponential table range of new BSR table

RAN2 has now made the agreement that one new static table should be added which should try to cover all traffic cases. As has been discussed this would imply a large range since many types of future traffic variations need to be supported. A proposal was earlier been made on a linear table with the range of 44 289 bytes (Bmin) to 781 250 bytes (Bmax), hereby called QLinear. This 8-bit table thus have a step size of 2879 bytes if it uses linear distribution. However in last RAN2 meeting the possibility of using exponential distribution was also brought up. Similar to legacy tables such table would have shorter step sizes at low range and longer step sizes at upper range in the table. This table is hereby called QExponential. Through simulations the impact of introducing these table types can be evaluated. Relevant for analysing the impact is to make comparisons to both legacy and other possible tables. Simulation results for such evaluations is shown in Figure 1. As can be seen there is a significant change in the amount of supported eMBB traffic at a specific XR capacity with different tables. With finer granularity tables the amount of supported eMBB users can be increased. For linear tables improvements can be observed all the way down to a small step size of 200 bytes. However also the QExponential show performance of close to the best linear tables. A noteworthy detail is that with a smaller step size the Bmin can also be set lower, both for exponential and linear tables. This is preferred as having a lower quantization error for lower buffer values has been shown to be beneficial.

Figure 1 XR capacity at various eMBB load points with different table ranges and step sizes (in bytes).

- An exponential table distribution can achieve performance similar to short range high granularity linear tables while also improving the step size granularity up to the estimated maximum buffer sizes for high rate XR traffic.

The conclusion from these results is that there is room for more gains with finer granularity tables than the linear table that has so far been proposed. Preferably RAN2 should find a solution that captures a majority of these gains. Leaving out the configurable table solution, that was downvoted in earlier RAN2 meeting, the simplest solution for this seems to be the exponential range of the new fixed table.

RAN2 should use an exponential distribution for the new XR table.

### 2.1.3	BSR format

A new long BSR format should continue reporting the buffer size for those LCGs which do have data in the buffer. There are two questions to resolve:

- Can two BS tables be configured per each LCG? And,

- How to indicate if the UE is using legacy BS table or a new BS table?

A new additional bit Ti could be added per logical channel, in total one byte, indicating whether the new or the legacy table is used. Each bit would indicate whether the legacy table is used or the new table is used, as shown in Figure 2. In this example, 2 LGCs would set the T flag to “1”, while 1 LCG would set the T flag to “0” to indicate it is using legacy table.

Figure 2 – Example of new table/BS format

Therefore a new BSR MAC CE should be created in TS38.321, where the difference from the legacy BSR MAC CE would be that one byte is added with the individual bits indicating if the correspondent LCG is using the legacy or the new BS table.

Create a new BSR MAC CE based on the legacy BSR MAC CE, with one extra byte, where the individual bits are indicating whether the correspondent LCG is using the legacy or the new BS table, as shown in example in Figure 2.

### 2.1.4	BSR operation and triggers

When the BSR is triggered and the UE calculates the total buffer size of the LCG, there may be the cases where both the legacy and the new table contains an index which can represent the UE buffer size. In those cases the new table should be used.

When the buffer size value is outside the range covered in the new table, i.e. when only legacy table contains an index which can represent the UE buffer size, the UE shall then use the legacy BS table and formats.

When both the legacy and the new BS table contain an index which can represent the UE buffer size, the UE shall use the new BS table and format.

When only one table (i.e. the legacy BS table) contains an index which represents the UE buffer size, the UE shall use the legacy BS table and format.

Legacy BSR triggering are considered suitable to meet the QoS requirements. However, it can be investigated if other triggers show additional benefits. On the other hand, XR traffic with long periodicity of data may still benefit from frequent BSR transmissions but this can be accommodated by configuration of frequent periodic BSR. These aspects need to be considered if new triggers are suggested.

Current BSR triggering conditions are the baseline conditions for the new BSR introduced in Section 2.1.

## 2.2	Delay reporting

During the Rel-18 SI simulations were performed showing that adding delay information to the BSR is beneficial for XR capacity by utilizing the information in a delay scheduler [2]. Delay scheduling works by prioritizing users efficiently by utilizing early delay information. This led to the conclusion that some delay information should be reported coupled with buffer data [1]. In the last RAN2 meeting it was agreed that a new MAC CE called DSR (Delay Status Report) should be defined which is separate from the legacy BSR MAC CEs. This format should have its own report triggers not coupled with the BSR triggering. Delay information should be reported with associated buffer information in this new DSR format. The triggering of he DSR should be based on NW configured thresholds.

Claims have been made that only a single value of delay information should be enough to report. However, it can be shown in examples that reporting without any granularity on the time scale will not work well in all scenarios, e.g. when there are several PDU Sets from one traffic flow at the same time in the buffer they will have different time left until deadline and the network scheduler need to be aware of this to make the correct scheduling decisions (see appendix A for detailed example of various time reporting scenarios). It is thus important that the delay reporting introduced should support reporting on a more granular level than only a single value for each LCG. It should be noted that XR traffic has high latency requirement and thus data will not reside in the buffer for a very long time, or it will anyway be regarded as useless and discarded. It can thus be assumed that there will never be a need for a large number of delay values to be reported. Further as there may be multiple traffic flows with different delay requirements ongoing from a UE the delay reporting should be done per LCG where preferably only LCIDs with similar delay requirements are grouped.

Considering above and the agreement that DSR is triggered based on threshold. The straight forward solution is to have the network configure the suitable number of thresholds that it thinks is beneficial and then the UE reports the data that belong to each threshold (e.g. the delay range or the delay bucket), see Figure 3 as an example how such configuration can look. By indicating in the DSR which threshold the data belong to the network will know the remaining time for the associated data. It should be noted that reporting the exact delay value is not needed since configuring the suitable thresholds will provide the granularity needed.

Figure 3 Example of configuration of multiple thresholds (delay buckets). The example contains three thresholds at 5ms, 10ms and 15ms.

DSR supports configuration of multiple thresholds and reporting of the associated data for each threshold.

### 2.2.1	Delay reporting configuration

Since delay/latency reporting can be done in a similar fashion to the buffer size reporting procedure, by reporting an index which would indicate a delay threshold, these delay thresholds can be configured by the network and indicated via RRC. In this way the granularity can be flexible around the most relevant parts, i.e. the lower indexes indicate a smaller delay range in the table. The tables could be configured by the network by indicating each threshold value. With this approach there is more flexibility of the ranges where the scheduler thinks the precision need to be higher.

A delay table per LCG can be configured.

The delay table can be defined by explicit indication of the threshold values.

Up to 8 thresholds can be configured.

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

…

[[

DelayTablesList-r18  					SEQUENCE(Size (1..maxLCG (8))) of DelayTableList    						OPTIONAL

]]

DelayTableList ::=            SEQUENCE {

logicalChannelGroupIndex					ENUMERATED (0..maxLCG-ID),

delayThresholdConfigList					SEQUENCE(Size (1..maxThresholds (8))) of DelayThresholdConfig

}

DelayThresholdConfig ::=            SEQUENCE {

thresholdValue               			INTEGER (xx..yy),

triggerDelayReport						ENUMERATED (TRUE),					OPTIONAL

}

Adopt the ASN.1 outlined above to configure the delay table.

### 2.2.2	Delay reporting operation and triggers

As the simulations show [2] early delay information is important for the scheduler to do smart selection of users. Thus the delay information should be reported as early as possible, preferably it is already sent when new data arrives to the UE buffer. Then if data has waited a while in the buffer and got closer to deadline, it could trigger new DSR reports which then includes the updated delay values since this provides network with more accurate estimates of remaining time. The frequency of the DSR reporting could be controlled by network configuring which thresholds that should be triggering DSR reports.

Delay reporting is triggered when remaining time for data pass selected thresholds. The thresholds which trigger the delay reporting are configured by the network, as exemplified in proposal 10.

### 2.2.3	Delay reporting format

#### New long delay reporting

It would be ideal that the delay reporting is as similar as possible to the BSR reporting adding, on top, a delay indication e.g., an index. Some companies suggested to report for one threshold, the most priority one. This solution would require at least 1 bit to indicate if the threshold is included or not. Adding 1 bit will, in any case, lead to add one extra byte in the format, if the BS format is used as a baseline. Thus, if one extra byte is added, it is better to report all thresholds which have data.

There are multiple possible formats to convey bucket and buffer status. Here we show the formats which comply with that the report should include multiple thresholds, an indication of the table, and the buffer size for each of those.

For each LCG that has been configured with “delay reporting”, when the LCG has data, the reporting will include two fields indicating the presence of the buffer status for a threshold, and the table used for that buffer status. Figure 5 exemplifies how the format would look like. There are 8 thresholds and the new table or the legacy table can be indicated for each of the thresholds. In the case of all thresholds containing data, 8 1-byte BS fields would follow but this will rarely be the case. For most of the cases, only few LCGs will be configured with delay reporting and only a few delays thresholds will contain data. In this example the report contains delay reports for two LCGs. The first LCG only reports buffer status in one delay threshold and the second LCG reports buffer status in two delay thresholds.

Figure 5 – Example of the delay reporting format.

One bit is used to indicate the presence of data in a threshold.

1 byte per LCG that reports delay is introduced to indicate 8 thresholds. BS is reported as described in Figure 2.

The eLCID (1 octet) is used to for this new delay reporting MAC CE.

# 3 Conclusion

In the previous sections we made the following observations:

- An exponential table distribution can achieve performance similar to short range high granularity linear tables while also improving the step size granularity up to the estimated maximum buffer sizes for high rate XR traffic.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should use an exponential distribution for the new XR table.

Proposal 2	Create a new BSR MAC CE based on the legacy BSR MAC CE, with one extra byte, where the individual bits are indicating whether the correspondent LCG is using the legacy or the new BS table, as shown in example in Figure 2.

Proposal 3	When both the legacy and the new BS table contain an index which can represent the UE buffer size, the UE shall use the new BS table and format.

Proposal 4	When only one table (i.e. the legacy BS table) contains an index which represents the UE buffer size, the UE shall use the legacy BS table and format.

Proposal 5	Current BSR triggering conditions are the baseline conditions for the new BSR introduced in Section 2.1.

Proposal 6	DSR supports configuration of multiple thresholds and reporting of the associated data for each threshold.

Proposal 7	A delay table per LCG can be configured.

Proposal 8	The delay table can be defined by explicit indication of the threshold values.

Proposal 9	Up to 8 thresholds can be configured.

Proposal 10	Adopt the ASN.1 outlined above to configure the delay table.

Proposal 11	Delay reporting is triggered when remaining time for data pass selected thresholds. The thresholds which trigger the delay reporting are configured by the network, as exemplified in proposal 10.

Proposal 12	One bit is used to indicate the presence of data in a threshold.

Proposal 13	1 byte per LCG that reports delay is introduced to indicate 8 thresholds. BS is reported as described in Figure 2.

Proposal 14	The eLCID (1 octet) is used to for this new delay reporting MAC CE.

# 4 References

- R2-2304711, “BSR and delay status reporting for XR ”, Qualcomm Incorporated, 3GPP TSG RAN WG2 #122, 22-26 May, 2023.

- R2-2301507, “Discussion on XR-specific capacity improvements”, Ericsson, 3GPP TSG-RAN WG2 #121, Feb.-Mar. 2023.

- 3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”

# Appendix A

Shown in this appendix is a number of examples on how different delay information will possibly impact the scheduling. The scenarios use only two UEs with different traffic arrival. The packet delay requirement (PDB) and traffic periodicity changes between scenarios. Different solution for delay information reporting is utilized in the different scenarios. For simplicity constant bitrates is assumed, i.e. the same amount of resources is scheduled every time slot.

Scenario 1, PDB=2: No delay information

Scheduling taking place at t=2 is not optimal since no delay information is available (a round robin scheme is utilized).

Scenario 1, PDB=2: Single value delay information

Scheduling with delay information make it possible for the scheduler to utilize delay scheduling and make the correct decision in t=2.

Scenario 2, PDB=3: No delay information

In a more complicated scenario (e.g. higher periodicity, jitter, congestion) multiple packets simultaneously will exist in the buffer and without delay information more packets risk to miss the deadline target.

Scenario 2, PDB=3: Single delay information

With a single delay value reported the scheduler can improve the situation also in the complicated scenario but still risk to make the incorrect decision (t=4) since it doesn’t know that the packets belonging to another PDU Set should belong in a different delay bucket.

Scenario 2, PDB=3: Delay buckets

With a delay bucket solution the scheduler can get the delay granularity to take the optimal decision at all time instances.