3GPP TSG-RAN WG2 Meeting #123-bis											  	   R2-23xxxxx

# Xiamen, CN Oct 09 – 13, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | xxx | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the capability independentGapConfig-maxCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In previous meeting RAN2#123, per the post meeting discussion (R2-2309610), it was agreed that the description of the independentGapConfig-maxCC need further clarifications. Based on the discussion there were 2 options provided to address this request. Providing a summary table below to highlight the differences betweent the 2 options: UE Capability Container; UE capability fields; Type of Serving cells considered in the count of Option-1; Type of Serving cells considered in the count of Option-2. NR SA; fr1-Only-r17; FR1; FR1. fr2-Only-r17; FR2; FR2. fr1-AndFR2-r17; FR1+FR2; FR1+FR2. NR-DC; fr1-Only-r17; FR1+FR1; FR1+FR1. fr2-Only-r17; FR2+FR2; FR2+FR2. fr1-AndFR2-r17; FR1+FR2; FR1+FR2. MR-DC; fr1-Only-r17; LTE+FR1; LTE+FR1. fr2-Only-r17; LTE+FR2; Null. fr1-AndFR2-r17; LTE+FR1+FR2; LTE+FR2 LTE+FR1+FR2. The suggested changes of the parameters’ description in this CR are based on Option-1, as it seems more straight forward and consistent across all parameters. |
| Summary of change: |  | - Clarify the interpretation of these parameters when UE advertises independentGapConfig-maxCC capability in EN-DC. - Differentiate the description between NR-SA and NR-DC. - Replacing the term “per-FR Gap” by “the feature” Impact Analysis: Impacted 5G architecture options: LTE, (NG)EN-DC, NR-DC, NR-SA Impacted functionality: Independent gap configuration feature Interoperability issue: - if the network is implemented according to the CR and the UE is not, misalignment in the interpretation of the independentGapConfig-maxCC capability may occur, causing gap misconfiguration. - if the UE is implemented according to the CR and the network is not, misalignment in the interpretation of the independentGapConfig-maxCC capability may occur, causing gap misconfiguration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Expected network and UE behavior when configuring the gap at the UE remains undefined for the EN-DC and LTE case. |  |  |  |  |  |  |  |  |
| d |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 38.306 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Start

### 6.3.3	UE capability information elements

#### <<skipped>>

#### –	MeasAndMobParameters

The IE MeasAndMobParameters is used to convey UE capabilities related to measurements for radio resource management (RRM), radio link monitoring (RLM) and mobility (e.g. handover).

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParameters-v1700 ::=          SEQUENCE {

measAndMobParametersFR2-2-r17           MeasAndMobParametersFR2-2-r17           OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

-- R4 19-2 Concurrent measurement gaps

concurrentMeasGap-r17                   CHOICE {

concurrentPerUE-OnlyMeasGap-r17         ENUMERATED {supported},

concurrentPerUE-PerFRCombMeasGap-r17    ENUMERATED {supported}

}                                                                               OPTIONAL,

-- R4 19-1 Network controlled small gap (NCSG)

nr-NeedForGapNCSG-Reporting-r17         ENUMERATED {supported}                  OPTIONAL,

eutra-NeedForGapNCSG-Reporting-r17      ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-1 per FR Network controlled small gap (NCSG)

ncsg-MeasGapPerFR-r17                   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-2 Network controlled small gap (NCSG) supported patterns

ncsg-MeasGapPatterns-r17                BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-1-3 Network controlled small gap (NCSG) supported NR-only patterns

ncsg-MeasGapNR-Patterns-r17             BIT STRING (SIZE(24))                   OPTIONAL,

-- R4 19-3-2 pre-configured measurement gap

preconfiguredUE-AutonomousMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-3-1 pre-configured measurement gap

preconfiguredNW-ControlledMeasGap-r17   ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2-2-r17                   ENUMERATED {supported}                  OPTIONAL,

handoverFR2-1-FR2-2-r17                 ENUMERATED {supported}                  OPTIONAL,

-- RAN4 14-1: per-FR MG for PRS measurement

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL,

rrm-RelaxationRRC-ConnectedRedCap-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 25-3: Parallel measurements with multiple measurement gaps

parallelMeasurementGap-r17              ENUMERATED {n2}                         OPTIONAL,

condHandoverWithSCG-NRDC-r17            ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-r17              ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-ENDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NEDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NRDC-r17         ENUMERATED {supported}                  OPTIONAL,

gNB-ID-LengthReporting-NPN-r17          ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R4 25-1: Parallel measurements on multiple SMTC-s for a single frequency carrier

parallelSMTC-r17                        ENUMERATED {n4}                         OPTIONAL,

-- R4 19-2-1 Concurrent measurement gaps for EUTRA

concurrentMeasGapEUTRA-r17              ENUMERATED {supported}                  OPTIONAL,

serviceLinkPropDelayDiffReporting-r17   ENUMERATED {supported}                  OPTIONAL,

-- R4 19-1-4 Network controlled small gap (NCSG) performing measurement based on flag deriveSSB-IndexFromCellInter

ncsg-SymbolLevelScheduleRestrictionInter-r17  ENUMERATED {supported}            OPTIONAL

]],

[[

eventD1-MeasReportTrigger-r17           ENUMERATED {supported}                  OPTIONAL,

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               OPTIONAL

]],

[[

interSatMeas-r17                            ENUMERATED {supported}              OPTIONAL,

deriveSSB-IndexFromCellInterNon-NCSG-r17    ENUMERATED {supported}              OPTIONAL

]],

[[

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                     OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                     OPTIONAL

}                                                                               				OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=        SEQUENCE {

intraAndInterF-MeasAndReport            ENUMERATED {supported}                  OPTIONAL,

eventA-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

...,

[[

handoverInterF                          ENUMERATED {supported}                  OPTIONAL,

handoverLTE-EPC                         ENUMERATED {supported}                  OPTIONAL,

handoverLTE-5GC                         ENUMERATED {supported}                  OPTIONAL

]],

[[

sftd-MeasNR-Neigh                       ENUMERATED {supported}                  OPTIONAL,

sftd-MeasNR-Neigh-DRX                   ENUMERATED {supported}                  OPTIONAL

]],

[[

dummy                                   ENUMERATED {supported}                  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB                ENUMERATED {supported}              OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB             ENUMERATED {supported}              OPTIONAL,

csi-SINR-Meas                               ENUMERATED {supported}              OPTIONAL,

csi-RS-RLM                                  ENUMERATED {supported}              OPTIONAL,

...,

[[

handoverInterF                              ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC                             ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC                             ENUMERATED {supported}              OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM                ENUMERATED {n2, n4, n6, n8}         OPTIONAL

]],

[[

simultaneousRxDataSSB-DiffNumerology        ENUMERATED {supported}              OPTIONAL

]],

[[

nr-AutonomousGaps-r16                       ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-ENDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NEDC-r16                  ENUMERATED {supported}              OPTIONAL,

nr-AutonomousGaps-NRDC-r16                  ENUMERATED {supported}              OPTIONAL,

dummy                                       ENUMERATED {supported}              OPTIONAL,

cli-RSSI-Meas-r16                           ENUMERATED {supported}              OPTIONAL,

cli-SRS-RSRP-Meas-r16                       ENUMERATED {supported}              OPTIONAL,

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

simultaneousRxDataSSB-DiffNumerology-Inter-r16  ENUMERATED {supported}          OPTIONAL,

idleInactiveNR-MeasReport-r16               ENUMERATED {supported}              OPTIONAL,

-- R4 6-2: Support of beam level Early Measurement Reporting

idleInactiveNR-MeasBeamReport-r16           ENUMERATED {supported}              OPTIONAL

]],

[[

increasedNumberofCSIRSPerMO-r16             ENUMERATED {supported}              OPTIONAL

]]

}

MeasAndMobParametersFR2-2-r17 ::=           SEQUENCE {

handoverInterF-r17                          ENUMERATED {supported}              OPTIONAL,

handoverLTE-EPC-r17                         ENUMERATED {supported}              OPTIONAL,

handoverLTE-5GC-r17                         ENUMERATED {supported}              OPTIONAL,

idleInactiveNR-MeasReport-r17               ENUMERATED {supported}              OPTIONAL,

...

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

#### <<skipped>>

#### –	MeasAndMobParametersMRDC

The IE MeasAndMobParametersMRDC is used to convey capability parameters related to RRM measurements and RRC mobility.

MeasAndMobParametersMRDC information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERSMRDC-START

MeasAndMobParametersMRDC ::=            SEQUENCE {

measAndMobParametersMRDC-Common         MeasAndMobParametersMRDC-Common                 OPTIONAL,

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff               OPTIONAL,

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff               OPTIONAL

}

MeasAndMobParametersMRDC-v1560 ::=      SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560      OPTIONAL

}

MeasAndMobParametersMRDC-v1610 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1610      MeasAndMobParametersMRDC-Common-v1610        OPTIONAL,

interNR-MeasEUTRA-IAB-r16                  ENUMERATED {supported}                       OPTIONAL

}

MeasAndMobParametersMRDC-v1700 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1700      MeasAndMobParametersMRDC-Common-v1700        OPTIONAL

}

MeasAndMobParametersMRDC-v1730 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1730   MeasAndMobParametersMRDC-Common-v1730           OPTIONAL

}

MeasAndMobParametersMRDC-v1770 ::=      SEQUENCE {

measAndMobParametersMRDC-Common-v1770   MeasAndMobParametersMRDC-Common-v1770

}

MeasAndMobParametersMRDC-Common ::=     SEQUENCE {

independentGapConfig                    ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1610 ::=   SEQUENCE {

condPSCellChangeParametersCommon-r16        SEQUENCE {

condPSCellChangeFDD-TDD-r16                 ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeFR1-FR2-r16                 ENUMERATED {supported}                  OPTIONAL

}                                                                                       OPTIONAL,

pscellT312-r16                              ENUMERATED {supported}                      OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1700 ::=   SEQUENCE {

condPSCellChangeParameters-r17              SEQUENCE {

inter-SN-condPSCellChangeFDD-TDD-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-NRDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFDD-TDD-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

inter-SN-condPSCellChangeFR1-FR2-ENDC-r17       ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

mn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1FDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR1TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL,

sn-InitiatedCondPSCellChange-FR2TDD-ENDC-r17    ENUMERATED {supported}              OPTIONAL

}                                                                                       OPTIONAL,

condHandoverWithSCG-ENDC-r17                        ENUMERATED {supported}              OPTIONAL,

condHandoverWithSCG-NEDC-r17                        ENUMERATED {supported}              OPTIONAL

}

MeasAndMobParametersMRDC-Common-v1730 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-Common-v1770 ::= SEQUENCE {

independentGapConfig-maxCC-r17          SEQUENCE {

fr1-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr2-Only-r17                            INTEGER (1..32)                             OPTIONAL,

fr1-AndFR2-r17                          INTEGER (1..32)                             OPTIONAL

}

}

MeasAndMobParametersMRDC-XDD-Diff ::=   SEQUENCE {

sftd-MeasPSCell                         ENUMERATED {supported}                          OPTIONAL,

sftd-MeasNR-Cell                        ENUMERATED {supported}                          OPTIONAL

}

MeasAndMobParametersMRDC-XDD-Diff-v1560 ::=    SEQUENCE {

sftd-MeasPSCell-NEDC                           ENUMERATED {supported}                   OPTIONAL

}

MeasAndMobParametersMRDC-FRX-Diff ::=          SEQUENCE {

simultaneousRxDataSSB-DiffNumerology           ENUMERATED {supported}                   OPTIONAL

}

-- TAG-MEASANDMOBPARAMETERSMRDC-STOP

-- ASN1STOP

#### <<skipped>>

#### –	UE-MRDC-Capability

The IE UE-MRDC-Capability is used to convey the UE Radio Access Capability Parameters for MR-DC, see TS 38.306 [26].

UE-MRDC-Capability information element

-- ASN1START

-- TAG-UE-MRDC-CAPABILITY-START

UE-MRDC-Capability ::=              SEQUENCE {

measAndMobParametersMRDC            MeasAndMobParametersMRDC                                                        OPTIONAL,

phy-ParametersMRDC-v1530            Phy-ParametersMRDC                                                              OPTIONAL,

rf-ParametersMRDC                   RF-ParametersMRDC,

generalParametersMRDC               GeneralParametersMRDC-XDD-Diff                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

tdd-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode                                                   OPTIONAL,

fr1-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

fr2-Add-UE-MRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode                                                   OPTIONAL,

featureSetCombinations              SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

pdcp-ParametersMRDC-v1530           PDCP-ParametersMRDC                                                             OPTIONAL,

lateNonCriticalExtension            OCTET STRING (CONTAINING UE-MRDC-Capability-v15g0)                              OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1560                                                        OPTIONAL

}

-- Regular non-critical extensions:

UE-MRDC-Capability-v1560 ::=        SEQUENCE {

receivedFilters                     OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)                         OPTIONAL,

measAndMobParametersMRDC-v1560      MeasAndMobParametersMRDC-v1560                                                  OPTIONAL,

fdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

tdd-Add-UE-MRDC-Capabilities-v1560  UE-MRDC-CapabilityAddXDD-Mode-v1560                                             OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1610                                                        OPTIONAL

}

UE-MRDC-Capability-v1610 ::=        SEQUENCE {

measAndMobParametersMRDC-v1610      MeasAndMobParametersMRDC-v1610                                                  OPTIONAL,

generalParametersMRDC-v1610         GeneralParametersMRDC-v1610                                                     OPTIONAL,

pdcp-ParametersMRDC-v1610           PDCP-ParametersMRDC-v1610                                                       OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1700                                                        OPTIONAL

}

UE-MRDC-Capability-v1700 ::=        SEQUENCE {

measAndMobParametersMRDC-v1700      MeasAndMobParametersMRDC-v1700,

nonCriticalExtension                UE-MRDC-Capability-v1730                                                        OPTIONAL

}

UE-MRDC-Capability-v1730 ::=        SEQUENCE {

measAndMobParametersMRDC-v1730      MeasAndMobParametersMRDC-v1730                                                  OPTIONAL,

nonCriticalExtension                UE-MRDC-Capability-v1770                                                                     OPTIONAL

}

UE-MRDC-Capability-v1770 ::=        SEQUENCE {

measAndMobParametersMRDC-v1770      MeasAndMobParametersMRDC-v1770

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

-- Late non-critical extensions:

UE-MRDC-Capability-v15g0 ::=        SEQUENCE {

rf-ParametersMRDC-v15g0             RF-ParametersMRDC-v15g0                                                         OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                                     OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-XDD-Diff       MeasAndMobParametersMRDC-XDD-Diff                                           OPTIONAL,

generalParametersMRDC-XDD-Diff          GeneralParametersMRDC-XDD-Diff                                              OPTIONAL

}

UE-MRDC-CapabilityAddXDD-Mode-v1560 ::=    SEQUENCE {

measAndMobParametersMRDC-XDD-Diff-v1560    MeasAndMobParametersMRDC-XDD-Diff-v1560                                  OPTIONAL

}

UE-MRDC-CapabilityAddFRX-Mode ::=   SEQUENCE {

measAndMobParametersMRDC-FRX-Diff       MeasAndMobParametersMRDC-FRX-Diff

}

GeneralParametersMRDC-XDD-Diff ::= SEQUENCE {

splitSRB-WithOneUL-Path             ENUMERATED {supported}                                                          OPTIONAL,

splitDRB-withUL-Both-MCG-SCG        ENUMERATED {supported}                                                          OPTIONAL,

srb3                                ENUMERATED {supported}                                                          OPTIONAL,

dummy                               ENUMERATED {supported}                                                          OPTIONAL,

...

}

GeneralParametersMRDC-v1610 ::= SEQUENCE {

f1c-OverEUTRA-r16                   ENUMERATED {supported}                                                          OPTIONAL

}

-- TAG-UE-MRDC-CAPABILITY-STOP

-- ASN1STOP

UE-MRDC-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList and supportedBandCombinationListNEDC-Only in UE-MRDC-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

Change End