3GPP TSG RAN WG2 Meeting #123bis	R2-2311073

Xiamen, China, October 9-13, 2023

Agenda Item:	7.5.4.1 (BSR Enhancements for XR)

Source: 	Samsung

Title: 	Discussion on BSR enhancements for XR

Document for:	Discussion & decision

Introduction

The agreements have been made in the last RAN2 meeting are as follows. In this paper, we propose to discuss on the remaining issues on 1) new BS table, and 2) DSR, based on the previous agreements.

Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS.

Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

PDCP discard timer for PDU sets supports cases where PDUs of a PDU Set arrive at different instances of time.

New BS Table

For the distribution of the codepoints of new BS table, it has been already agreed to consider at least linear/uniform distribution, which is suitable to reduce the quantization error, i.e., the interval of the buffer size covered by a codepoint, across the entire buffer size range of the new BS table. If exponential distribution is used, we still face the same issue, i.e., quantization error increases as codepoint increases, which was expected to be addressed by introducing a new BS table.

Proposal 1: RAN2 is kindly asked to consider that the codepoints of new BSR table follow uniform/linear distribution.

gNB configures the LCGs eligible to use the new BS table based on its implementation. Note that gNB is aware of the QoS characteristics of each QoS flow of each LCH, and capable of managing mapping between LCH and LCG.

Proposal 2: gNB configures the LCG(s) eligible to use new BS table, based on its implementation.

For an LCG that can use both legacy and new BS tables, UE chooses the BS table with less quantization error, if both tables cover the buffer size of the LCG. Note that new BS table may not always provide less quantization error for a buffer size it covers, if we adopt uniform distribution for the codepoints of the new BS table.

Proposal 3: For an LCG that is eligible to use both legacy and new BS tables, UE chooses the BS table with less quantization error, if both tables cover the buffer size of the LCG.

For an LCG that is eligible to use both legacy and new BS tables, if not being informed of, NW is not aware of which BS table it should refer to when interpreting the buffer size field. If we define a new BSR MAC CE format, we can add ‘BS table indicator’ per LCG into the new BSR MAC CE format, which indicates explicitly the BS table that is used per LCG. If we reuse the legacy BSR MAC CE format to support the new BS table, a new LCID or eLCID that indicates the use of the new BS table should be assigned.

Proposal 4: If legacy BSR MAC CE format is reused to support the use of the new BS table, a new LCID or eLCID is allocated, to indicate the use of the new BS table.

Delay Status Report

Regarding the remaining time of a UL data (PDCP/RLC SDU/PDU), we can consider it as the remaining PDCP discard timer value of the corresponding PDCP SDU. While, if the UL data is part of a PDU set, the remaining time should be the remaining PDCP discard timer value of the PDCP SDU corresponding to the first PDU of the PDU Set, if PDU set level discarding is enabled. Note that, when a certain PDCP SDU encounters discardTimer expiry, all the PDCP SDUs belonging to the same PDU Set are supposed to be discarded, based on the PDU set level discarding mechanism.

Proposal 5: For the UL data (PDCP/RLC PDU/SDU) not belonging to a PDU set, the remaining time is defined as the remaining PDCP discard timer value of the corresponding PDCP SDU.

Proposal 6: For the UL data (PDCP/RLC PDU/SDU) belonging to a PDU set, the remaining time is defined as the remaining PDCP discard timer value of the PDCP SDU corresponding to the first PDU of the PDU Set, if PDU set level discarding is enabled.

Since the DSR triggering condition as well as the amount of delay-critical UL data (if reported), have already conveyed the remaining time information implicitly, there is not much additional value that explicit reporting of the remaining time can bring. That is, being informed of only the information on the amount of delay-critical UL data, a gNB is capable of making a decent decision on how to allocate the available UL resource. Note that the prioritization among LCGs within a single UE, as well as across different UEs, can be conducted by taking the respective remaining time threshold (configured by gNB) into account.

Proposal 7: Regarding what information DSR should contain, RAN2 is kindly asked to only consider delay-critical UL buffer size per LCG, and not to consider explicitly reporting remaining time

If we agree to report the amount of delay-critical UL data, i.e., the data with remaining time less than remaining time threshold, via DSR, the reporting granularity can be per LCG, since the remaining time threshold is agreed to be configured per LCG.

On the other hand, configuring multiple remaining time thresholds per LCG can introduce too much UE complexity in DSR triggering as well as reporting. If there is no critical issue, one remaining time threshold per LCG is a natural option we should go with.

Proposal 8: RAN2 is kindly asked to consider at most one remaining time threshold per LCG in DSR configuration.

Also, it is better to use eLCID that has been reserved, for DSR, to save LCID pool for future usage.

Proposal 9: RAN2 is kindly asked to allocate one reserved eLCID to DSR MAC CE.

It may be useful to allow padding DSR as the status of the delay-critical UL data may dynamically change and there is a benefit in updating gNB without cost when padding DSR is available. The approach can be similar to padding BSR:

- UL resources are allocated and number of padding bits is equal to or larger than the size of the DSR MAC CE plus its subheader, in which case the DSR is referred to as 'Padding DSR';

- Based on the number of padding bits available, if all the LCGs which have delay-critical UL data could not be accommodated, a truncated DSR MAC CE is reported.

Proposal 10: RAN2 to consider introducing a padding DSR and allowing truncated version of the DSR MAC CE to facilitate dynamic updating to the gNB about the delay-critical UL data.

It is clear that discardTimer is managed per PDCP SDU, and hence, it is more likely to be accessible at PDCP layer. While, there could be more than one solutions on how RLC can be aware of the remaining discardTimer value of a certain UL data in RLC layer. It is not desirable to impose certain restrictions on the interactions between RLC and PDCP internally, in order to embrace various options for different UE implementations.

Proposal 11: How RLC determines the remaining discardTimer value of a UL data in RLC layer is less than a threshold should be up to UE implementation.

The UL data with discardTimer expired is discarded by PDCP/RLC, and hence, should not be considered in DSR triggering/reporting. That is, the UL data with discardTimer expired should be excluded from the case that remaining discardTimer value is less than a threshold. Also, there is no need to capture it as a separate case. Note that legacy PDCP/RLC specifications do not specify it as a separate case in data volume calculation.

Proposal 12: The UL data with discardTimer expired should be excluded from the case of UL data with the remaining discardTimer value less than a threshold, and there is no need to explicitly capture it as a separate case.

We have agreed that DSR is triggered if there is UL data with remaining time less than the remaining time threshold corresponding to the associated LCG, configured by gNB. To prevent too many DSR triggering events to occur, we need to specify further conditions besides the previous agreement, as in the case of BSR triggering.

Proposal 13: RAN2 is kindly asked to consider the following events as DSR triggering events:

- Delay-critical UL data, for a logical channel which belongs to an LCG configured for DSR, is available to the MAC entity; and either

  - none of the logical channels, which belong to an LCG configured for DSR, contains any available delay-critical UL data; or

  - this delay-critical UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available delay-critical UL data which belong to any LCG configured for DSR

In determining the delay-critical data volume reported via DSR, there is an issue when one or more PDUs of a PDU set have not arrived at PDCP layer. Specifically, if one or more PDUs of the PDU set have not arrived yet, we should discuss on whether the entire PDU set size (including the PDU(s) that have not arrived yet) or the size of PDUs that have arrived, is included in the data volume calculation of delay-critical UL data. Note that, based on the previous agreement, when at least one PDU of a PDU set has arrived at PDCP, the PDU set size, which is part of the PDU set information, can be identified via UE implementation. The most obvious pros and cons of including the entire PDU set size could be:

- Pros: It can improve the capacity and delay performance, since a UL grant, sufficient to embrace the whole PDU set, can come right away, instead of multiple UL grants across multiple UL scheduling.

- Cons: if a UL grant (in response to DSR) that can embrace the whole PDU set, comes earlier than the arrival of the subsequent PDUs, the portion of the UL grant corresponding to the PDUs that have not arrived yet may be wasted.

Besides, considering the implication of PDU set based discard mechanism, it might be essential to reflect the entire PDU set size in the amount of delay-critical UL data.

Proposal 14: RAN2 is kindly asked to discuss about whether to include the entire PDU set size in the data volume reported by DSR, when one or more PDUs of the PDU set have not arrived at PDCP layer.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Proposal 1: RAN2 is kindly asked to consider that the codepoints of new BSR table follow uniform/linear distribution.

Proposal 2: gNB configures the LCG(s) eligible to use new BS table, based on its implementation.

Proposal 3: For an LCG that is eligible to use both legacy and new BS tables, UE chooses the BS table with less quantization error, if both tables cover the buffer size of the LCG.

Proposal 4: If legacy BSR MAC CE format is reused to support the use of the new BS table, a new LCID or eLCID is allocated, to indicate the use of the new BS table.

Proposal 5: For the UL data (PDCP/RLC PDU/SDU) not belonging to a PDU set, the remaining time is defined as the remaining PDCP discard timer value of the corresponding PDCP SDU.

Proposal 6: For the UL data (PDCP/RLC PDU/SDU) belonging to a PDU set, the remaining time is defined as the remaining PDCP discard timer value of the PDCP SDU corresponding to the first PDU of the PDU Set, if PDU set level discarding is enabled.

Proposal 7: Regarding what information DSR should contain, RAN2 is kindly asked to only consider delay-critical UL buffer size per LCG, and not to consider explicitly reporting remaining time

Proposal 8: RAN2 is kindly asked to consider at most one remaining time threshold per LCG in DSR configuration.

Proposal 9: RAN2 is kindly asked to allocate one reserved eLCID to DSR MAC CE.

Proposal 10: RAN2 to consider introducing a padding DSR and allowing truncated version of the DSR MAC CE to facilitate dynamic updating to the gNB about the delay-critical UL data.

Proposal 11: How RLC determines the remaining discardTimer value of a UL data in RLC layer is less than a threshold should be up to UE implementation.

Proposal 12: The UL data with discardTimer expired should be excluded from the case of UL data with the remaining discardTimer value less than a threshold, and there is no need to explicitly capture it as a separate case.

Proposal 13: RAN2 is kindly asked to consider the following events as DSR triggering events:

- Delay-critical UL data, for a logical channel which belongs to an LCG configured for DSR, is available to the MAC entity; and either

  - none of the logical channels, which belong to an LCG configured for DSR, contains any available delay-critical UL data; or

  - this delay-critical UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available delay-critical UL data which belong to any LCG configured for DSR

Proposal 14: RAN2 is kindly asked to discuss about whether to include the entire PDU set size in the data volume reported by DSR, when one or more PDUs of the PDU set have not arrived at PDCP layer.