3GPP TSG-RAN WG2 Meeting #123-bis		R2-2311090

Xiamen, China: 9th October – 13th October 2023

Source:	Qualcomm Incorporated

Title:	Discussion on RA report and logged MDT report for RedCap UEs

Agenda Item:	6.1.3.2 UE capabilities

Document for:	Discussion and Decision

# Introduction & Background

In Rel-17, RedCap was introduced in the 3GPP. Compared to the normal UEs, RedCap UEs may have limited memory, computation resources, and other resources. For the RA report for normal UEs, 8 entries are supported. Similarly, for the logged MDT for normal UEs, a minimum of 64KB of memory is allocated. However, these requirements create significant implementation challenges for RedCap UEs. In this paper, we discuss reducing memory requirements for supporting RA reports and logged MDT reports for RedCap UEs.

# Discussion

- Reducing memory requirements for supporting RA report and logged MDT report for RedCap UEs

As discussed previously, the memory requirement for logged MDT reports and the number of entries for RA report creates significant implementation challenges for the RedCap UEs. While RA reports is important for RACH resource optimizations and logged MDT report is important for coverage and capacity optimization, implementing RA reports and logged MDT report for RedCap UEs may not be feasible due to required memory. In our understanding, considering the benefits of RA and logged MDT reports in network optimizations, and memory constraints at the UE, it will be good if the memory required for logged MDT and the number of entries for RA report for RedCap UEs can be relaxed. For example, the memory required for logged MDT for RedCap UEs can be reduced to 16 KBs or 32 KBs and the number of entries for the RA report for RedCap UEs can be reduced to 2 or 4.

Observation 1: Memory requirement for logged MDT report and the number of entries for RA report induces significant implementation challenges for the RedCap UEs.

Proposal 1: RAN2 is requested to consider reducing memory requirements for RA-report and logged MDT report for RedCap UEs, i.e.,

- Reduce the number of entries in the report to a lower number, e.g., 2 or 4 entries.

- Reduce the minimum logged MDT memory required, e.g., reduce the logged MDT memory from 64 KBs to 16 KBs or 32 KBs.

# Conclusion

Observation 1: Memory requirement for logged MDT report and the number of entries for RA report induces significant implementation challenges for the RedCap UEs.

Proposal 1: RAN2 is requested to consider reducing memory requirements for RA-report and logged MDT report for RedCap UEs, i.e.,

- Reduce the number of entries in the report to a lower number, e.g., 2 or 4 entries.

- Reduce the minimum logged MDT memory required, e.g., reduce the logged MDT memory from 64 KBs to 16 KBs or 32 KBs.

# Reference

[1] RAN2#123-bis meeting report