3GPP TSG-RAN WG2 Meeting #123bis              			       R2-2311102

Xiamen, China, 9 - 13 Oct 2023                                              Revision of R2-2308797

Agenda item:	7.8.2

Source:	LG Electronics Inc.

Title:	Measurement report enhancement

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss stage-3 details regarding remaining issue list as follows:

- Altitude and height related aspects: - <<removed>> - Consistent description of events including Hx: “…height is…” or “…height becomes…” - Further clarifications regarding the agreement on: “…it is up to UE implementation whether the UE keeps the old measurement report or cellstriggeredlist…”: - Whether this should be rather a specified and predictable behavior - On height-dependent ssb-ToMeasure and measurement configuration: - Should ssb-toMeasure be required to have the same height ranges as the height-dependent events AxHy (including hysteresis)? If not, what should the behavior of the cellsTriggeredList be when a UE switches to a new height range in either SSB to Measure or in eventAxHy? - Should the numberOfTriggeringCells mechanism be allowed to consider all SSBs despite the SSB to Measure configuration for the purpose of interference reporting (which limits the use to some specific SSBs only) - <<removed>>

# 2. Discussion

## 2.1 Height dependent MR

UE behavior with height event (H1/H2 event)

According to running CR, whether the height condition is satisfied is determined by the current height of the Aerial UE. For example, for an H1 event, it is fulfilled if the Aerial UE height is above the threshold. For H2 events, it is fulfilled if the Aerial UE height is below the threshold.

However, in the field description, it is described to “Aerial UE height becomes higher than absolute threshold“ and “Aerial UE height becomes lower than absolute threshold“ for H1 and H2 event, repectively.  Depending on the interpretation, the description could mistakenly imply sending a one-shot report when the UE's height satisfies the threshold criteria with movement direction. In such case, it is possible that the required MR may not be triggered. To avoid misunderstandings, RAN2 should clarify UE behavior regarding height event.

Proposal 1. To clarify the UE behavior related to height event (H1/H2):

- UE triggers MR for H1 event if its height is above the absolute threshold. This approach also applies during the initial configuration of the H1 event; if UE's height is above the H1 threshold at setup, MR for H1 can be triggered

- UE triggers MR for H2 event if its heithg is below the absolute threshold. This approach also applies during the initial configuration of the H2 event; if UE’s height is below the H2 threshold a setup, MR for H2 can be triggerd

UE behavior with new event (AxHy event)

Similarly, above approach can be applied to the new event (AxHy). New events are introduced to implement both height-dependent MR configurations and event combinations. Considering both objectives, different UE behaviors may lead to unexpected consequences from a network perspective, e.g., unexpected MR. To avoid misunderstandings, RAN2 can clarify the UE behavior related to new event similar with P1.

Proposal 2. To clarify the UE behavior related to new event (AxHy):

- For AxH1 event, UE evaluates Ax event if its height is above absolute threshold. This approach also applies during the initial configuration of the AxH1 event; if UE’s height is above the H1 threshold at setup, UE can evaluates Ax event

- For AxH2 event, UE evalueates Ax event if its height is below absolute threshold. This approach also applies during the initial configuration of the AxH2 event; if UE’s height is below the H2 threshold at setup, UE can evaluates Ax event

Height threshold in new event (AxHy)

In the latest meeting, RAN2 has agreed to include only one height threshold in new event. However, applying only one height threshold can result in unintended UE behavior as shown below.

- Above H1 area: Ax threshold associated with H1 can be applied to entire region above H1

- Below H2 area: Ax threshold associated with H2 can be applied to the entire area below H2

(a)                                                                        (b)

<Figure 1. (a) Ascending case (b) Descending case>

In order to avoid redundant height-dependent MR in overlap area, each Hx event can include an additional leaving condition to specify height region. Alternatively, the UE can consider the other height values in different Hx event to determine the height region. However, implementing these solutions on the UE side may be complex and less ideal.

Note that the network can estimate the approximate height of the UE using H1/H2 MR and/or location information. The network can (de)configure the AxHy event based on the UE's current height, effectively managing redundant MRs in overlap areas. In other words, it is up to network implementation whether to configure AxHy events over overlap area (no specification changes required). Consequently, the UE follows the network configuration without additional solution for overlap area.

Proposal 3. It is left up to network implementation whether to configure AxHy events over overlap area

## 2.2 Height dependent parameters

Relation between SSB-ToMeasure and AxHy

There is an issue related to height range for the SSB-ToMeasure and AxHy event.

Open issue: Should ssb-toMeasure be required to have the same height ranges as the height-dependent events AxHy (including hysteresis)? If not, what should the behavior of the cellsTriggeredList be when a UE switches to a new height range in either SSB to Measure or in eventAxHy?

In the case of SSBs, the maximum number within an SS burst set is determined by the frequency range. For example, it's 4 for frequency ranges up to 3GHz, 8 for 3-6 GHz, and 64 for above 6GHz. The network can transmit each SSB in different directions, and the UE can detect optimal beams among them. Depending on the frequency range and directional characteristics, different height regions can be distinguished for SSBs.

On the other hand, when it comes to event conditions related to radio quality, threshold values are set for purposes such as mobility or DCCA connection, and these values are not direction-specific. Since there isn't a direct correlation between the SSB's height range and the Hy threshold, aligning the two height ranges is highly restrictive.

Proposal 4. Do not consider height range dependencies between height dependent SSBs and AxHy events

For the subsequent question in this issue (If not, what should be the behavior of the cellsTriggeredList when a UE switches to a new height range in either SSB measurement or eventAxHy?), RAN2 can consider it in two separate parts.

First, regarding the SSB-ToMeasure, we have already agreed that UE behavior related to L1 and L3 measurements.

RAN2 #122

For UE behavior on L1 and L3 measurement, it is left to UE implementation whether to keep/discard the old samples while UE moves to a new height region with a different SSB-ToMeasure value

Note that the UE generates the 'cellsTriggeredList' based on L3 measurement results. Since the decision to keep or discard L3 measurement results in a new height region is determined by UE implementation, the handling of 'cellsTriggeredList' can also be left up to UE implementation in the new height region. If the UE keeps the L3 measurement results, the 'cellsTriggeredList' can also be keeps, while if it discards them, the list can be discarded as well.

Proposal 5. It is left up to UE implementation whether to keep/discard the cellsTriggeredList when UE moves to a new height region with a different SSB-ToMeasure value

Second, regarding the event AxHy, we have reached an agreement related to 'numberOfTriggeringCells,' while the following remains an open issue.

RAN2 #123

When the UE moves to a new height region with a different numberOfTriggeringCells value for one configured event, it is up to UE implementation whether the UE keeps the old measurement report or cellsTriggeredList

Open issue:

Further clarifications regarding the agreement on: “…it is up to UE implementation whether the UE keeps the old measurement report or cellstriggeredlist…”:

Whether this should be rather a specified and predictable behavior

Before discussing this issue, it is necessary to discuss how to implement height-dependent 'numberOfTriggeringCells.' Instead of introducing a new height range for 'numberOfTriggeringCells,' it can be achieved simply by including it into an AxHy event.

Proposal 6. To confirm, height-dependent 'numberOfTriggeringCells' is implemented by including it in the AxHy event configuration, avoiding the need to introduce additional height ranges for 'numberOfTriggeringCells' within a single configured event

With Proposal 6, if the UE moves to a different height region for the AxHy event, it implies that the UE does not meet the conditions for the corresponding AxHy event. As per the agreement mentioned above, whether the UE keeps or discards the old measurement report or cellsTriggeredList in the different height range is determined by the UE's implementation. Consequently, the handling of the cellsTriggeredList can also be decided by the UE. There is no need to specify the UE's operation in RRC, but the expected behavior of the UE is as follows:

- If the UE keeps the list and returns to the previous height range, it can add or remove cells from the stored cellsTriggeredList

- If the UE discards the list and returns to the previous height range, it can add or remove cells from the initial cellsTriggeredList

In both cases, if the number of cells in the cellsTriggeredList satisfies the conditions related to numberOfTriggeringCells, the UE triggers measurement reports.

Proposal 7. It is left up to the UE implementation to decide whether to keep/discard the cellsTriggeredList for an AxHy event when the UE moves out of the height range defined for the AxHy event

SSB Measurement with numberOfTriggeringCells

There is an issue related to SSB measurement when numberOfTriggeringCells is configured int the report configuration associated with the corresponding measurement object.

Open issue : Should the numberOfTriggeringCells mechanism be allowed to consider all SSBs despite the SSB to Measure configuration for the purpose of interference reporting (which limits the use to some specific SSBs only)?

Conventionally, if the SSB-ToMeasure is not configured, the UE measures on all SS blocks. Since whether all SS blocks should be measured or not is configurable, it is not necessary to specify additional UE operations. In other words, regardless of whether the measurement object is associated with the report configuration with numberOfTriggeringCells, the UE can measure height-dependent SSBs and derive measurement results.

//38331 ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not to be measured while value 1 indicates that the corresponding SS/PBCH block is to be measured (see TS 38.215 [9]). When the field is not configured the UE measures on all SS blocks. Regardless of the value of this field, SS/PBCH blocks outside of the applicable smtc are not to be measured. See TS 38.215 [9] clause 5.1.1.

Proposal 8.  Regardless of whether the measurement object is associated with report configuration including numberOfTriggeringCells, UE measures height dependent SSBs for the current height (If any SSB-ToMeasure is not configured, UE measures all SS blocks as a legacy operation)

# 3. Conclusion

Proposal 1. To clarify the UE behavior related to height event (H1/H2):

- UE triggers MR for H1 event if its height is above the absolute threshold. This approach also applies during the initial configuration of the H1 event; if UE's height is above the H1 threshold at setup, MR for H1 can be triggered

- UE triggers MR for H2 event if its heithg is below the absolute threshold. This approach also applies during the initial configuration of the H2 event; if UE’s height is below the H2 threshold a setup, MR for H2 can be triggerd

Proposal 2. To clarify the UE behavior related to new event (AxHy):

- For AxH1 event, UE evaluates Ax event if its height is above absolute threshold. This approach also applies during the initial configuration of the AxH1 event; if UE’s height is above the H1 threshold at setup, UE can evaluates Ax event

- For AxH2 event, UE evalueates Ax event if its height is below absolute threshold. This approach also applies during the initial configuration of the AxH2 event; if UE’s height is below the H2 threshold at setup, UE can evaluates Ax event

Proposal 3. It is left up to network implementation whether to configure AxHy events over overlap area

Proposal 4. Do not consider height range dependencies between height dependent SSBs and AxHy events

Proposal 5. It is left up to UE implementation whether to keep/discard the cellsTriggeredList when UE moves to a new height region with a different SSB-ToMeasure value

Proposal 6. To confirm, height-dependent 'numberOfTriggeringCells' is implemented by including it in the AxHy event configuration, avoiding the need to introduce additional height ranges for 'numberOfTriggeringCells' within a single configured event

Proposal 7. It is left up to the UE implementation to decide whether to keep/discard the cellsTriggeredList for an AxHy event when the UE moves out of the height range defined for the AxHy event

Proposal 8.  Regardless of whether the measurement object is associated with report configuration including numberOfTriggeringCells, UE measures height dependent SSBs for the current height (If any SSB-ToMeasure is not configured, UE measures all SS blocks as a legacy operation)

# 4. References

[1]      38.331 17.5.0

[2]      R2-23xx-UAV-RRC-running-CR-v00-rapp