3GPP TSG-RAN WG2 #123bis	R2-2311103

Xiamen, 9th October – 13th October 2023

Source:                    	DENSO CORPORATION

Title:  	Consideration on flight path reporting for NR UAV

Document for:        	Discussion and decision

Agenda Item:         	7.8.3 	Flight path reporting

- Introduction

In the recent RAN2 meetings, the following have been agreed about the flight path reporting.

- RAN2#121[1]:

- The granularity of flightpath timestamp is 1s. - Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

- RAN2#121bis-e[2]:

- Flightpath update indication in UAI is configurable by the network - Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE - Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] - As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) - As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling

- RAN2#122[3]:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

- RAN2#123[4]:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.No delta signaling for updated flight path will be supported - Confirm that same indication is used for both initial and updated flightpath plan - If any waypoint/timestamp has changed more than the configured threshold the UE triggers flightpath update indication. - When both distance and time threshold are configured, the flightpath update can be triggered if either of those conditions are met. No prohibit timer will be introduced.

In accordance with the above agreements and FFS, this paper attempts to investigate about the remining consideration on the flight path reporting.

- Discussion

  - Triggering conditions for updating flight path

As mentioned above, RAN2 already have some discussions about contents of triggering condition, however, we think RAN2 has not yet discussed signaling details how to configure the triggering condition to UE.

During last meeting, some papers mentioned that UEInformationRequest could be used to configure the triggering condition. We agree UEInformationRequest could be used, however, network does not know the scale/granularity of the flightpath in the case, since UEInformationRequest is sent before UEInformationResponse. Therefore, other message might be better if network wants to consider the scale/granularity of the flightpath to configure the triggering condition to UE.

Additionally, there is a distance-based triggering condition named D1 in the current measurement report specification. We think some parts of the mechanism for D1 event could be applicable for the triggering condition of the flight path update. Therefore, we would like to propose to use mechanism of ReportConfig of measurement report for the triggering condition configuration.

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

<skip unrelated part>

...,

[[

eventD1-r17                                 SEQUENCE {

distanceThreshFromReference1-r17            INTEGER(1.. 65525),

distanceThreshFromReference2-r17            INTEGER(1.. 65525),

referenceLocation1-r17                      ReferenceLocation-r17,

referenceLocation2-r17                      ReferenceLocation-r17,

reportOnLeave-r17                           BOOLEAN,

hysteresisLocation-r17                      HysteresisLocation-r17,

timeToTrigger-r17                           TimeToTrigger

}

]]

},

<skip unrelated part>

}

Figure 1. Example of distance-based triggering condition (event D1)

Proposal 1: The mechanism of ReportConfig of measurement report could be used for the triggering condition configuration by the network

In RAN2#122 meeting, it was agreed that UE should update the reported flightpath when network-configured triggering condition is satisfied. With this agreement, effectiveness of the flightpath could be possibly kept up to requirement of the network. However, periodicity of evaluation for the triggering condition was not discussed. To keep the effectiveness of the flightpath information, the periodicity of evaluation would be one of important factor. For example, the flight path information would be outdated if the periodicity of evaluation is very long.

Proposal 2: RAN2 to consider the periodicity of evaluation if the triggering condition is satisfied

Aside from network-configured triggering condition, there might be some cases where UE needs to update the reported flightpath like below:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

Therefore, we would like to propose to discuss how UE should behave when such exception cases occur.

Proposal 3: RAN2 to discuss about exception cases where UE needs to update flightpath information even though network-configured triggering condition is not satisfied, such as:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

- Summary and proposal

This paper discussed the consideration on flight path reporting of NR support for UAV. In summary, the followings were observed and proposed:

Proposal 1: The mechanism of ReportConfig for measurement report could be used for the triggering condition configuration by the network

Proposal 2: RAN2 to consider the periodicity of evaluation if the triggering condition is satisfied

Proposal 3: RAN2 to discuss about exception cases where UE needs to update flightpath information even though network-configured triggering condition is not satisfied, such as:

- UE changes the destination

- Subsequent flightpath needs to be reported in case only a part of flightpath was reported before

- UE cancels the flight (reported flightpath should be discarded)

- References

[1] R2-2301903, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[2] R2-2304203, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[3] R2-2306543, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”

[1] R2-2308963, “Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”