3GPP TSG-RAN WG2 #123bis	R2-2311104

Xiamen, China, 9th – 13th Oct 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on delay status reporting for XR

Document for:        	Discussion and decision

Agenda Item:         	7.5.4.1		BSR enhancements for XR

- Introduction

In RAN#99, updated WID on XR enhancements was approved [1], and the following objectives were specified regarding XR-specific capacity improvements.

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

And the following agreements related to delay status reporting (DSR) were reached in RAN2#123 meeting.

Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG. When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time. RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE. Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS. Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS.

In this contribution, we provide the discussion on delay status reporting for XR.

- Discussion

DSR would be beneficial for XR capacity as it allows a UE to report the amount of urgent data with the short remaining time. However, if there is no urgent data in the UE's buffer, DSR transmissions may increase unnecessary overhead. Therefore, DSR needs to have separate trigger conditions from BSR and should not be triggered by the legacy BSR trigger conditions. From this aspect, it would be better to define a new separate MAC CE for DSR reporting.

For the DSR trigger conditions, it was agreed to support threshold based DSR reporting, but it has not been determined whether multiple thresholds can be configured. We basically consider a single threshold to be sufficient, as multiple thresholds can cause frequent DSR transmissions if applied simultaneously for DSR triggering. However, if timer-based solution for PSI-based discarding is supported, it may be useful to have multiple thresholds. In other words, if another discard timer value is applied in case of congestion, another threshold value may be needed accordingly. In this case, multiple thresholds can be configured per LCG, and one of them can be used for DSR triggering based on the indication to apply PSI-based discard mechanism. In addition, the cancellation condition for triggered DSR should be discussed. There could be a case where DSR is triggered but no available UL resources are allocated. If the corresponding data is discarded due to the expiration of timer before DSR is transmitted, the triggered DSR is no longer valid and should be cancelled so that the gNB doesn’t misunderstand the amount of urgent data.

Proposal 1:	The following working assumption can be confirmed.

- Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS

Proposal 2:	If timer-based solution for PSI-based discarding is supported, multiple thresholds for DSR triggering can be considered. Otherwise, a single threshold would be sufficient.

Proposal 3:	Triggered DSR can be cancelled when the corresponding urgent data is discarded before the DSR transmission.

One option for DSR format is to introduce a new buffer size field which allows each LCG to report the amount of buffered data where the remaining time is lower than a predefined or configured threshold. In this case, since there is no need to introduce an additional field to explicitly indicate the remaining time, the DSR format can be defined in the same way as the legacy BSR format, as shown in Figure 1. That is, LCGi field can indicate the presence of the Urgent Buffer Size field for the logical channel group i. The Urgent Buffer Size field can identify the total amount of data with short remaining time and 8-bit BSR table can be reused for this field. Compared to other options that introduce separate fields indicating the remaining time and the associated buffer size, this can be achieved with less specification effort. Furthermore, the urgent buffered data with different remaining times can be accommodated in one urgent BS field, resulting in less signalling overhead.

Figure 1: Example of DSR format

Proposal 4:	DSR can include a field which allows each LCG to report the amount of buffered data where the remaining time is lower than a predefined or configured threshold.

In RAN2#122 meeting, it was agreed that the reference time for the remaining time is determined based on the timing of the first transmission of DSR. For the FFS of this agreement, in our view, intra-UE prioritization can impact the determination of the remaining time to be reported. In the current specification, if de-prioritized configured grant is configured with autonomousTx, the UE autonomously transmits the de-prioritized PDU as a new transmission using the subsequent configured grant with the same HARQ process. If DSR MAC CE is included in this de-prioritized PDU, the remaining time reported by the UE may differ from the actual remaining time. As shown in Figure 2, if DSR transmission is de-prioritized at T1 and autonomously transmitted at T1', the remaining time reported by the UE would be determined based on T1. On the other hand, since the gNB considers the transmission at T1' to be the first transmission, it would recognize the remain time based on T1’, which may lead to the misunderstanding of the actual remaining time.

Figure 2: Example of the remaining time for intra-UE prioritization

Observation 1:	Intra-UE prioritization may mislead the gNB about the actual remaining time of buffered data.

Proposal 5:	RAN2 to consider the impact of intra-UE prioritization on the determination of the remaining time.

- Summary and proposal

In this contribution, we discussed the potential DSR enhancements for XR and provided the following observation and proposals.

Proposal 1:	The following working assumption can be confirmed.

- Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS

Proposal 2:	If timer-based solution for PSI-based discarding is supported, multiple thresholds for DSR triggering can be considered. Otherwise, a single threshold would be sufficient.

Proposal 3:	Triggered DSR can be cancelled when the corresponding urgent data is discarded before the DSR transmission.

Proposal 4:	DSR can include a field which allows each LCG to report the amount of buffered data where the remaining time is lower than a predefined or configured threshold.

Observation 1:	Intra-UE prioritization may mislead the gNB about the actual remaining time of buffered data.

Proposal 5:	RAN2 to consider the impact of intra-UE prioritization on the determination of the remaining time.

- References

[1]	RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm