3GPP TSG-RAN WG2 #123bis	R2-2311125

Xiamen, China, 9th – 13th October 2023

Source:	NTT DOCOMO, INC.

Title:	Current status of functional issues on 2TA enhancement

Document for:	Discussion and decision

Agenda Item:	7.20.2

- Introduction

As can be seen in [Post123][MIMOevo] Remaining open issues (NTT Docomo), followings are remaining open issues on Two TAs for multi-DCI.

Two TAs for multi-DCI [Functional issue] - TA modelling: whether to confirm assumption on 2-PTAG model - Behaviors when one PTAG for SpCell expires - Behaviors when one TAG for SCell expires - Indication of TAG IDs in RACH procedure (if additional RAN1 agreement has RAN2 impact) [MAC spec issue] - Indication of TAG IDs in Absolute TAC MAC CE [RRC spec issue] - Whether and how the original definition of tag-Id is affected by introduction of second TAG ID - Details on additionalCFRA-ToAddModList-r18 (i.e., additional PRACH configuration used for CFRA in inter-cell case) which RAN1 left up to RAN2

This contribution provides our understanding on detailed status of functional issues so far (plus one new issue: Trigger of random access problem).

- Discussion

  - Remaining open issues

    - Behaviors when one TAT for STAG expires

In the last meeting, RAN2 reached a following working assumption. Note that this assumption is referring to following candidate actions.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

8.	consider all running timeAlignmentTimers as expired.

RAN2#123 [1] Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

In other words, RAN2 already assumed that (action 1) any UL transmission except Msg1 and MsgA is not performed and (action 7) NTA is maintained, when one TAT for STAG while the other TAT for SCell is running. On the other hand, 2-6 are FFS. In this section, we would like to provide our understanding and our preference on whether to skip for each action.

For action 2 (flush all HARQ buffers), we do not think HARQ buffers for corresponding TRP have to be flushed. This is because UE can retransmit HARQ via the other TRP which is associated with running TAT even after one TAT is expired.

For action 3 (notify RRC to release PUCCH, if configured), according to TS 38.331 [2], PUCCH-Config is configured per BWP. Basically, in 2-TA operation, the same BWP is used even if the serving cell has multiple TRPs in both intra-cell case and inter-cell case. It will not help if PUCCH-Config is released even if the access to the other TRP is valid, thus we prefer not to conduct action 3. (Indeed PUCCH-Config includes a list of PUCCH resources, which enables to release part of PUCCH resources, but we could not find association between PUCCH resources and TRP# as far as we understand. We doubt therefore if UE can release part of PUCCH resources only associated with the TRP with expired TAT. This is why we do not propose to release only PUCCH resource for TRP with expired TAT.)

For action 4 (notify RRC to release SRS, if configured), action 5 (clear any configured downlink assignments and configured uplink grants) and action 6 (clear any PUSCH resource for semi-persistent CSI reporting), we do not prefer to conduct as well for the same reason as action 3 (SRS-Config, ConfiguredGrantConfig and SPS-Config are configured per BWP).

Proposal 1.	When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), following behaviors are not applied.

-	flush all HARQ buffers;

-	notify RRC to release PUCCH, if configured;

-	notify RRC to release SRS, if configured;

-	clear any configured downlink assignments and configured uplink grants;

-	clear any PUSCH resource for semi-persistent CSI reporting;

    - Behaviors when one TAT for PTAG expires

In the last meeting, RAN2 reached a following working assumption. Note that this assumption is referring to following candidate actions.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

8.	consider all running timeAlignmentTimers as expired.

RAN2#123 [1] Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

In our understanding, expected behaviors upon expiry of one TAT while the other TAT is still running are the same regardless of whether the expired TAT is in PTAG or STAG (except the case where expired TAT is in PTAG and the other PTAG is not running. In this case the same procedure as legacy PTAG expiry, i.e., 1-8, is applied).

Proposal 2.	When the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell) and the other PTAG is running, the same procedure as expiry of one TAT in SCell, i.e., actions in Proposal X, is applied.

    - How to indicate TAG ID in intra-cell PDCCH ordered CFRA

RAN1 made an agreement that in intra-cell case PDCCH ordered CFRA, the network indicates which TAG ID (first TAG ID or second TAG ID) is associated using 1 bit as part of Timing Advance Command in RAR.

RAN1#114 [3] Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured.

However, according to TS 38.321 [4], Timing Advance Command MAC CE has no reserved bit.

6.1.3.4 Timing Advance Command MAC CE The Timing Advance Command MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.4-1): - TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG containing the SpCell has the TAG Identity 0. The length of the field is 2 bits; - Timing Advance Command: This field indicates the index value TA (0, 1, 2… 63) used to control the amount of timing adjustment that MAC entity has to apply (as specified in TS 38.213 [6]). The length of the field is 6 bits. Figure 6.1.3.4-1: Timing Advance Command MAC CE

Therefore, in our understanding, we cannot implement indication with 1 bit as part of TA command. Therefore, we would like to ask RAN1:

Q1.	Whether it is correct that “indicate a representation of the TAG ID with 1 bit as part of TA command in RAR” means to use 1 reserved bit in Timing Advance Command MAC CE.

Q2.	If answer to Q1 is yes, that is not feasible because of shortage of reserved bit in Timing Advance Command MAC CE. Could you find another solution?

Proposal 3.	Send LS to RAN1 to ask following questions.

Q1.	Whether it is correct that “indicate a representation of the TAG ID with 1 bit as part of TA command in RAR” means to use 1 reserved bit in Timing Advance Command MAC CE.

Q2.	If answer to Q1 is yes, that is not feasible because of shortage of reserved bit in Timing Advance Command MAC CE. Could you find another solution?

    - How to indicate TAG ID in intra-cell CBRA

RAN2 made an agreement that in intra-cell case CBRA, as a baseline, we reuse the mechanism for PDCCH ordered CFRA, i.e., indication via RAR.

RAN2#123 [1] The following is taken as baseline (for intra-cell case): for CBRA, we reuse the mechanism agreed for CFRA case, i.e. use the RA RAR to indicate the TAG.

But in our understanding, as described in 2.1.3, the mechanism for intra-cell CFRA case is not feasible. We should first clarify 2.1.3.

    - Trigger of radio link failure

In TS 38.300 [5], it is specified that a random access procedure failure triggers to declare a radio link failure.

9.2.7 Radio Link Failure … The UE declares Radio Link Failure (RLF) when one of the following criteria are met: - Expiry of a radio problem timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); or - Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running; or - Random access procedure failure; or - RLC failure; or - Detection of consistent uplink LBT failures for operation with shared spectrum channel access as described in 5.6.1; or - For IAB-MT, the reception of a BH RLF indication received from its parent node.

This is also captured in TS 38.331 [2] as following. In short, if random access problem is found in MCG MAC, radio link failure is considered as detected for the MCG.

5.3.10.3 Detection of radio link failure The UE shall: 1> if any DAPS bearer is configured and T304 is running: 2> ... 1> else: 2> … 2> upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running and SDT procedure is not ongoing; or 2> … 3> if the indication is from MCG RLC and CA duplication is configured and activated for MCG, and for the corresponding logical channel allowedServingCells only includes SCell(s): 4> ... 3> else: 4> consider radio link failure to be detected for the MCG, i.e. MCG RLF; 4> …

If RLF is triggered, in most cases (in other words, except RLF in the source cell during DAPS handover and CHO), UE initiates RRC re-establishment. Followings are TS 38.300 descriptions.

9.2.7 Radio Link Failure … After RLF is declared, the UE: - stays in RRC_CONNECTED; - in case of DAPS handover, for RLF in the source cell: - … - in case of CHO, for RLF in the source cell: - … - otherwise, for RLF in the serving cell or in case of DAPS handover, for RLF in the target cell before releasing the source cell: - selects a suitable cell and then initiates RRC re-establishment; - enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

By the way, in 2-TA scenario, we assume four types of RA procedure, intra-cell CBRA, inter-cell CBRA, intra-cell PDCCH ordered CFRA, and intra-cell CBRA. In our understanding, in all of four types of RAs UEs establish only one TAT of the serving cell. Intra/inter-cell CBRA are for cases where UEs did not maintain any TAT before RA procedure starts while intra/inter-cell PDCCH ordered CFRA are for cases to establish the other TAT when the UE already maintains one of two TATs.

Then the problem is, should UE trigger radio link failure even when intra/inter-cell PDCCH ordered CFRA is unsuccessful? As mentioned above, radio link failure results in RRC re-establishment even though the other TAT may be still valid (i.e., in case of PDCCH ordered CFRA). We think that behavior is not expected in efficient 2-TA operation.

Proposal 4.	RAN2 discuss whether and how to make changes to avoid initiation of RRC re-establishment when RACH fails while the other TAT is valid.

  - Working assumptions

    - 2-PTAG model

RAN2 made a following working assumption, i.e., we assumed that both of TAGs associated with SpCell are PTAGs.

RAN2#123 [1] Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired.

    - Maximum number of TAGs per cell group

RAN2 asked RAN1 whether to increase the maximum number of TAGs per cell group from current number (up to 4 TAGs), and RAN1 answered that they have not reached consensus on increasing it. The RAN2 made a following working assumption, i.e., there is no need to extend from 4 TAGs.

Reply LS from RAN1 [6] Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Answer: RAN1 has not reached consensus to increase the current number of TAGs per cell group.

RAN2#123 [1] RAN2 assumes the current 4 TAGs per cell group is sufficient to support Rel-18 mDCI mTRP with 2 TAs.

  - Resolved issues

    - Per-TRP UE-initiated RACH procedure

RAN1 agreed that it is up to RAN2 whether to support per-TRP UE-initiated RACH procedure. Then RAN2 concluded not to support it.

RAN1#111 [7] Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure. - Agreement: Send LS to RAN2 regarding the conclusion.

RAN2#121bis-e [8] From RAN2 perspective, per TRP UE-initiated RACH procedure is not supported.

    - TAG grouping rule

RAN2 asked RAN1 whether there is any restriction on association of TAT to TRPs via LS, and then RAN1 answered that they had not agreed such restriction. RAN2 concluded that we do not assume such restrictions unless RAN1 change their thoughts.

Reply LS from RAN1 [6] Question 1 on TAG grouping RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs. Q1a: For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs? Answer: Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2.

RAN2#123 [1] RAN2 do not assume any restriction on grouping serving cells/TRPs to TAGs unless RAN1 indication comes.

    - Behaviors when both of TATs for SpCell expire

RAN2 made an agreement that the same procedure as legacy PTAG expiry is conducted when both TATs associated with SpCell (i.e., TATs for both of PTAGs).

Legacy procedures means all of following actions.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

8.	consider all running timeAlignmentTimers as expired.

RAN2#123 [1] At least when both TATs for a SpCell are expired, 1-8 are applied to all TRPs of all serving cells.

    - Behaviors when both of TATs for SCell expire

RAN2 made an agreement that the same procedure as legacy STAG expiry is conducted when both TATs associated with SCell (and at least one PTAG of the cell group is still running).

Legacy procedures means 1-7 of following actions.

1.	not perform any uplink transmission except the Random Access Preamble and MSGA transmission;

2.	flush all HARQ buffers;

3.	notify RRC to release PUCCH, if configured;

4.	notify RRC to release SRS, if configured;

5.	clear any configured downlink assignments and configured uplink grants;

6.	clear any PUSCH resource for semi-persistent CSI reporting;

7.	maintain NTA (defined in TS 38.211 [8]) of this TAG;

8.	consider all running timeAlignmentTimers as expired.

RAN2#123 [1] At least when both TATs for a SCell are expired (assuming PTAG(s) of the cell group still running), 1-7 are applied to all TRPs associated to the TAG with the expired TAT (including both TRPs of the concerned SCell).

    - How to indicate TAG ID in inter-cell PDCCH ordered CFRA

RAN2 made following agreement based on RAN1 conclusion.

RAN2#123 [1] For inter-cell PDCCH order CFRA to the additionalPCI, - PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement),

Thus, the UE can interpret which TAG (for TRP#1 or TRP#2) to update based on the indication in the PDCCH order. This means no additional indication of TAG ID is needed.

    - How to indicate TAG ID in inter-cell CBRA

There is no agreement so far about TAG ID indication for inter-cell CBRA, but considering that in inter-cell case, SSBs are partitioned belonging to two coresetPoolIndex (for serving cell TRP and non-serving cell TRP). Then UE can interpret which TAG (for TRP#1 or TRP#2) to update from the configuration of SSBs, thus no more solution for indicating TAG ID will not be needed.

- Summary and proposal

Proposal 1.	When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), following behaviors are not applied.

-	flush all HARQ buffers;

-	notify RRC to release PUCCH, if configured;

-	notify RRC to release SRS, if configured;

-	clear any configured downlink assignments and configured uplink grants;

-	clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 2.	When the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell) and the other PTAG is running, the same procedure as expiry of one TAT in SCell, i.e., actions in Proposal X, is applied.

Proposal 3.	Send LS to RAN1 to ask following questions.

Q1.	Whether it is correct that “indicate a representation of the TAG ID with 1 bit as part of TA command in RAR” means to use 1 reserved bit in Timing Advance Command MAC CE.

Q2.	If answer to Q1 is yes, that is not feasible because of shortage of reserved bit in Timing Advance Command MAC CE. Could you find another solution?

Proposal 4.	RAN2 discuss whether and how to make changes to avoid initiation of RRC re-establishment when RACH fails while the other TAT is valid.

- References

[1]	“Report from NR MIMO evolution session,” Session Chair (CATT), RAN2#123.

[2]	TS 38.331, V17.6.0.

[3]	“RAN1 Chair’s Notes,” RAN1#114.

[4]	TS 38.321, V17.6.0.

[5]	TS 38.300, V17.6.0.

[6]	R2-2307018, “Reply on LS 2TA for multi-DCI multi-TRP,” From: RAN1, To: RAN2, RAN2#123.

[7]	“Final Report of 3GPP TSG RAN WG1 #111,” RAN1#111.

[8]	“Report from NR MIMO evolution session,” Session Chair (CATT), RAN2#121bis-e.