3GPP TSG-RAN WG2 #123bis	R2-2311128

Xiamen, China, 9th – 13th October 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on remaining issues on Unified TCI framework extension

Document for:	Discussion and decision

Agenda Item:	7.20.3

- Introduction

As can be seen in [Post123][MIMOevo] Remaining open issues (NTT Docomo), followings are remaining open issues on Unified TCI framework extension.

Unified TCI framework extension [Functional issue] - Whether to introduce separate MAC CEs for joint TCI and separate UL/DL TCI [MAC spec issue] - Structure(s) of new MAC CE(s) for extended unified TCI state activation

This contribution explains our understanding on detailed discussion point and provides a possible solution.

- Discussion

  - Necessary information for activation MAC CE for joint TCI state

In the last meeting, RAN2 agreed information to be indicated by TCI state activation MAC CE for separate DL/UL TCI mode [1].

The following information can be indicated by the MAC CE (for separate DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) - if the unified TCI codepoint is for all, or sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}

Related to the third bullet, Pi in the existing MAC CE for activation/deactivation of joint TCI states (specified as a figure below [2]) indicates whether i-th TCI codepoint includes separate UL and DL TCI states or a single DL/joint TCI state, and D/U indicates whether the TCI state ID in the same octet is for joint/downlink or uplink TCI state. These fields are not sufficient to accommodate the granularity for multi-TRP, i.e., unable to indicate whether each TCI state ID is for first or second TCI state. That was why RAN2 agreed to introduce a new MAC CE.

Figure 6.1.3.47-1: Unified TCI state activation/deactivation MAC CE

On the other hand, we would like to note that indication of D/U is not needed if the cell is in joint TCI mode. Therefore if we introduce separate MAC CEs for joint TCI mode and separate DL/UL TCI mode, required information to be indicated by MAC CE for joint TCI is,

-	if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs

-	if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

-	if the unified TCI codepoint is for both or one of {first joint TCI state, second joint TCI state}

Figure 1 depicts one example of MAC CE for TCI state activation for joint TCI mode, taking above requirements into account, where;

-	Pi: This field indicates whether each TCI codepoint has multiple TCI states or single TCI state. If Pi field is set to 1, it indicates that ith TCI codepoint includes the TCI state for 1st TRP and the TCI state for 2nd TRP. If Pi field is set to 0, it indicates that ith TCI codepoint includes only the TCI state for 1st TRP or the TCI state for 2nd TRP.

-	F/S: This field indicates whether the TCI state ID in the same octet is for 1st TRP or 2nd TRP. If this field is set to 1, the TCI state ID in the same octet is for 1st TRP. If this field is set to 0, the TCI state ID in the same octet is for 2nd TRP.

Figure 1 shows that TCI state activation MAC CE for joint TCI mode can be introduced without increasing number of octets compared with legacy unified TCI state activation MAC CE while TCI state activation MAC CE for separate DL/UL TCI mode is expected to need more octets.

Figure 1. Example of TCI state activation MAC CE for joint TCI mode

Proposal 1.	RAN2 discuss whether separate MAC CEs are introduced for joint TCI State and separate DL/UL TCI States respectively.

Proposal 2.	If RAN2 introduce two MAC CEs, following information is indicated in unified TCI state activation MAC CE for joint TCI mode:

-	if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs

-	if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

-	if the unified TCI codepoint is for both or one of {first joint TCI state, second joint TCI state}

- Summary and proposal

Proposal 1.	RAN2 discuss whether separate MAC CEs are introduced for joint TCI State and separate DL/UL TCI States respectively.

Proposal 2.	If RAN2 introduce two MAC CEs, following information is indicated in unified TCI state activation MAC CE for joint TCI mode:

-	if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs

-	if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

-	if the unified TCI codepoint is for both or one of {first joint TCI state, second joint TCI state}

- References

[1]	R2-2308972, “Report from NR MIMO evolution session,” Session Chair (CATT), RAN2#123.

[2]	TS 38.321, V17.5.0.