3GPP TSG-RAN WG2 #123bis					Tdoc R2-2311136

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.3.5

Source:	Ericsson

Title:	Discussion on CHO for NES

Document for:	Discussion, Decision

# 1	Introduction

A new WI on Network energy savings for NR was approved at RAN#98[2]. The WI aims to specify the following enhancements for connected mode mobility:

- Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

# 2	Discussion

In conditional handover, the UE applies the stored handover command message and the associated RRC configuration when the CHO trigger condition is satisfied for one of the configured candidate target cells. The network may also configure two CHO trigger conditions for the UE and associate both to the stored handover command, i.e., the handover command is applied only if both CHO trigger conditions are fulfilled, e.g., conditions configured for different types of measurement quantities, like cell coverage represented by RSRQ/RSRQ or time or location.

The time-based event is defined by T1 and a duration. The event itself becomes fulfilled when time measured at UE becomes more than configured threshold T1 but is less than T2 where T2 is T1+duration. As time-based event is configured together with signal quality/strength event, the CHO can be performed to a candidate target cell when the signal strength related event is fulfilled during the time between T1 and T2.

A location-based trigger condition is based on a distance threshold from the UE to the source cell and to a candidate target cell, i.e., distance between the device and the serving cell reference location, referenceLocation1, becomes larger than absolute threshold1 and the distance between device, and the candidate target cell reference location, referenceLocation2, becomes shorter than absolute threshold2.

The time and location-based trigger conditions are only supported in combination with a measurement-based trigger condition. It is not possible to configure time and location-based trigger conditions simultaneously.

It is possible that a failure is detected while the UE is monitoring the configured conditions. When a failure is detected, e.g., a radio link failure or handover failure, the UE can prioritize a cell for which it has a stored handover command and, instead of performing re-establishment, it performs a conditional handover, which reduces the interruption time and the signalling over the air interface.

In this WI, enhancements for CHO are proposed to target the NES scenarios. For example, a UE may be prepared with conditions related to a source/target gNB NES state where the term NES can be anything from a gNB completely turned off to a gNB operating a more specific NES mode discussed in cell DTX/DRX AI.

In RAN2#121bis meeting the following agreements were reached for CHO:

Agreements:

- Study whether CHO enhancements are needed for the purpose of turning off the cell

- Continue discussing CHO in the context of different NES techniques.

In RAN2#122 meeting the following agreements were reached for CHO:

Agreements

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

In RAN2#12b meeting the following agreements were reached for CHO:

Agreements

1	We will support the CHO triggers for the use case of turning off the cell

2	(At least for cell DTX/DRX) Time-based CHO is not to be considered in NES.

3	Do not consider using an indication in SIB1 for triggering NES CHO execution condition

Regardless of the NES specific enhancements, network needs to support good mobility operation which means timely HO(or CHO) to cell that fulfills optimized RSRP thresholds for normal mobility. These thresholds are likely higher RSRP values than the thresholds to be applied if the source cell is going to turn off or to enter in NES mode which requires at least part of the UEs to be handed off to other cells. If the source cell enters NES mode, there may be need to HO UEs to neighbor cell assuming a lower RSRP thresholds than otherwise optimum. However, there should always be target cell which is good enough for the UE to move into in order to not to cause a coverage hole by network node entering to NES mode.

If the source cell enters NES mode, which could be either cell about to turn off or cell configuring cell DTX/DRX, there may be need to HO UEs to neighbor cell assuming a lower RSRP thresholds than otherwise optimum.

To provide good mobility performance in normal mode and NES optimized mobility, the CHO configuration should contain two threshold levels where the lower NES specific threshold is applied if source cell enters to NES mode. The same trigger or configuration which is used to configure the cell DTX/DRX may be used to indicate to the UE when the NES specific threshold should be applied.

- The CHO configuration should contain two threshold levels where the lower NES specific threshold is applied if source cell enters to NES mode(cell about to turn off or cell configuring cell DTX/DRX).

- The same trigger or configuration which is used to configure the cell DTX/DRX may be used to indicate to the UE when the NES specific threshold should be applied.

The network should also be prepared for the case the UE does not see good enough target cells, even with NES specific RSRP triggers. For this the UE should inform the network if it does not see good enough candidate target cells.

Network should know whether there are UEs that definitely need to be served by the cell without causing data interruption of the HO, or possible HO failure if the RSRP is not sufficient in the target cell.

- Network needs to know if there are no good enough candidate target cells for CHO at the time the cell is about to turn off or the cell is enabling cell  DTX/DRX.

As the key thing is to prevent network creating coverage holes, other solutions can be considered as well. Main issue is that the network should be aware if coverage hole is created by cell entering to NES mode. Also, the level of performance degradation would be beneficial to be known at the network side.

Main issue is that the network should be aware if coverage hole is created and the level of performance degradation created by cell entering to NES mode.

RAN2 to discuss if UE feedback would be beneficial for the cell to determine when to turn off the cell or to configure cell DTX/DRX.

It is a tradeoff between performance and power savings for the cell whether to enter in NES mode or not. This is further different at different network loads and depends on whether the cell is serving legacy UEs or NES capable UEs.

It is possible to enhance CHO mechanism to provide better means for the network for NES based load balancing.

Enhance CHO procedure to enable prioritization of candidate target cells by the UE based on NES mode.

If the UE is performing HO or CHO and experiences HO failure due to source or target cell being or entering in NES mode, UE should inform the network about the HO failure cause. Also, when the HO is successful, UE should inform the network about NES conditions of the source/target cell.

If the UE is performing HO or CHO, UE should inform the network about the HO trigger cause, i.e. if the HO was triggered because the serving cell wanted to enter NES mode, either in HO failure or HO success reports.

# Conclusion

In the previous sections we made the following observations:

Observation 1	If the source cell enters NES mode, which could be either cell about to turn off or cell configuring cell DTX/DRX, there may be need to HO UEs to neighbor cell assuming a lower RSRP thresholds than otherwise optimum.

Observation 2	Network should know whether there are UEs that definitely need to be served by the cell without causing data interruption of the HO, or possible HO failure if the RSRP is not sufficient in the target cell.

Observation 3	Main issue is that the network should be aware if coverage hole is created and the level of performance degradation created by cell entering to NES mode.

Observation 4	It is possible to enhance CHO mechanism to provide better means for the network for NES based load balancing.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The CHO configuration should contain two threshold levels where the lower NES specific threshold is applied if source cell enters to NES mode(cell about to turn off or cell configuring cell DTX/DRX).

Proposal 2	The same trigger or configuration which is used to configure the cell DTX/DRX may be used to indicate to the UE when the NES specific threshold should be applied.

Proposal 3	Network needs to know if there are no good enough candidate target cells for CHO at the time the cell is about to turn off or the cell is enabling cell  DTX/DRX.

Proposal 4	RAN2 to discuss if UE feedback would be beneficial for the cell to determine when to turn off the cell or to configure cell DTX/DRX.

Proposal 5	Enhance CHO procedure to enable prioritization of candidate target cells by the UE based on NES mode.

Proposal 6	If the UE is performing HO or CHO, UE should inform the network about the HO trigger cause, i.e. if the HO was triggered because the serving cell wanted to enter NES mode, either in HO failure or HO success reports.

# References

TR 38.864, Study on network energy savings for NR

RP-223540, New WID: Network energy savings for NR