3GPP TSG-RAN WG2 Meeting #123bis	R2-2311141

Xiamen, China, October 9th – 13th, 2023

Agenda item:		7.5.4.2

Source:		Futurewei

Title:		Remaining issues in PSI-based SDU discarding

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #121, RAN2 agreed on the following with respect to SDU discarding for XR [1]:

5. 	Introduce UL PDU Set Importance. How UE derives this will be handled in UE implementation.

RAN2 thinks PSI can be useful for PDU set-based discard. RAN2 aims to introduce a mechanism to allow UE to handle discarding of packets with different PSI in case of congestion. FFS for other cases.

In RAN2 meeting #122, RAN2 further agreed on the following [2]:

2: PDU-set discard indication for UL is configured using RRC to handle the PDU Set based discard functionality (i.e. whether UE discards all packets in PDU set when one PDU is discarded). The configuration is per PDCP entity.

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling.

FFS how/whether to minimize additional UL signalling after this indication.

FFS if the NW indication is a one-shot or also subsequent packets

In RAN2 meeting #123, several options for PSI-based SDU discarding were discussed. However, no consensus on a solution could be reached [3].

In this contribution, we further discuss remaining issues in the PSI-based SDU discarding.

# Discussions

## On how the network selects a UE or UEs to perform the PSI-based discarding

For the gNB to select one or more UEs to perform the PSI-based SDU discarding, the gNB should estimate a congestion level of the network congestion being experienced to know how much relief is required. One goal of the gNB in selecting UE(s) for performing the PSI-based discarding is to minimize the number of UEs whose QoE will be impacted and/or to minimize the impact to the QoE of the selected UE(s). To achieve that, the gNB should determine how much relief each candidate UE can potentially contribute to by discarding SDUs. How much relief that a candidate UE can contribute depends on a number of factors, such as the UL channel condition of the UE, what amount of UL XR traffic that the UE can discard for a given threshold, and how the QoE of the UE may be impacted by discarding that amount of UL XR traffic.

Currently, the gNB can estimate the UL channel condition of a UE. However, the gNB doesn’t know how many different PSI values that the UE will use nor how the UL XR traffic of the UE are distributed among the different PSI values, because, according to RAN2 agreement, the PSI values are determined by the UE implementation.

Observation 1: The gNB doesn’t know how many different PSI values that an XR UE uses nor how the UL XR traffic of the UE are distributed among the different PSI values.

## On how the network determines the right threshold to indicate to a UE

After selecting a UE, the gNB is to indicate a threshold to the UE to apply for handling the SDU discarding. If the threshold itself is an amount or a percentage of XR traffic that the UE is to discard, then it is completely up to the UE implementation to determine which packets are to be discarded and which ones are to be retained for transmission, because the PSI value itself is determined by the UE implementation in the first place. On the other hand, if the gNB-indicated threshold is a PSI threshold, then, ideally speaking, the gNB needs to know how the UL XR traffic of the UE are distributed among the different PSI values before it can give a meaningful PSI threshold to the UE.

Observation 2: Unless the gNB-indicated threshold itself is an amount or a percentage of XR traffic that the UE is to discard, the gNB needs to know how the UL XR traffic of the UE are distributed among the different PSI values before it can give a meaningful PSI threshold to the UE.

Indicating to the UE to discard certain amount of its UL XR traffic will degrade the QoE of the user using the XR service. It is unknown how much impact on the QoE will be resulted from the UE’s discarding certain amount of UL XR traffic nor how much QoE degradation may be acceptable to the XR user. On one hand, discarding a small amount of UL XR traffic may not help to relieve the network congestion; on the other hand, discarding too much, e.g., more than 20% of the video packets, may result in such bad QoE that the user may simply give up the XR service. Actually, forcing a single XR user, e.g., the one with the worst channel condition, out of the service may produce a greater relief to the network congestion than simultaneously hitting multiple users with the PSI-based discarding. Therefore, more study may be needed on how the network determines the right threshold for PSI-based discarding for a given UE, taking into account the impact to the QoE and a level of QoE degradation that is acceptable to the user of the XR service. For that, RAN2 may need some help from other WGs such as SA4 and SA2 to better understand the potential impacts on the QoE by the PSI-based SDU discarding and whether any assistance information can be provided from CN to RAN for the gNB to set a proper PSI threshold that is acceptable from service/QoE PoV.

Observation 3: More study may be needed on how the network determines the right threshold for PSI-based discarding for a given UE, taking into account the impact to the QoE and a level of QoE degradation that is acceptable to the user of the XR service, and on whether any assistance information can be provided from CN to RAN for the gNB to set a proper PSI threshold that is acceptable from service/QoE PoV.

## On Soft (timer-based) discarding vs. Hard discarding

We think most proposed solutions on the table can be classified as either a soft-discarding (e.g., timer-based) approach or a hard-discarding approach (i.e., discarding upon arrival).

For the timer-based soft-discarding approach, if the timer duration is a non-zero value, the SDU will be buffered at the PDCP entity for a period of time and hence a chance that one or more of the following steps may have occurred when the timer associated with the SDU expires: 1) a COUNT value has been assigned to the SDU; 2) PDCP pre-processing has been done to the SDU with the assigned COUNT value (because pre-processing may be so critical for meeting the stringent delay requirement in XR that it may begin as soon as the SDU arrives and a COUNT value is assigned); 3) the PDCP PDU produced by the SDU has been submitted to the underlying RLC entity but a transmission of it hasn’t begun yet; and 4) a transmission of a RLC PDU produced from the PDCP PDU has begun but not finished yet.

When the SDU is discarded after a COUNT value has been assigned to it, whether the COUNT value can be reused on the next SDU to be transmitted depends on a number of factors. For example, if a transmission of a resultant RLC PDU of the SDU has begun, the COUNT value cannot be reused. If the PDCP pre-processing on the next SDU to be transmitted has been done, it is also unlikely or undesirable that the UE spends extra computing and battery power and extra time to re-do the PDCP processing to reuse the COUNT value assigned to the discarded SDU. Therefore, the timer-based soft discarding with a non-zero timer duration may result in a higher chance of having a gap of missing COUNT values at the receiving PDCP entity. The COUNT gap may trigger unnecessary reordering if out-of-order delivery is not configured.

Meanwhile, for the hard discarding approach, because the discarding of an SDU is evaluated and executed upon (or before) its arrival at the transmitted PDCP entity, no COUNT value will be assigned to it and hence no COUNT gap will be created at the receiving PDCP entity due to the discarding of the SDU.

## Observation 4: The timer-based soft discarding with a non-zero timer duration may result in a higher chance of having a COUNT gap at the receiving PDCP entity; meanwhile, the hard discarding approach doesn’t have that problem.

Based on the above observations, we propose the following:

Proposal 1: RAN2 consider the above observations when designing for the PSI-based SDU discarding mechanism for network congestion.

Proposal 2: RAN2 send an LS to SA4/SA2 asking them to evaluate any potential impacts on the QoE by the PSI-based SDU discarding and whether any assistance information can be provided from CN to RAN for the gNB to set a proper PSI threshold that is acceptable from service/QoE PoV.

# Conclusions

Observation 1: The gNB doesn’t know how many different PSI values that an XR UE uses nor how the UL XR traffic of the UE are distributed among the different PSI values.

Observation 2: Unless the gNB-indicated threshold itself is an amount or a percentage of XR traffic that the UE is to discard, the gNB needs to know how the UL XR traffic of the UE are distributed among the different PSI values before it can give a meaningful PSI threshold to the UE.

Observation 3: More study may be needed on how the network determines the right threshold for PSI-based discarding for a given UE, taking into account the impact to the QoE and a level of QoE degradation that is acceptable to the user of the XR service, and on whether any assistance information can be provided from CN to RAN for the gNB to set a proper PSI threshold that is acceptable from service/QoE PoV.

## Observation 4: The timer-based soft discarding with a non-zero timer duration may result in a higher chance of having a COUNT gap at the receiving PDCP entity; meanwhile, the hard discarding approach doesn’t have that problem.

Based on the above observations, we propose the following:

Proposal 1: RAN2 consider the above observations when designing for the PSI-based SDU discarding mechanism for network congestion.

Proposal 2: RAN2 send an LS to SA4/SA2 asking them to evaluate any potential impacts on the QoE by the PSI-based SDU discarding and whether any assistance information can be provided from CN to RAN for the gNB to set a proper PSI threshold that is acceptable from service/QoE PoV.

# References

R2-2301902, RAN2-121 Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia).

R2-2306542, RAN2-122 Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia).

R2-2308962, RAN2-123 Report on LTE, MUSIM, QoE and XR, Vice Chairman (Nokia).