3GPP TSG-RAN WG2 #123bis	Tdoc R2-2311170

Xiamen, China, October 9th- 13th, 2023

Agenda Item:	7.8.1

Source:	Ericsson

Title:	Signalling of NS values for Aerial UEs

Document for:	Discussion, Decision

# Introduction

The WID for supporting UAV in NR [1] contains an objective to address the additional out-of-band emissions (OOBE) requirements mandated by ECC Decision 22(07) for aerial UEs.

5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information.

The same requirements apply to LTE and is captured in a separate WID [2].

RAN4 has worked on this objective and sent an LS to RAN2 [3], asking RAN2 to evaluate two solutions identified by RAN4 for signalling the additional OOBE requirements to aerial UEs. We present our view on these solutions in the present contribution.

# Signalling of additional OOBE requirements

Regarding how network indicates to aerial UEs that they need to fulfill new emission requirements, it is evident from the RAN4 LS that there is a consensus in RAN4 on introducing new NS values applicable to aerial UEs. However, there are two options related to the detailed signalling, as follows:

RAN4 identified two different alternatives that would be applicable to both NR and LTE, without identified impact for legacy users. - Option1: Introduce new NS indexes (related to the IE additionalSpectrumEmission) that will be applicable only for aerial UEs. The multiNS support feature already designed by RAN2 might be used to provide further NS signaling for the aerial UEs. However, RAN4 noticed this multiNS feature is mandatory for NR UEs but optional for LTE UEs. - Option2: Specify a new Information Element. This new IE should contain the list of NS values applicable for UAV UEs only for the considered bands. When attaching to the cell, the UAV shall then support both the list of legacy NS values (from “NR-NS-PmaxList” in SIB4), and the list of “UAV specific” NS values (from the new IE element).

and the expected RAN2 action:

RAN4 kindly asks RAN2 to: - Evaluate the feasibility of the above two options, checking there won’t be any non-backward compatibility issue with legacy LTE and NR UEs, and that an aerial UE always apply the aerial NS when configured together with other non-aerial NS, and consider the implementation of the best option after informing RAN4 of RAN2’s choice.

As of now, for both Option 1 and Option 2, we don’t see any non-backward compatibility issue with legacy LTE and NR UEs. However, Option 2 has one drawback. That is, it requires LTE UEs capable of Rel-18 aerial features to also support multiNS-PmaxList (which is currently an optional feature), as analysed in [4]. In our view, Option 2 is neater and can avoid any potential undetected issues with Option 1.

Besides signalling the aerial-specific NS values to aerial UEs, the network should also indicate when an aerial UE should apply these NS values. That is because, in our understanding, the extra OOBE requirements should only be mandatory when the aerial UE is airborne to limit the potentially high interference to other bands due to the light-of-sight propagation condition. When the aerial UE is grounded or near the ground and is connected to network, e.g., to upload the data it has collected in a completed mission, the UE should be able to apply the legacy NS values of terrestrial UEs.

Therefore, it is necessary to the network to indicate conditions under which an aerial UE shall apply aerial-specific NS values. For example, the aerial UE shall apply the aerial-specific NS values if the UE is in the flying state, e.g., when the altitude of the UE is above a height threshold.

- Add an “additionalSpectrumEmissionUAV” applicable for UAVs when they are flying (Option 2 in RAN4 LS).

There is another aspect we believe RAN2 should clarify in the LS reply to RAN4. That is, RAN4 should mandate that the Aerial UEs shall support both the legacy NS values and the new NS values for the considered bands.

- Reply to RAN4 that RAN4 should mandate aerial UEs to support both legacy NS values and the newly defined aerial-specific NS values in the considered bands.

# Aerial UE capabilities

In the LS, RAN4 also asks RAN2 about aerial UE capabilities:

RAN4 kindly asks RAN2 to: - Evaluate the feasibility of the above two options, checking there won’t be any non-backward compatibility issue with legacy LTE and NR UEs, and that an aerial UE always apply the aerial NS when configured together with other non-aerial NS, and consider the implementation of the best option after informing RAN4 of RAN2’s choice. - Indicate to RAN4 in whether a RAN2 capability will indicate that the UE supports UAS (Uncrewed Aircraft Systems) in NR.

In our view, it is natural for RAN2 to adopt the approach in Rel-15, namely defining some mandatory aerial UE features. Which features out of the ones being specified in Rel-18 to be mandatory can be discussed at the end of the release.

- RAN2 to inform RAN4 the support of Aerial UE features in Rel-18. Which features to be mandatory are FFS.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add an “additionalSpectrumEmissionUAV” applicable for UAVs when they are flying (Option 2 in RAN4 LS).

Proposal 2	Reply to RAN4 that RAN4 should mandate aerial UEs to support both legacy NS values and the newly defined aerial-specific NS values in the considered bands.

Proposal 3	RAN2 to inform RAN4 the support of Aerial UE features in Rel-18. Which features to be mandatory are FFS.

# References

RP-230782, Revised WID: NR support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

RP-230783, New WID: Enhanced LTE support for UAV (Uncrewed Aerial Vehicles), RAN#99, March 2023

R4-2314700, LS on the handling of additional regulatory requirements for UAV UEs, RAN4#108

R4-2313431, On the identification of UAVs and NS signalling, Nokia, RAN4#108

# 6	Annex – Draft LS

3GPP TSG-RAN2 Meeting #123bis R2-23xxxxx # Xiamen, P.R. China, 9-13 October 2023 Title: Reply LS on the handling of additional regulatory requirements for UAV UEs Response to: Release: Rel-18 Work Item: - Source: Ericsson (to be RAN WG2) To: RAN WG4 Cc: - Contact person: Helka-Liina Määttänen helka-liina.maattanen@ericsson.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: - # 1 Overall description RAN2 thanks RAN4 for the LS in R4-2314700. Regarding NS-values: RAN2 would like to inform RAN4 that RAN2 has selected Option 2 described in R4-2314700 by adding a field additionalSpectrumEmissionUAV which the UAVs shall apply when flying above a threshold. RAN2 will include this field in their Rel-18 specifications. Further, RAN2 agreed that aerial UEs to support both legacy NS values and the newly defined aerial-specific NS values in the considered bands. Regarding UE capabilities: RAN2 will discuss further which features are mandatory and reply RAN4 at a later time. # 2 Actions To RAN WG4 ACTION: RAN2 asks RAN4 to take the above into account in their future work. # 3 Dates of next TSG RAN WG2 meetings RAN2#124 November 13-17, 2023 Chicago, USA RAN2#125 February 26 – March 1, 2024 Athens, Greece