3GPP TSG-RAN WG2 #123-bis	R2-2311200

Xiamen, P.R. China, October 9th – 13th 2023

Agenda Item:	7.13.5

Source:	Ericsson

Title:	Enhancements of SON reports for NR-U

Document for:	Discussion, Decision

# 1	Introduction

Given the progress in RAN2#123, only few open issues are left to be discussed in the context of NR-U.

In this paper, we discuss those few open issues that are partially addressed also in the email discussion [Post122][590][R18 SONMDT] Open issues of SON NR-U (Ericsson).

# 2	Discussion

### 2.1 Enhancements to the RA-InformationCommon

The first issue to be addressed on the RA-InformationCommon is related to the following agreement:

From RAN2#122:

On how to represent the preamble transmission attempts blocked by LBT,

Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

It was discussed whether the number of preamble transmissions should be represented by a new field, or if some existing field can be reused. Related to the latter approach, it was proposed that the fields numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS could be modified, such that they account for all the preamble transmission attempts both the ones blocked by LBT and the successful ones. From our point of view, the main problem of this option is that currently the maximum  values for the numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS is limited to 200. This value was dimensioned considering only the RA preambles actually transmitted. If now, this field also considers the preambles blocked by LBT, the total numberOfPreamblesSentOnSSB (numberOfPreamblesSentOnCSI-RS) might exceed 200. In our view this should be avoided, and the approach of introducing a new field is more robust from a technical perspective.

A new field is introduced to count the number of preamble transmissions blocked by LBT per RA procedure.

Proponents of the approach of reusing existing fields claim that an advantage of this approach is that the UE will always set the numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS to a value. Otherwise, there is the problem that numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS are mandatory fields and cannot be set to 0, which could happen in case of no preamble transmitted in an SSB. However, we note that the same issue affects also the perRAAttemptInfoList that should be always set. We note that leaving empty the entry of the perRAAttemptInfoList (i.e. avoid setting the fields in perRAAttemptInfoList) cannot be a viable solution, because there are other scenarios in which an entry of the perRAAttemptInfoList can be left empty (e.g. CFRA).

Hence, irrespective of the solution, RAN2 should discuss how to handle the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS and perRAAttemptInfoList. For example, one solution could be that it is clarified in the specification (e.g. in a NOTE) that in case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS. RAN2 should then discuss how the UE should inform the network that all the preamble transmission attempts in that beam were blocked by LBT.

In case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the UE implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS.

RAN2 to discuss how the UE should inform the network that all the preamble transmission attempts in a beam were blocked by LBT.

### 2.2 Enhancements to the RLF-Report

Related to which BWP include in the RAInformationCommon within the RLF-Report, the following was agreed:

As baseline, RAN2 assumes the following:

a.	Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b.	The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, FFS whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c.	The above bullets may be revisited case by case depending on future agreements.

We note that for the case of RA-Report, the UE includes the information associated to all BWPs in which consistent LBT failure was detected in the RA-Report. There seems to be no clear reason for not including the information about consistent LBT failures in the RA-InformationCommon included in the RLF-Report.

The benefit of this is quite clear. In NR-U systems, the reason for switching the BWP upon consistent LBT failure is because the UE is expected to experience different LBT performances in another BWP, i.e. LBT performed in different chunks of the NR-U band may lead to different LBT results.
Hence knowing the BWPs in which consistent LBT failure occurred prior to the RLF/HOF allows the NW to identify the problematic BWPs and optimize the usage of those. For example, the network can figure out that the consistent LBT failures are persistently experienced in a certain BWP, and then optimize the configuration/allocation of this BWP in the NR-U spectrum.

The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of all the BWPs in which the UE experienced the consistent UL LBT failures prior the RLF/HOF.

#### 2.2.1 RSSI reporting

Related to RSSI reporting in the RLF-Report, the following agreements were taken in RAN2#122:

From RAN2#122:

The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

8	The UE should log the following RSSI values in the RLF-Report:

a.	For RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency.

b.	FFS: For HOF, the latest measured RSSI of the NR-U channel of the source cell, and  the latest measured RSSI of the NR-U channel of the target cell, if measRSSI-ReportConfig is configured for the corresponding frequency.

Related to RLF case, in our view the RSSI measurements of the source cell should be considered also for the HOF. As in classical MRO, the network can use the RSSI information (together with the legacy RSRP/RSRQ/SINR) to compare the quality of the frequency of the source cell with the quality of the frequency of the target cell or neighbouring cells. For example, if the RLF-Report shows very bad RSSI on the target frequency, and still acceptable RSSI on the source frequency, then the NW can conclude that this HO was executed at a bad point in time, and the UE could have been kept longer time in the source frequency. Accordingly, the NW can adjust the thresholds on HO triggering.
Please note that this is exactly the same reasons for which the RSRP/RSRQ/SINR of the source as well as neighbouring cells is included in legacy.

The RSSI measurement results of the frequency associated to the last serving cell are included in the RLF-Report for the source PCell (in case of HO failure) and for the PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequency.

Similarly, for neighbouring frequencies, if the RLF-Report shows very bad RSSI on the target frequency, but good RSSI on another neighbouring frequency, then the NW can conclude that this HO was executed to a wrong frequency, and hence it can adjust the thresholds on HO triggering.

The UE includes in the RLF-Report the available RSSI measurement results of the frequencies associated to the neighbouring cells other than the source PCell (in case of HO failure) or PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequencies.

#### 2.2.3 On the inclusion of lbt-FailureRecoveryConfig in the RLF-Report

An LS [2] was sent to RAN3 asking the possibilities of keeping the LBT failure recovery configuration at the network side. In the reply [3] to LS, RAN3 has replied that although one similar mechanism exists in RAN3, it would be practically minimal to store such information as LBT recovery procedure for a long time since that limits the network functionality. This is shown on the following excerpt of RAN3 LS [2].

RAN3 observes though, that the above mechanism was designed with intention to retrieve UE context or the configuration information if the UE attempts reconnection and reports the failure right after connection failure; if the failure information is fetched from the UE hours after the failure, then the likelihood that the source and the last serving node can retrieve the needed information depends on RAN implementation and is practically minimal (this depends on RAN implementation, e.g., how long the gNB stores the UE context or how long that allocated C-RNTI is not reused by the RAN).

According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

In order to minimize the UE impact and the size of the RLF-Report, we propose a UE-based solution where UE can store the LBT-Recovery configuration information in the RLF report only in case of re-establishment procedure failure after the RLF.

UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

### 2.3 Enhancements to the SHR

Similar to the legacy SHR, RSSI measurements of source/target/neighbouring frequencies should be included. In SHR, the source/target/neighbouring frequencies are always included irrespective of the SHR triggering condition. That is because in order to optimize the HO, and reduce the probability of possible RLF or performance degradation, the network has to evaluate the quality of neighbouring frequencies. For example, if SHR is generated due to T310 triggering condition, it could be that the source cell quality was not optimal; however, in order to avoid this issue, e.g. taking the HO at an earlier point in time to another cell, the network has to know the quality of the target/neighbouring cells.

The SHR includes the RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A new field is introduced to count the number of preamble transmissions blocked by LBT per RA procedure.

Proposal 2	In case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the UE implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS.

Proposal 3	RAN2 to discuss how the UE should inform the network that all the preamble transmission attempts in a beam were blocked by LBT.

Proposal 4	The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of all the BWPs in which the UE experienced the consistent UL LBT failures prior the RLF/HOF.

Proposal 5	The RSSI measurement results of the frequency associated to the last serving cell are included in the RLF-Report for the source PCell (in case of HO failure) and for the PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequency.

Proposal 6	The UE includes in the RLF-Report the available RSSI measurement results of the frequencies associated to the neighbouring cells other than the source PCell (in case of HO failure) or PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequencies.

Proposal 7	UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

Proposal 8	The SHR includes the RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

# 4	References

- R3-225241, LS on NR-U support for MRO, 3GPP TSG-RAN WG2 #117-e

- R2-2305424, Discussion on SON for NR-U, Nokia, Nokia Shanghai Bell