3GPP TSG-RAN WG2 Meeting #123bis	R2-2311231

Xiamen, China, 9 – 13 October 2023

Agenda Item:	7.7.1

Source:	Ericsson

Title:	Rapporteur’s input to 38.331 regarding TN area information

Document for:	Information

# 1	Discussion

After RAN2#123 and during the post meeting email discussion [1], TN coverage area ID indication was added to SIB4 and SIB5 following the relevant agreements taken in previous meetings:

- An RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency, regardless of the frequency priority

- Reuse the same format of Rel-17 referenceLocation and distanceThresh for signaling the TN coverage area centre and radius

- Both of the NR TN coverage and EUTRA TN coverage can be provided.

- We introduce a new SIB to provide the TN coverage information

- A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

Its ASN.1 implementation was the following, which limits a single TN Area ID per carrier frequency.

InterFreqCarrierFreqInfo-v1800 ::=  SEQUENCE {

tn-areaId-r18                       TN-AreaId-r18                                               OPTIONAL     -- Need R

}

Right after the conclusion of the RRC running CR revision, a few companies highlighted a very relevant comment regarding the inclusion of TN coverage areas in System Information. Previous contributions and the final agreement in the list above reflect the intention to associate a single carrier frequency with several TN coverage areas given that commonly frequencies are spread and re-used in large areas. Thus, this new running RRC CR version includes a few changes that support this new implementation.

A list of TN areas has been added per carrier frequency in both SIB4 and SIB5, for example:

InterFreqCarrierFreqInfo-v1800 ::=  SEQUENCE {

tn-areaIdList-r18                   SEQUENCE (SIZE (1..maxTN-areaInfo-r18)) OF TN-AreaId        OPTIONAL     -- Need R

}

A new IE has been created for the TN Area ID since it will be re-used in different SIBs (SIB4/5/XX).

The IE TN-AreaId is used to identify a TN coverage area within an NTN cell.

TN-AreaId information element

-- ASN1START

-- TAG-TN-AREAID-START

TN-AreaId :: =                 INTEGER (1..maxTN-AreaInfo)

-- TAG-TN-AREAID-STOP

-- ASN1STOP

# References

R2-2309341, Stage 3 Running RRC CR for NR NTN Rel-18, Ericsson, RAN2#123, Toulouse, France, August 2023.