3GPP TSG-RAN WG2 #123bis	R2-2311242

Xiamen, China, Oct. 9 – 13, 2023

Agenda Item:	7.6.2.2

Source:	Ericsson

Title:	R18 IoT NTN GNSS operation enhancements

Document for:	Discussion, Decision

# 1	Introduction

In this paper we discuss the remining issues for GNSS enhancements.

# 2	Discussion

## 2.1	Reporting GNSS validity duration

The validity duration reported by the UE is always an estimate that may depend on the quality of the UEs sensors and the UEs ability to compensate movements by utilizing such sensors. Also, the UE may after a while have a better estimation of the validity duration.

The GNSS validity duration and the GNSS position fix duration are always estimates by the UE

For the issue of informing the eNB of successful acquisition of a new GNSS position, the UE has to provide a GNSS validity duration for each position fix, thus eNB reception of an GNSS validity duration means the UE acquired a new position fix.

Reception of a GNSS validity duration indicates to the eNB that the UE has acquired a new GNSS position fix.

Further, there is only one reserved UL-SCH LCID left and in the Running CR, it is proposed to use it for “Remaining GNSS Validity Duration Report”. It can be questioned if NTN application shall be allowed to use this last reserved LCID – possible future important functionality will need it.

We may for example

1) define eLCIDs for the new report

2) consider reusing some of the already defined LCIDs that are not necessary to support in non-terrestrial networks (possibly the AUL MAC CE LCIDs is unnecessary in NTN)

RAN2 to discuss how to accommodate the UL-SCH MAC CE LCID needed for IoT NTN.

## 2.2	Extended GNSS validity duration

RAN1 have made the following agreements on extension of the validity of the GNSS position after the validity duration:

Agreement

Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time

Agreement

At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

- FFS: with legacy closed loop time correction or enhanced closed loop time correction

- This mechanism is enabled/configured by eNB

- FFS: whether such mechanism will be specified depends on the outcome of this study

Agreement

From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition.

RAN1 will decide further details of the above.

Agreement

From RAN1 perspective, down select one for the duration X:

- Alt-3: when timeAlignmentTimer is not infinity, X is equal to remaining timeAlignmentTimer;

when timeAlignmentTimer is infinity, X is equal to Y;

  - FFS: whether X can be used to extend the original GNSS validity duration

  - Y is a configured value.

Note 1: The feature can be enabled/disabled by network

Note 2 (as already agreed): The duration X is where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition.

There are some details remaining to decide for RAN1, however we can start implementing the functionality in the MAC and RRC.

Add RRC parameter gnss-ExtendedValidity, ENUMERATED {enabled} OPTIONAL Need OR, that enables the extended GNSS validity functionality.

For Y, add RRC parameter gnss-ExtendedValidityTimer, range FFS by RAN1, OPTIONAL Need OR.

The GNSS validity is kept track of in RRC, while the TAT handling is in MAC. This new functionality may be in either RRC or MAC, both options will require some signalling between the layers.

We believe it is easier, when this functionality is enabled, that RRC indicates GNSS validity expired to MAC (and possibly when/if a new GNSS position fix is acquired to cancel the extended GNSS validity functionality), and MAC can then indicate back to RRC when TAT expires (such that RRC can trigger to go to IDLE).

In RRC_CONNECTED when gnss-ExtendedValidity is enabled, when GNSS becomes out-of-date RRC shall indicate to lower layers (MAC) that extended GNSS validity is started.

If RRC_CONNECTED when gnss-ExtendedValidity is enabled and extended validity has been started and UE acquires GNSS position, RRC indicates extended GNSS validity is stopped to lower layers.

When MAC receives indication from upper layers that extended GNSS validity is started, MAC shall decide the remaining GNSS validity (X in RAN1 agreement) and start a timer for remaining validity.

If MAC receives indication from upper layers that extended GNSS validity is stopped, the MAC entity stops the timer for extended validity.

When the timer for remaining validity expires, the MAC entity shall inform upper layers that extended GNSS validity has expired.

If RRC receives indication from lower layers that extended GNSS validity has expired, RRC shall perform the actions related to going to IDLE.

## 2.3	Triggering GNSS measurement

This

#### 6.1.3.xx	GNSS Measurement Command MAC Control Element

The GNSS Measurement Command MAC Control Element is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1.

Editor’s note: FFS on the figure of GNSS Measurement Command MAC CE.

If the gap configuration is done with RRC, there is no need to indicate any gap length in the trigger nor to have a figure.

The following RAN1 agreements are then considered:

Agreement

For the GNSS measurement gap aperiodically triggered with MAC CE, the duration for the GNSS measurement gap can be configured by eNB.

- The gap duration is equal to the latest reported GNSS position fix time duration for measurement when the duration for GNSS measurement gap is not included in the configuration by eNB.

Agreement

- UE reports GNSS position fix time duration for measurement at least during the initial access stage.

  - which message carries this information is up to RAN2

Agreement

UE reports only one GNSS position fix time duration for GNSS measurement at least when moving to RRC connected state.

Agreement

From RAN1 perspective, during connected mode, reporting of GNSS position fix time duration is not needed except via RRCConnectionReestablishmentComplete, RRCConnectionReestablishmentComplete-NB and RRCConnectionReconfigurationComplete for HO case.

Agreement

Network can configure the length for GNSS measurement timer via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31] second.

- FFS: other component values

- Note: RAN2 can further discuss whether separate configurations are needed for GNSS measurement gap and GNSS measurement timer

In RRC, add configuration of gnss-MeasGap for the duration of a GNSS measurement gap with possible values are [1,2,3,4,5,6,7,13,19,25,31].

If gnss-MeasGap is not configured, then the duration of a GNSS measurement gap is the latest reported GNSS position fix duration.

In MAC 6.1.3.xx, add for the MAC CE “It has a fixed size of zero bits.”.

Further, RAN1 made the following agreements:

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at

- n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC,

- or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled

- X1 is predefined values, where X1=12ms for NB-IoT, and FFS X1 for eMTC

- FFS: X2 is predefined value or configured value.

Agreement

For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, the start time of the gap should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled

- X1=12ms for NB-IoT

- X1=6ms for eMTC

However, there is no reason whatsoever for the network to know EXACTLY when a measurement gap start, nor when it ends.

The eNB cannot schedule the UE during the gap, and as the GNSS acquisition may fail, there is no good reason to schedule the UE after the gap, if the old validity duration has expired – unless the UE has made clear that the measurement was successful.

The only reasonable eNB implementation will after sending the UE to measure GNSS (which is expected to take between 1 and 31 seconds which is 1000 to 31000 subframes) – a procedure that may fail and where the UE always shall report back a new GNSS validity duration when finished - will wait until the UE reports the new GNSS validity duration.

The time for the UE to measure GNSS is thousands of subframes, during which the UE may have moved, or link adaptation have become outdated, or satellite coverage may have changed, or timing advance have changed significantly more than the UE specific Koffset was configured for.

Scheduling a UE after a GNSS measurement gap can be a waste of resources because the GNSS acquisition can fail.

The potential delay gain by scheduling a UE after a measurement gap, before the UE has indicated successful GNSS acquisition to the eNB, is neglectable in comparison to the total time for GNSS measurement.

After triggering a UE to measure GNSS, there are no technical reasons for the eNB to schedule the UE before the UE indicate a successful GNSS acquisition to the eNB.

RAN1 already agreed the UE shall report the new GNSS validity duration:

Agreement

In RRC connected, every time after successful GNSS measurement, UE reports the new remaining GNSS validity duration.

FFS: Whether UE should report the new remaining GNSS validity duration within a duration D.

And UE can send RA to tell when it is done:

Agreement

The UE is not required to monitor N/MPDCCH within the aperiodic GNSS measurement gap, except after a CBRA (PRACH) is sent.

- CBRA (PRACH) can be sent at least to request UL resource to report the remaining GNSS validity duration.

Note1: The CBRA (PRACH) can only be sent within the duration after UE reacquires GNSS successfully to the end of the gap.

Note2: Whether CBRA (PRACH) is sent is up to UE implementation.

Note3: no change to existing CBRA procedures

FFS: whether other RA procedure is needed.

Therefore, we propose to have the UE trigger random access if the old validity duration has expired, to inform eNB of the result – this is also very useful if the UE finishes the measurement early.

After a successful acquisition of GNSS, the UE shall trigger random access to inform the eNB of the success by reporting the GNSS validity duration.

Waiting until a UE comes back with a random access after a GNSS measurement ensures the UE is reachable as the cell specific Koffset is used during RA. The UE can report the TA, and eNB can update link adaptation and the UE specific Koffset, if needed.

For failed GNSS measurements, RAN1 agreed:

Agreement

From RAN1 perspective, after autonomous GNSS measurement timer expires if UE failed to re-acquire GNSS position fix within the autonomous GNSS measurement timer UE goes to IDLE mode

Agreement

From RAN1 perspective, for the aperiodic GNSS measurement gap triggered by eNB with MAC CE, down select one of the alternatives for the failure of GNSS measurement:

- Alt-1: UE goes to IDLE mode after the end of GNSS measurement gap if UE failed to re-acquire GNSS position fix within GNSS measurement gap.

After a failed acquisition of GNSS, the UE shall go to IDLE if the GNSS validity duration has expired.

## 2.4	The running RRC CR

Running RRC CR says:

#### 5.3.3.21	UE actions upon indication of out-of-date GNSS position

Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall:

1>	if the UE does not support performing GNSS fix in GNSS measurement gap:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

1>	else if no indication of network triggered GNSS measurement is received from lower layers, and gnss-AutonomousEnabled is not configured:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

Editor’s Note: Agreement:	UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry

Editor’s Note: Agreement:	If there is neither network aperiodically trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_IDLE after GNSS becomes invalid. It’s FFS how to decide GNSS valid or invalid considering duration X and Y.

First there is a small difference in terminology to the existing spec:

In RRC CR 5.3.3.21, change “if the UE does not support performing GNSS fix in GNSS measurement gap:” to “if the UE does not support acquiring GNSS position in a GNSS measurement gap:”

Then there is a problem if the UE is already measuring the GNSS when the GNSS position becomes outdated, the UE shall not go to idle if the measurement is already ongoing.

In RRC CR 5.3.3.21, change “else if no indication of network triggered GNSS measurement is received from lower layers, and gnss-AutonomousEnabled is not configured:” to “else if no GNSS measurement is ongoing, and gnss-AutonomousEnabled is not configured:”

Then we need to decide what the UE shall do if the GNSS measurement fails.

In RRC CR 5.3.3.21, at the end of section add what shall happen when a GNSS measurement fails and there is no valid GNSS position: 
“If the UE do not have a valid GNSS position in RRC_CONNECTED and the UE fails to acquire a GNSS position fix during a GNSS measurement gap, the UE shall perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.”

Here is a text proposal for the RRC spec for the proposals in section 2.3, 2.4 and 2.5 (this section):

#### 5.3.3.21	UE actions upon indication of out-of-date GNSS position

Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall:

1>	if the UE does not support acquiring GNSS position in a GNSS measurement gap:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

1>	else if gnss-ExtendedValidity is enabled:

2>	indicate extended GNSS validity start to lower layers.

1>	else if no GNSS measurement is ongoing:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

Adopt the text proposal above for RRC spec section 5.3.3.21.

### 5.5.x	GNSS measurement triggering and reporting

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

While in RRC_CONNECTED the UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap, as specified in TS 36.213 [23];

Editor’s Note: FFS on how to determine the gap length, and whether to implement it in RRC or MAC spec.

1>	if gnss-AutonomousEnabled is configured:

2>	perform GNSS measurement using autonomous gaps;

Editor’s Note: FFS the start of autonomous gaps, e.g. “after X expires on top of GNSS validity duration expiry” or X is included within the GNSS validity duration. The exact X (and Y) is pending on RAN1 further discussion.

NOTE:	UE can also autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid, and the exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon indication that GNSS becomes valid:

2>	indicate to lower layers that GNSS is valid and to report the remaining GNSS measurement validity duration (see TS 36.321 [6]).

1>	if the UE fails acquiring a GNSS position during a measurement gap; and

1>	 if the UE do not have a valid GNSS position:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

1>	 if indication of extended GNSS validity expiry is received from lower layers:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

Adopt the text proposal above for RRC spec section 5.5.x.

For

[[	gnss-AutonomousEnabled-r18		ENUMERATED {true}				OPTIONAL	-- Need ON

]]

We think the correct shall be

[[	gnss-AutonomousEnabled-r18		ENUMERATED {true}				OPTIONAL	-- Need OR

]]

Because otherwise it is not possible to stop using the autonomous GNSS acquisition, for example for a handover to a cell that does not support autonomous GNSS acquisition.

The gnss-AutonomousEnabled shall have need code ON

At the same place we can add the config of gnss-ExtendedValidity:

[[	gnss-AutonomousEnabled-r18		ENUMERATED {true}				OPTIONAL,	-- Need OR

gnss-ExtendedValidity-r18		ENUMERATED {true}				OPTIONAL,	-- Need OR

gnss-ExtendedValidityTimer-r18	ENUMERATED {FFS}				OPTIONAL	-- Need OR

]]

Adopt the above text proposal for adding gnss-ExtendedValidity and gnss-ExtendedValidityTimer.

## 2.5	The running MAC CR

This:

## 5.xx GNSS measurement

The network may request a NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network to perform GNSS measurement by sending the GNSS Measurement Command MAC CE described in clause 6.1.3.xx.

The MAC entity shall:

-	if the MAC entity receives a GNSS Measurement Command MAC CE on a Serving Cell:

-	indicate to upper layers to require the UE to perform GNSS measurement.

Editor’s note: UE can autonomously start GNSS measurement during the inactive state of C-DRX.

Editor’s note:	The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation.

We propose to reformulate to align with other clauses in the MAC spec. First GNSS is not related to a certain Serving Cell, thus no need to mention that:

In 5.xx, change “if the MAC entity receives a GNSS Measurement Command MAC CE on a Serving Cell” to “if the MAC entity receives a GNSS Measurement Command MAC CE”

Then it is a request for performing the measurement:

In 5.xx, change “indicate to upper layers to require the UE to perform GNSS measurement.” To “indicate to upper layers a request to perform GNSS measurement.”

The same place is suitable for the triggering of random access when GNSS acquisition was successful.

In 5.xx, at the end add 
“-	if upper layers indicate successful GNSS acquisition:
		-	initiate a Random Access procedure (see clause 5.1) on the SpCell.”

Further, for extended validity duration we propose to have a new section to capture the behaviour (or include in the 5.xx if the title is updated):

## 5.yy	Extended GNSS validity

When gnss-ExtendedValidity is configured, upon indication of extended GNSS validity duration start from upper layers, the MAC entity shall:

-	if the timeAlignmentTimer associated with the pTAG is configured to infinity:

-	start VALIDITY_TIMER with a value of gnss-ExtendedValidityTimer.

-	else if the timeAlignmentTimer associated with the pTAG is running:

-	start VALIDITY_TIMER with a value of timeAlignmentTimer associated with the pTAG.

-	else:

-	indicate expiry of extended GNSS validity to upper layers.

Upon expiry of VALIDITY_TIMER, the MAC entity shall:

-	indicate expiry of extended GNSS validity to upper layers.

Upon indication from upper layers that GNSS position is valid:

-	stop the VALIDITY_TIMER.

Adopt the above text proposal for Extended GNSS validity in the MAC spec.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	The GNSS validity duration and the GNSS position fix duration are always estimates by the UE

Observation 2	The time for the UE to measure GNSS is thousands of subframes, during which the UE may have moved, or link adaptation have become outdated, or satellite coverage may have changed, or timing advance have changed significantly more than the UE specific Koffset was configured for.

Observation 3	Scheduling a UE after a GNSS measurement gap can be a waste of resources because the GNSS acquisition can fail.

Observation 4	The potential delay gain by scheduling a UE after a measurement gap, before the UE has indicated successful GNSS acquisition to the eNB, is neglectable in comparison to the total time for GNSS measurement.

Observation 5	After triggering a UE to measure GNSS, there are no technical reasons for the eNB to schedule the UE before the UE indicate a successful GNSS acquisition to the eNB.

Observation 6	Waiting until a UE comes back with a random access after a GNSS measurement ensures the UE is reachable as the cell specific Koffset is used during RA. The UE can report the TA, and eNB can update link adaptation and the UE specific Koffset, if needed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reception of a GNSS validity duration indicates to the eNB that the UE has acquired a new GNSS position fix.

Proposal 2	RAN2 to discuss how to accommodate the UL-SCH MAC CE LCID needed for IoT NTN.

Proposal 3	Add RRC parameter gnss-ExtendedValidity, ENUMERATED {enabled} OPTIONAL Need OR, that enables the extended GNSS validity functionality.

Proposal 4	For Y, add RRC parameter gnss-ExtendedValidityTimer, range FFS by RAN1, OPTIONAL Need OR.

Proposal 5	In RRC_CONNECTED when gnss-ExtendedValidity is enabled, when GNSS becomes out-of-date RRC shall indicate to lower layers (MAC) that extended GNSS validity is started.

Proposal 6	If RRC_CONNECTED when gnss-ExtendedValidity is enabled and extended validity has been started and UE acquires GNSS position, RRC indicates extended GNSS validity is stopped to lower layers.

Proposal 7	When MAC receives indication from upper layers that extended GNSS validity is started, MAC shall decide the remaining GNSS validity (X in RAN1 agreement) and start a timer for remaining validity.

Proposal 8	If MAC receives indication from upper layers that extended GNSS validity is stopped, the MAC entity stops the timer for extended validity.

Proposal 9	When the timer for remaining validity expires, the MAC entity shall inform upper layers that extended GNSS validity has expired.

Proposal 10	If RRC receives indication from lower layers that extended GNSS validity has expired, RRC shall perform the actions related to going to IDLE.

Proposal 11	In RRC, add configuration of gnss-MeasGap for the duration of a GNSS measurement gap with possible values are [1,2,3,4,5,6,7,13,19,25,31].

Proposal 12	If gnss-MeasGap is not configured, then the duration of a GNSS measurement gap is the latest reported GNSS position fix duration.

Proposal 13	In MAC 6.1.3.xx, add for the MAC CE “It has a fixed size of zero bits.”.

Proposal 14	After a successful acquisition of GNSS, the UE shall trigger random access to inform the eNB of the success by reporting the GNSS validity duration.

Proposal 15	After a failed acquisition of GNSS, the UE shall go to IDLE if the GNSS validity duration has expired.

Proposal 16	In RRC CR 5.3.3.21, change “if the UE does not support performing GNSS fix in GNSS measurement gap:” to “if the UE does not support acquiring GNSS position in a GNSS measurement gap:”

Proposal 17	In RRC CR 5.3.3.21, change “else if no indication of network triggered GNSS measurement is received from lower layers, and gnss-AutonomousEnabled is not configured:” to “else if no GNSS measurement is ongoing, and gnss-AutonomousEnabled is not configured:”

Proposal 18	In RRC CR 5.3.3.21, at the end of section add what shall happen when a GNSS measurement fails and there is no valid GNSS position:  “If the UE do not have a valid GNSS position in RRC_CONNECTED and the UE fails to acquire a GNSS position fix during a GNSS measurement gap, the UE shall perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.”

Proposal 19	Adopt the text proposal above for RRC spec section 5.3.3.21.

Proposal 20	Adopt the text proposal above for RRC spec section 5.5.x.

Proposal 21	The gnss-AutonomousEnabled shall have need code ON

Proposal 22	Adopt the above text proposal for adding gnss-ExtendedValidity and gnss-ExtendedValidityTimer.

Proposal 23	In 5.xx, change “if the MAC entity receives a GNSS Measurement Command MAC CE on a Serving Cell” to “if the MAC entity receives a GNSS Measurement Command MAC CE”

Proposal 24	In 5.xx, change “indicate to upper layers to require the UE to perform GNSS measurement.” To “indicate to upper layers a request to perform GNSS measurement.”

Proposal 25	In 5.xx, at the end add  “- if upper layers indicate successful GNSS acquisition:   - initiate a Random Access procedure (see clause 5.1) on the SpCell.”

Proposal 26	Adopt the above text proposal for Extended GNSS validity in the MAC spec.

# References