3GPP TSG-RAN WG2 Meeting #123bis	R2-2311250

Xiamen, China, 9 - 13 Oct, 2023	 Revision of R2-2309870

Title: 	Rapporteur proposals to address open issues in MAC running CRs (open issue list)

Source: 	Huawei, HiSilicon

Agenda item:	7.4.2.2

Document Type:	Discussion and Decision

- Open issue list

Following are the open issue list before RAN2#123-bis meeting.

1.1 Early TA acquisition

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| Power ramping | How to handle the power ramping counter, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order? This is already implemented in the endorsed running CR. | Proposal 1: RAN2 confirm that power ramping counter is neither reset nor increased, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order. |
|  | Coexistence with LBT: FFS if UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if preamble transmission encounter the LBT failure. | Proposal 16: UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if prior preamble transmission encounters LBT failure. The update is made based on Tdoc: R2-2309721 (vivo), R2-2309770 (Samsung), R2-2310373 (OPPO), R2-2311093 (Qualcomm) |
| UE based TA measurement | How does the UE apply the TA value based on UE TA measurement? | Rapporteur think no MAC CE information is needed (i.e. RRC configuration and UE capability signalling are sufficient ) Proposal 2: If UE is configured by RRC to perform UE based TA measurement, UE applies the measured TA value and performs RACH-less LTM, upon LTM cell switch. |
|  | How to determine RACH-less LTM for the UE based TA measurement case? |  |
|  | Any other RRC configuration related to UE based TA measurement (e.g. DL timing difference between service and candidate)? Whether RAN2 still wait for RAN1/RAN4 further progress or start to implement this feature? | Update is made based on Tdoc: R2-2309786 (Futurewei), R2-2309546 (CATT), R2-2309851 (Samsung), R2-2310889 (Ericsson), R2-2311094 (Qualcomm) |

1.2 RACH-less cell switch

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| CG for RACH-less LTM | Whether UE/target cell is allowed to use both DG and CG for RACH-less LTM? | Proposal 3: No specification impact/restriction on the UE to use both DG and CG for RACH-less LTM. |
|  | Whether the CG configured for RACH-less LTM can be used for subsequent UL data transmission, after the successful transmission of first UL data (if not released by NW)? If yes, how to select the CG occasion for subsequent CG transmission (e.g. using any CG occasion or based on the indicated SSB)? | Proposal 4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion (to close one EN in MAC running CR): - Option 1: UE skips/stops using those CG; - Option 2: UE can continue use those CG. FFS UE can use any CG occasion (no need to further consider the SSB and CG occasion association). |
|  | Whether UE considers RACH-less LTM failure, if the configuredGrantTimer expires and if a PDCCH addressed to the MAC entity's C-RNTI has not been received after initial transmission (similar as in CG-SDT)? | Proposal 5: RAN2 to select either way (to close one EN in MAC running CR): whether the UE considers RACH-less LTM failure, if the configuredGrantTimer expires before LTM completion/T304 expiry. |
| LTM completion | “RAN2 assumes For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data” Does the agreement includes the DL assignment for “new transmission”? This is already implemented in the endorsed running CR. | Proposal 6: RAN2 agree that it can be either DL assignment or UL grant for the “new transmission”, in the last meeting agreement: “For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data.” |
|  | “FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE” Any further issue to address for LTM completion and FFS LTE MAC CE is always include? | Proposal 7: Not to revert the agreement, i.e. the LTE MAC CE is not introduced/not always included. |
| MAC reset | How to capture the trigger of MAC reset in RACH-less LTM? This is already implemented in the endorsed running CR. | Proposal 8a: In RACH-less LTM, the MAC reset operation is performed before applying the TA value of target cell; Proposal 8b: RAN2 to select one option: - Option 1: LTM MAC reset is triggered by MAC layer; - Option 2: LTM MAC reset is triggered by RRC layer (Reconfiguration with sync) and MAC layer applies the TA value only after MAC reset operation. The updates is made considering the Tdoc: R2-2310099 (ZTE), R2-2310889 (Ericsson). |

1.3 LTM MAC CE format

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| CFRA information | FFS on the detailed CFRA resource related information in LTM MAC CE. | Proposal 9: As to the CFRA resource related information in LTM MAC CE, RAN2 to select one option: - Option 1 [suggested]: The information similar to those in the legacy PDCCH order triggered RACH, including preamble index, UL/SUL indicator, SSB index PRACH Mask index, and FFS on the Msg1 repetition number; - Option 2: one bit indication about the CFRA resource availability; - Option 3: One index pointing to one of the CFRA configuration in the RRC configured list. The update is made based on Tdoc: R2-2309546 (CATT), R2-2309721 (vivo), R2-2309769 (Samsung), R2-2310099 (ZTE), R2-2310373 (OPPO), R2-2310581 (Xiaomi), R2-2310889 (Ericsson), R2-2311094 (Qualcomm) |
|  | FFS CFRA resource related information in LTM MAC CE is supported in inter-DU. | Proposal 10: CFRA resource related information in LTM MAC CE is limited to only intra-DU case. |
| TA value/indication | “When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this. ” FFS on whether and how is the UE aware of the “same TA value as the source” case? | Proposal 11: As for providing the TA for “same TA value as source” case, RAN2 to select either option: - Option 1: Implicit way by directly providing the TA value; - Option 2: Explicit flag (FFS 1 bit or TAG ID or specific code-point of TA value). The update is made based on the Tdoc: R2-2309546 (CATT), R2-2309721 (vivo), R2-2310338 (Apple), R2-2310373 (OPPO), R2-2310392 (MTK), R2-2310580(Xiaomi), R2-2310889(Ericsson), R2-2310804 (InterDigital), R2-2311094 (Qualcomm) |
|  | The presence of TA value field, by considering the “same as source” and “UE based TA measurement” cases. | TBD, after RAN2 concludes other issues. |
| TCI state ID | Whether the TCI state ID should be provided in RACH-less LTM? | Proposal 12: The presence of TCI state field and TA value field are decoupled in the LTM cell switch MAC CE. Proposal 13: In RACH-less LTM, TCI state field should be provided in the LTM cell switch MAC CE, i.e. UE uses the beam indicated by the NW in RACH-less LTM. |
|  | Whether the TCI state ID should be provided in RACH-based LTM? If yes, FFS on the UE behaviours on the SSB used for RO selection of RACH (CBRA and CFRA case)? | Proposal 14a: In case TCI state field is included in RACH-based LTM, if further agreed by RAN1 (e.g. to activate the TCI state), RAN2 to discuss whether this is the SSB used for RO selection during RACH： Options could be considered for CBRA: - Option 1a: UE performs SSB selection for RACH based on RSRP as legacy; - Option 2a: UE uses the SSB (indicated by TCI state field in MAC CE) for RACH. Options could be considered for CFRA: - Option 1b: UE performs SSB selection for RACH based on RSRP as legacy; - Option 2b: UE uses the indicated SSB for RACH (FFS the one indicated by TCI state field or the SSB index if agreed as CFRA related information in MAC CE). |

- Stage-3/CR implementation open issue list

Following are the minor CR implementation open issue list before RAN2#123-bis meeting.

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| Cross-layer inter-action | Editor’s note: FFS on the need of RRC layer indication on the time when the target configuration has been applied, so that MAC layer can start to process the later fields e.g. TA/TCI state. FFS on the need of indication to RRC layer that those fields has been applied in MAC, so that RRC can submit the RRCReconfigurationComplete message for transmission. | See the running CR update. Note: rapporteur understands that the operation order should be: - MAC receives the LTM MAC CE triggers LTM cell switch and deliver the Target Configuration ID to RRC; - RRC applies the target RRC configuration (e.g. RRC performs the Reconfiguration with sync (first reset MAC and then applies spCellConfigCommon etc.); - MAC layer applies the TA value and TCI state fields, if needed; - RRC submits the RRCReconfigurationComplete message to lower layers; - RACH or RACH-less MAC operations; - RRC layer complete the LTM and stop T304.Also see the update on “MAC reset” issue. |
|  | Editor’s note: FFS on the need of MAC indicating to RRC to trigger/skip the Random Access procedure for this LTM cell switch (since RA will not be triggered if the TA is valid when sending the RRCReconfigurationComplete message). | Proposal 15: MAC layer does not indicate RRC layer to trigger/skip RACH upon receiving the LTM cell switch command MAC CE. (to close one EN in MAC running CR) The update is made based on Tdoc: R2-2309546 (CATT), R2-2309721 (vivo), R2-2310099 (ZTE), R2-2310889 (Ericsson). Even though the RRC running CR has following description, the skip RACH indication” is not needed, since RRC only cares about the complete indication (either RACH completion or RACH-less completion.) 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or, 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and the RRCReconfiguration message is applied due to an LTM cell switch execution without performing a Random Access procedure and upon an indication from lower layer that the LTM cell switch execution has been successfully completed:” |
| Minor clarification | Editor’s note: The information other than Target Configuration ID in this MAC CE (e.g. Timing Advance Command, the TCI state ID) refers to the (target) RRC configuration after the UE has applied the candidate configuration indicated by Target Configuration ID. It does not refer to the (source) RRC configuration in use upon reception of this MAC CE. FFS how to capture this clarification on UE behaviours. | See the running CR update. |

- Conclusion

In this paper, rapporteur propose following proposals for the MAC open issues:

Early TA acquisition

Power ramping

Proposal 1: RAN2 confirm that power ramping counter is neither reset nor increased, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order.

UE based TA measurement

Proposal 2: If UE is configured by RRC to perform UE based TA measurement, UE applies the measured TA value and performs RACH-less LTM, upon LTM cell switch.

RACH-less cell switch

CG for RACH-less LTM

Proposal 3: No specification impact/restriction on the UE to use both DG and CG for RACH-less LTM.

Proposal 4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion (to close one EN in MAC running CR):

- Option 1: UE skips/stops using those CG;

- Option 2: UE can continue use those CG. FFS UE can use any CG occasion (no need to further consider the SSB and CG occasion association).

Proposal 5: RAN2 to select either way (to close one EN in MAC running CR): whether the UE considers RACH-less LTM failure, if the configuredGrantTimer expires before LTM completion/T304 expiry.

LTM completion

Proposal 6: RAN2 agree that it can be either DL assignment or UL grant for the “new transmission”, in the last meeting agreement: “For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data.”

Proposal 7: Not to revert the agreement, i.e. the LTE MAC CE is not introduced/not always included.

MAC reset

Proposal 8a: In RACH-less LTM, the MAC reset operation is performed before applying the TA value of target cell.

Proposal 8b: RAN2 to select one option:

- Option 1: LTM MAC reset is triggered by MAC layer;

- Option 2: LTM MAC reset is triggered by RRC layer (Reconfiguration with sync) and MAC layer applies the TA value only after MAC reset operation.

LTM MAC CE format

CFRA information

Proposal 9: As to the CFRA resource related information in LTM MAC CE, RAN2 to select one option:

- Option 1 [suggested]: The information similar to those in the legacy PDCCH order triggered RACH, including preamble index, UL/SUL indicator, SSB index PRACH Mask index, and FFS on the Msg1 repetition number;

- Option 2: one bit indication about the CFRA resource availability;

- Option 3: One index pointing to one of the CFRA configuration in the RRC configured list.

Proposal 10: CFRA resource related information in LTM MAC CE is limited to only intra-DU case.

TA value/indication

Proposal 11: As for providing the TA for “same TA value as source” case, RAN2 to select either option:

- Option 1: Implicit way by directly providing the TA value;

- Option 2: Explicit flag (FFS: 1 bit or TAG ID or specific code-point of TA value).

TCI state ID

Proposal 12: The presence of TCI state field and TA value field are decoupled in the LTM cell switch MAC CE.

Proposal 13: In RACH-less LTM, TCI state field should be provided in the LTM cell switch MAC CE, i.e. UE uses the beam indicated by the NW in RACH-less LTM.

Proposal 14a: In case TCI state field is included in RACH-based LTM, if further agreed by RAN1 (e.g. to activate the TCI state), RAN2 to discuss whether this is the SSB used for RO selection during RACH：

Options could be considered for CBRA:

- Option 1a: UE performs SSB selection for RACH based on RSRP as legacy;

- Option 2a: UE uses the SSB (indicated by TCI state field in MAC CE) for RACH.

Options could be considered for CFRA:

- Option 1b: UE performs SSB selection for RACH based on RSRP as legacy;

- Option 2b: UE uses the indicated SSB for RACH (FFS the one indicated by TCI state field or the SSB index if agreed as CFRA related information in MAC CE).

Cross-layer indication

Proposal 15: MAC layer does not indicate RRC layer to trigger/skip RACH upon receiving the LTM cell switch command MAC CE. (to close one EN in MAC running CR)

Early TA acquisition

LBT

Proposal 16: UE transmits the preamble without the power ramping upon reception of PDCCH order with retransmission indication if prior preamble transmission encounters LBT failure.

- Reference

- R2-2309546	Discussion on L2 Centric Parts	CATT	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2309721	Contents of LTM MAC CE and other MAC related issue for LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2309769	Cell Switching – Open Issues	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2309786	Support UE based TA determination and RACH-less LTM	Futurewei	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2309851	Support of UE-based TA acquisition for LTM	Samsung	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310099	Some views On Remaining L2 centric issues for LTM	ZTE Corporation, Sanechips	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310338	On closing L2 centric open issues in LTM	Apple	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310373	Discussion on L2-centric issues for LTM	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310392	RACH-Less LTM	MediaTek Inc.	discussion

- R2-2310580	RACH-less LTM and early TA	Xiaomi	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310581	Remaining issues for RACH-based LTM	Xiaomi	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310804	TA indication	Interdigital, Inc.	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2310889	Remaining MAC issues	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

- R2-2311093	Discussion on early TA acquisition and RACH-less LTM	Qualcomm Incorporated	discussion