3GPP TSG-RAN WG2 Meeting #123bis	R2-2311322

# Xiamen, China,  9th – 13th October 2023

Title:	LS on UE Location Information for NB-IoT NTN

Release:	Rel-18

Work Item:	IoT_NTN_enh

Source:	RAN2

To:	CT1, SA2

Cc:	RAN3

Contact person:	Luca Lodigiani

luca.lodigiani@inmarsat.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

A number of satellite operators as well as other members of the NTN community have reinstated in R2-2310192 the relevance and urgency of supporting a suitable UE coarse location reporting method for early NB-IoT deployments.

RAN2 discussed whether and how location reporting for an NB-IoT UE in NTN can be supported.

A severe issue is identified with the current lack of a viable solution for NB-IoT NTN UE to efficiently report their location (even coarse-grained) to the network, which has severe impact in at least the following three areas, which affect fundamental system operation, in some cases preventing the system to work at all:

- Basic Regulatory Compliance

- Efficient Radio Resource Management and Cell Mapping

- Efficient Mobility Management

Feedback from NTN operators is that current mechanisms of location reporting that rely on LPP cannot be relied upon, and a mechanism is required as early as possible during the initial access procedure, otherwise the service may not be provided.

The original proposal calls for an AS-based solution to support reporting of Coarse UE location, to the granularity of a terrestrial cell, with the understanding that, as previously noted by RAN2 in Release 17, for NB-IoT UEs, AS layer security cannot be assumed.

Some companies in RAN2 have the understanding that reporting of UE location to the granularity of a terrestrial cell does not pose any additional privacy or security concerns compared to existing TN specification and deployments.

Other companies in RAN2 still have concerns with reporting the coarse UE location via AS layer with no security context. RAN2 also discussed the possibility to provide location information via NAS layer as an alternative to delivering the location information via AS layer. At least for the moment, a NAS layer-based solution seems promising.

RAN2 would like to ask SA2 and CT1 to consider the following:

  - Study and specify a NAS layer based solution for an NB-IoT UE in NTN to report its location to the network via NAS message:

    - NAS security activation is assumed

    - UE location report should be provided as early as possible during the NAS procedures

    - The mechanism should be applicable for NB-IoT with CP solution for both EDT and non-EDT

    - A mechanism should also be specified for the network to efficiently request a new UE location report to all or groups of UEs in a cell or cell group

    - FFS whether a mechanism needs to be considered for periodic reporting or when the UE location changes, if enabled by the network.

# 2	Actions

To CT1

ACTION: 	RAN2 asks SA2 and CT1 to take the above into account, with the objective to study and specify a NAS layer based solution for an NB-IoT UE in NTN to report its location to the network as early as possible during the initial registration procedure.

# 3	Dates of next TSG RAN WG2 meetings

RAN2#124	13th – 17th November 2023	F2F meeting

RAN2#125	26th February – 1st March 2024 	F2F meeting