3GPP TSG-RAN WG2 #123	R2-2311394

Xiamen, P.R. China Oct 9tht – 13th, 2023

Agenda Item:	7.24.2

Source:	Ericsson

Title:	[AT123bis][431][TEI18] Bluetooth AoA/AoD positioning (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This document is a brief summary of the F2F offline discussion:

[AT123bis][431][TEI18] Bluetooth AoA/AoD positioning (Ericsson)

Scope: Discuss the details of Bluetooth AoA/AoD and how much device control can be captured in RAN2 specs.

Intended outcome: Report to Thursday CB session in R2-2311394

Deadline: Thursday 2023-10-12 1300 CST

Schedule: Thursday 2023-10-12 1100-1130, in Brk3 (Ericsson to confirm availability with secretary)

The discuss started from the baseline agreement

Agreements:

Support Bluetooth AoA/AoD positioning in the LPP Bluetooth positioning method.

Introduce BT AoA/AoD measurements in LPP.

FFS if further BT control information can be captured in our specs.

The following discussion template was used to describe the involved parts of the procedures for the agreed measurements, and what extensions would be needed to address “AoA control”

- Capability handling - UE can confirm support for AoA and/or AoD - Ext: specific capability for AoA control incl CTE config control - Assistance Data handling - UE to be provided with AoD information, antenna etc - Location Information - AoD: UE to be configured to report AoD measurements - AoA: UE to be requested to provide advertising address, periodicity and CTE config to enable Bluetooth beacon measurements - Ext: RequestLocInfo to include suggested AoA CTE config, UE to provide AOA+CTE config to be used AoA and CTE config includes Bluetooth advertisement address, primary channel advertisement interval and secondary channel periodic advertising interval, TX power and CTE configuration information of CTE length, CTE count and PHY type. AoD config includes Bluetooth beacon location information, beacon antenna array and antenna switching pattern configuration, and beacon advertising and CTE configurations.

The summary of the discussion was as follows:

- The “control” parts were not really discussed – time was spent on understanding the role and need for assistance data and the overall functionality – there is a need for stage-2 text proposals.

- One discussion point concerned whether assistance data is needed for AoD (QC, Huawei, Intel) – it was not needed in Rel 13, and in NR the power per beam is reported – the antenna details is known only in LMF. The difference is that Bluetooth AoD is based on switched antenna transmissions from Bluetooth beacon antenna elements forming an array, and the device uses antenna array description of element placements and phase measurements per antenna element to estimate AoD. The resulting AoD angles are used for reporting to LMF (UEA) or for device navigation (UEB).

- Another question was if this already cannot be handled by Bluetooth (QC) – the response was that LPP would bring alignment and compatibility to ensure multi-vendor as stressed by AT&T and u-blox.

- How is LMF and Bluetooth beacons configured? Are there aspects outside RAN2 to consider (Intel)? Are there means to configure RX and TX of the Bluetooth beacon also (Sony)? – a stage-2 text should be intended to address these, while the TEI18 is only for RAN2 so nothing addressed outside RAN2 at this point.

Draft CR is provided in Appendix based on [3] with some noted corrections and also markings to indicate the parts related to the “control” option.

Stage-2 text proposals are needed to explain overall structure and functionality of AoA/AoD addition to Bluetooth positioning method.

Draft CR and “control” option needs to further discussed

Discuss draft CR and “control” in a post meeting email discussion.

# Conclusion

Based on the discussion in the previous sections we make the following observations:

- Stage-2 text proposals are needed to explain overall structure and functionality of AoA/AoD addition to Bluetooth positioning method.

Draft CR and “control” option needs to further discussed

and propose the following:

Proposal 1	Discuss draft CR and “control” in a post meeting email discussion.

# References

Bluetooth Core Specification 5.1

Bluetooth Direction Finding: A Technical Overview, Bluetooth SIG v1.0.3 Oct 2021. Available at https://www.bluetooth.com/bluetooth-resources/bluetooth-direction-finding/

R2-2310853, Adding support for Bluetooth AoA/AoD, Ericsson, AT&T, Polaris Wireless, u-blox, T-Mobile

# Appendix Text Proposal for 37.355 V17.6.0

# 2	References

[…]

[xx]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.4", February 2023.

[…]

## 6.3	Message Body IEs

[…]

#### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

[[	sensor-RequestAssistanceData-r14

Sensor-RequestAssistanceData-r14	OPTIONAL,

tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

]],

[[	nr-Multi-RTT-RequestAssistanceData-r16	NR-Multi-RTT-RequestAssistanceData-r16	OPTIONAL,

nr-DL-AoD-RequestAssistanceData-r16		NR-DL-AoD-RequestAssistanceData-r16		OPTIONAL,

nr-DL-TDOA-RequestAssistanceData-r16	NR-DL-TDOA-RequestAssistanceData-r16	OPTIONAL

]],

[[

bt-RequestAssistanceData-r18	BT-RequestAssistanceData-r18		OPTIONAL

]]

}

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

]],

[[	nr-Multi-RTT-ProvideAssistanceData-r16

NR-Multi-RTT-ProvideAssistanceData-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-ProvideAssistanceData-r16

NR-DL-AoD-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16

NR-DL-TDOA-ProvideAssistanceData-r16

OPTIONAL	-- Need ON

]],

[[

bt-ProvideAssistanceData-r18		BT-ProvideAssistanceData-r18	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

[…]

#### 6.5.7.1	Bluetooth Location Information

#### –	BT-ProvideLocationInformation

The IE BT-ProvideLocationInformation is used by the target device to provide measurements for one or more Bluetooth beacons to the location server. It may also be used to provide Bluetooth positioning specific error reason or to provide Bluetooth AoA configuration of the target device.

-- ASN1START

BT-ProvideLocationInformation-r13 ::= SEQUENCE {

bt-MeasurementInformation-r13		BT-MeasurementInformation-r13	OPTIONAL,

bt-Error-r13						BT-Error-r13					OPTIONAL,

... ,

[[

bt-AoA-Config-r18			BT-AoA-Config-r18	OPTIONAL,

]]

}

BT-AoA-Config-r18 ::= SEQUENCE {

btAddr-r13				BIT STRING (SIZE (48)),

primaryAdvInterval-r18	INTEGER (32..16777)			OPTIONAL,

secondAdvInterval-r18	INTEGER (6..65535)			OPTIONAL,

txPower-r18				INTEGER (-127..20)			OPTIONAL,

cteLength-r18			INTEGER (1..20)				OPTIONAL,

cteCount-r18			INTEGER (1..16)				OPTIONAL,

tx-PHY-M2-r18			NULL						OPTIONAL,

...

}

-- ASN1STOP

BT- ProvideLocationInformation field descriptions

btAddr This field specifies the Bluetooth address of the Bluetooth beacona [xx]. In case the device updates its address during an established LPP session, the device shall provide the new address as unsolicited location information to the server

primaryAdvInterval This field specifies the Bluetooth primary advertisement channel periodicity that the device is requested to use, scaling factor 0.625 ms [xx].

secondAdvInterval This field specifies the Bluetooth periodic advertising interval on secondary advertisement channels that the device is requested to use, scaling factor 0.625 ms [xx].

txPower This field specifies the Bluetooth advertising TX power in dBm.

cteLength This field specifies the configured CTE length to be used by the device in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that the device will transmit each periodic advertising. If the server has included a requested CTE count in RequestLocationInformation, then the device will use the nearest supported CTE count.

tx-PHY-M2 This field, if present, indicates that Bluetooth TX PHY 2 Megasymbols/s shall be used for AoA, otherwise Bluetooth TX PHY 1 Megasymbols/s shall be used,

#### 6.5.7.2	Bluetooth Location Information Elements

#### –	BT-MeasurementInformation

-- ASN1START

BT-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

bt-MeasurementList-r13				BT-MeasurementList-r13		OPTIONAL,

...,

bt-MeasurementList-r18				BT-MeasurementList-r18		OPTIONAL,

}

BT-MeasurementList-r13 ::= SEQUENCE (SIZE(1..maxBT-Beacon-r13)) OF BT-MeasurementElement-r13

BT-MeasurementElement-r13 ::= SEQUENCE {

btAddr-r13						BIT STRING (SIZE (48)),

rssi-r13						INTEGER (-128..127)				OPTIONAL,

...

}

BT-MeasurementList-r18 ::= SEQUENCE (SIZE(1..maxBT-Beacon-r18)) OF BT-MeasurementElement-r18

BT-MeasurementElement-r18 ::= SEQUENCE {

btAddr-r18						BIT STRING (SIZE (48)),

bt-azimuth-r18					INTEGER (0..359),

bt-elevation-r18				INTEGER (0..180)				OPTIONAL,	-- Need ON

bt-MeasurementPerAntList-r18	BT-MeasurementPerAntList-r18	OPTIONAL,

rssi-r18						INTEGER (-128..127)				OPTIONAL,

...

}

BT-MeasurementPerAntList-r18 ::= SEQUENCE (SIZE(1..maxBT-BeaconAntElt-r18)) OF BT-MeasurementperAntElement-r18

BT-MeasurementPerAntElement-r18 ::= SEQUENCE {

antElementIndexShort-r18		INTEGER (1..16),

antElementIndexOffset-r18		ENUMERATED (o16, o32, o48, o64)	OPTIONAL,

ant-rssi-r18					INTEGER (-128..127)				OPTIONAL,

...

}

-- ASN1STOP

BT-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the Bluetooth measurements are performed and should take the form of YYMMDDhhmmssZ.

bt-MeasurementList This field provides the Bluetooth measurements for up to 32 Bluetooth beacons.

btAddr This field specifies the Bluetooth public address of the Bluetooth beacon [25].

rssi, ant-rssi This field provides the beacon received signal strength indicator (RSSI) in dBm.

bt-aximuth This field represents the estimated AoD azimuth in GCD relative the Bluetooth beacon reference position.

bt-elevation This field represents the estimated AoD elevation in GCD relative the Bluetooth beacon reference position.

#### 6.5.7.3	Bluetooth Location Information Request

#### –	BT-RequestLocationInformation

The IE BT-RequestLocationInformation is used by the location server to request Bluetooth measurements or request/provide AoA configuration from/to a target device.

-- ASN1START

BT-RequestLocationInformation-r13 ::= SEQUENCE {

requestedMeasurements-r13	BIT STRING {

rssi		(0),

aod-r18		(1),

rsrp-r18	(2)} (SIZE(1..8)),

...,

[[

bt-requestedAoA-Config-r18		BIT STRING {

aoa-advConfig   (0),

aoa-cteConfig    (1)} (SIZE(1..8))	OPTIONAL,

bt-providedAoA-Config-r18		BT-ProvidedAoA-Config-r18	OPTIONAL,

]]

}

BT-ProvidedAoA-Config-r18 ::= SEQUENCE {

cteStatus-r18			ENUMERATED {enabled, disabled}	OPTIONAL,

primaryAdvInterval-r18	INTEGER (32..16777)				OPTIONAL,

secondAdvInterval-r18	INTEGER (6..65535)				OPTIONAL,

txPower-r18				INTEGER (-127..20)				OPTIONAL,

cteLength-r18			INTEGER (1..20)					OPTIONAL,

cteCount-r18			INTEGER (1..16)					OPTIONAL,

tx-PHY-M2-r18			NULL							OPTIONAL,

...

}

-- ASN1STOP

BT-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the Bluetooth measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. rssi: Bluetooth beacon signal strength at the target aod: Estimated AoD per Bluetooth beacon ant-rssi: Estimated received signal power per Bluetooth beacon antenna

bt-requestedAoA-Config This field specifies the Bluetooth AoA configuration parameters requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. aoa-advConfig: The Bluetooth advertisement address and periodic intervals of the device aoa-cteConfig: The configured CTE length, count and PHY

cteStatus This field specifies the Bluetooth AoA CTE status of the device, and is used by the location server to enable or disable transmission of the BLE CTE by the device for AoA estimation enabled: The Bluetooth CTE shall be enabled by the device disabled: The Bluetooth CTE shall be disabled by the device

primaryAdvInterval This field specifies the Bluetooth primary advertisement channel periodicity that the device is requested to use, scaling factor 0.625 ms [xx].

secondAdvInterval This field specifies the Bluetooth periodic advertising interval on secondary advertisement channels that the device is requested to use, scaling factor 0.625 ms [xx].

txPower This field specifies the Bluetooth advertising TX power in dBm that the device is requested to use [xx].

cteLength This field specifies the configured CTE length to be used by the device in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that zthe device is requested to transmit each periodic advertising event

tx-PHY-M2 This field, if present, indicates that Bluetooth TX PHY 2 Megasymbols/s shall be used for AoA, otherwise Bluetooth TX PHY 1 Megasymbols/s shall be used,

#### 6.5.7.4	Bluetooth Capability Information

#### –	BT-ProvideCapabilities

The IE BT-ProvideCapabilites is used by the target device to provide its capabilities for Bluetooth positioning to the location server.

-- ASN1START

BT-ProvideCapabilities-r13 ::= SEQUENCE {

bt-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1),

ue-based		(2)}	(SIZE (1..8)),

bt-MeasSupported-r13	BIT STRING {	rssi-r13		(0)}	(SIZE (1..8)),

...,

[[

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL,

periodicalReportingSupported-r14

PositioningModes										OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL

]],

[[

bt-AoA-Capability-r18			BT-AoA-Capability-r18	OPTIONAL, -- Cond AoAsup

bt-AoD-Capability-r18			BT-AoD-Capability-r18	OPTIONAL  -- Cond AoDsup

]]

}

BT-AoA-Capability-r18 ::= SEQUENCE {

bt-AoA-TXadrType-r18			ENUMERATED {public, random-static, random-private-resolvable},

bt-TX-PHY-2M-r18				ENUMERATED {supported}	OPTIONAL,

...

}

BT_AoD-Capability-r18 ::= SEQUENCE {

bt-AoD-slotDuration-r18			ENUMERATED {us1, us2, both},

bt-RX-PHY-2M-r18				ENUMERATED {supported}	OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| AoAsup | The field is mandatory present if BT AoA is supported; otherwise it is not present. |
| AoDsup | The field is mandatory present if BT AoD is supported; otherwise it is not present. |

BT-ProvideCapabilities field descriptions

bt-Modes This field specifies the Bluetooth mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the Bluetooth mode is supported; a zero value means not supported.

bt-MeasSupported This field specifies the Bluetooth measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Bluetooth positioning method is supported by the target device which is reporting of the Bluetooth beacon identity. The following bits are assigned for the indicated measurements. rssi: Bluetooth beacon signal strength at the target device

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform BT measurements.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

bt-AoA-TXadrType This field specifies used TX address type in the advertisements - public – the device Bluetooth MAC address - random-static – a random, static address - random-resolvable – a random address made resolvable for the location server

bt-TX-PHY-2M This field, if present, indicates that the device supports 2M PHY mode with 2 Msymbols/s transmission rate (1 Msymbols/s rate is mandatory)

bt-AoD-slotDuration This field specifies which slot durations that are supported by the device, us1 (1 us), us2 (2 us) or both (both 1 us and 2 us).

bt-RX-PHY-2M This field, if present, indicates that the device supports 2M PHY mode with 2 Msymbols/s reception rate (1 Msymbols/s rate is mandatory)

#### 6.5.7.5	Bluetooth Capability Information Request

#### –	BT-RequestCapabilities

The IE BT-RequestCapabilities is used by the location server to request Bluetooth positioning capabilities from a target device.

-- ASN1START

BT-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.7.6	Bluetooth Assistance Data

#### –	BT-ProvideAssistanceData

The IE BT-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based and UE-assisted AoD positioning. It may also be used to provide Bluetooth positioning specific error reasons.

-- ASN1START

BT-ProvideAssistanceData-r18 ::= SEQUENCE {

bt-DataSet-r18	SEQUENCE (SIZE (1..maxBT-DataSets-r18)) OF BT-DataSet-r18

OPTIONAL,	-- Need ON

bt-Error-r18		BT-Error-r18										OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

BT-ProvideAssistanceData field descriptions

bt-DataSet This field provides data for sets of Bluetooth beacons.

bt-Error This field provides error information and may be included when a Provide Assistance Data is sent in response to a Request Assistance Data.

#### 6.5.7.7	Bluetooth Assistance Data Elements

#### –	BT-DataSet

The IE BT-DataSet is used by the location server to provide Bluetooth beacon information for one set of Bluetooth beacons.

-- ASN1START

BT-DataSet-r18 ::= SEQUENCE {

bt-beacon-List-r18				SEQUENCE (SIZE (1..maxBTbeacon-r18)) OF BT-Beacon-Data-r18,

...

}

BT-Beacon-Data-r18 ::= SEQUENCE {

btAddr-r18							BIT STRING (SIZE (48)),

referencePoint-r18					ReferencePoint-r16,

lcs-GCS-TranslationParameter-r18	LCS-GCS-TranslationParameter-r16,

antElementList-r18					SEQUENCE (SIZE (2..maxBT-BeaconAntElt-r18)) OF

AntElement-r18,

antArrayConfig-r18					AntArrayConfig-r18,

bt-AoD-Data-r18						BT-AoD-Data-r18						OPTIONAL,

antSwitchingPattern-r18 				SEQUENCE (SIZE (2..maxBT-BeaconAntElt-r18)) OF

AntSwitchElement-r18					OPTIONAL,

...

}

AntElement-r18 ::= SEQUENCE {

polarization-r18				ENUMERATED (m45, zero, p45, p90, circ),

...

}

AntArrayConfig-r18 ::= CHOICE {

Bt-ula-Linear-r18				BT-ULA-Linear-r18,

bt-ula-Rectangular-r18			BT-ULA-Rectangular-r18,

bt-ula-Circular-r18				BT-ULA-Circular-r18,

bt-genericArray-r18				BT-GenericArray-r18

}

BT-AoD-Data-r18 ::= SEQUENCE {

primaryAdvInterval-r18	INTEGER (32..16777)		OPTIONAL,

secondAdvInterval-r18	INTEGER (6..65535)		OPTIONAL,

cteLength-r18			INTEGER (1..20)			OPTIONAL,

cteCount-r18			INTEGER (1..16)			OPTIONAL,

tx-PHY-M2-r18			NULL					OPTIONAL,

...

}

AntSwitchElement-r18 ::= SEQUENCE {

antElementIndexShort-r18		INTEGER (1..16),

antElementIndexOffset-r18		ENUMERATED (o16, o32, o48, o64)		OPTIONAL,

...

}

-- ASN1STOP

BT-DataSet field descriptions

btAddr This field specifies the Bluetooth public address of the Bluetooth beacon [xx].

referencePoint This field specifies the reference point used to define the location of a Bluetooth beacon antenna.

lcs-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44].

polarization This field specifies the antenna element polarization in degrees relative the positive y-axis, where m45, zero, p45, p90 represents -45, zero, 45 and 90 degrees respectively towards the z-axis, and circ represents circular polarization.

bt-AoD-Data This field specifies Bluetooth beacon AoD configuration in terms of advertising periodicities and CTE configuration to support the device to configure its scan windows and intervals.

primaryAdvInterval This field specifies the Bluetooth primary advertisement channel periodicity that the device is requested to use, scaling factor 0.625 ms [xx].

secondAdvInterval This field specifies the Bluetooth periodic advertising interval on secondary advertisement channels that the device is requested to use, scaling factor 0.625 ms [xx].

cteLength This field specifies the configured CTE length to be used by the beacon in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that are transmitted each periodic advertising event

tx-PHY-M2 This field, if present, indicates that Bluetooth TX PHY 2 Megasymbols/s is used by the beacon, otherwise Bluetooth TX PHY 1 Megasymbols/s is used,

antSwitchingPattern This field specifies the Bluetooth antenna switching pattern as a list of indices, where each index is the order value of a specific antenna element in the antElementList-r18 attribute of the IE BT-DataSet-r18 – first element in the list corresponds to index 1 and so on. If the antenna switching pattern is shorter than the number of available sample slots, then the antenna switching patterns continues from the beginning of the antSwitchingPattern-r18. If antenna switching pattern is longer than the number of available sample slots, then the elements in antSwitchingPattern-r18 are discarded. If this field is not present, the target device can assume an antenna switching pattern with the configured antenna element in the same order as in the antElementList-r18.

antElementIndexShort This field specifies short part of the antenna element index

antElementIndexOffset This field specifies offsett of the antenna element index, where o16, o32, o48 and o64 respresents 16, 32, 48 and 64 respectively to offset the short part of the antenna element index. If not present, the offset is zero.

#### –	BT-ULA-Linear

The IE BT-ULA-Linear is used by the location server to define a linear antenna array as a formula based on the antenna element index.

-- ASN1START

BT-ULA-Linear-r18 ::= SEQUENCE {

bt-NoElements-r18				INTEGER (2..maxBT-BeaconAntElt-r18),

bt-ElementDelta-r18				FFS

}

-- ASN1STOP

BT-ULA-Linear field descriptions

bt-NoElements This field specifies the number of antenna elements in the linear antenna array. It is the same as the number of antenna elements in the antElementList-r18 of the IE BT-DataSet-r18.

bt-ElementDelta This field specifies the distance between to adjacent elements in the linear antenna array.

The antenna element locations of the antenna array are defined along the y-axis from the reference point. The coordinates of the elements are x=0, z=0 and y = (index-1)*bt-ElementsDelta-r18, where index is the order value of a specific antenna element in the antElementList-r18 attribute of the IE BT-DataSet-r18 – first element in the list corresponds to index 1 and so on.

#### –	BT-ULA-Rectangular

The IE BT-ULA-Rectangular is used by the location server to define a rectangular antenna array as a formula based on the antenna element index.

-- ASN1START

BT-ULA-Rectangular-r18 ::= SEQUENCE {

bt-NoElementsY-r18				INTEGER (1..maxBT-BeaconAntElt-r18),

bt-NoElementsZ-r18				INTEGER (1..maxBT-BeaconAntElt-r18),

bt-ElementDeltaY-r18			FFS

bt-ElementDeltaZ-r18			FFS

}

-- ASN1STOP

BT-ULA-Rectangular field descriptions

bt-NoElementsY This field specifies the number of antenna elements in the L-shaped antenna array along the y-axis. The product bt-ElementDeltaY* bt-ElementDeltaZ is the same as the number of antenna elements in the antElementList-r18 of the IE BT-DataSet-r18.

bt-NoElementsZ This field specifies the number of antenna elements in the linear antenna array along the z-axis.

bt-ElementDeltaY This field specifies the distance between to adjacent elements in the rectangular antenna array along the y-axis.

bt-ElementDeltaZ This field specifies the distance between to adjacent elements in the rectangular antenna array along the z-axis.

The antenna element locations of the rectangular antenna array are defined row by row along the y-axis with an increasing offset in the z-direction for each row from the reference point. The coordinates of the elements of the

- first row are x=0, z=0 and y = (index-1)*bt-ElementsDelta-r18, for index 1 to bt-NoElementsY

- second row are x=0, z = bt-ElementDeltaZ and y = (index-bt-NoElementsY-1)*bt-ElementsDelta-r18, for index (bt-NoElementsY+1) to 2*bt-NoElementsY

- row N are x=0, z = (N-1)*bt-ElementDeltaZ and y = (index-(N-1)*bt-NoElementsY-1)*bt-ElementsDelta-r18, for index ((N-1)*bt-NoElementsY+1) to N*bt-NoElementsY, where N = 1 to NoElementsZ.

#### –	BT-ULA-Circular

The IE BT-ULA-Circular is used by the location server to define a circular antenna array as a formula based on the antenna element index.

-- ASN1START

BT-ULA-Circular-r18 ::= SEQUENCE {

bt-NoElements-r18				INTEGER (2..maxBT-BeaconAntElt-r18),

bt-Radius-r18					FFS

}

-- ASN1STOP

BT-ULA-Circular field descriptions

bt-NoElements This field specifies the number of antenna elements in the circular antenna array. It is the same as the number of antenna elements in the antElementList-r18 of the IE BT-DataSet-r18

bt-Radius This field specifies the radius from the reference point to the antenna elements in the circular antenna array.

The antenna element locations of the antenna array are defined along a circle with the reference point as center. The coordinates of the elements are:

- x=0

- y = bt-Radius-r18* cos(2*p*(index-1)/ bt-NoElements-r18)

- z = bt-Radius-r18* sin(2*p*(index-1)/ bt-NoElements-r18)

where index is the order value of a specific antenna element in the antElementList-r18 attribute of the IE BT-DataSet-r18 – first element in the list corresponds to index 1 and so on.

#### –	BT-GenericArray

The IE BT-GenericArray is used by the location server to define a generic antenna array as a list of offset locations for each antenna element, where each ordered entry in BT-ULA-Generic-r18 is associated to the same ordered entry in the antElementList-r18 attribute of the IE BT-DataSet-r18.

-- ASN1START

BT-ULA-Generic-r18 ::= SEQUENCE (SIZE (2..maxBT-BeaconAntElt-r18)) OF BT-ULA-GenericAntElement-r18

BT-ULA-GenericAntElement-r18 ::= SEQUENCE {

deltaY-r18						INTEGER (-100..100)			OPTIONAL,	-- Need OP

deltaX-r18						INTEGER (-100..100)			OPTIONAL,	-- Need OP

deltaZ-r18						INTEGER (-100..100)			OPTIONAL,	-- Need OP

}

-- ASN1STOP

BT- ULA-Generic field descriptions

deltaX, deltaY, deltaZ This field specifies the antenna element location offset in X, Y, Z directions respectively. Scale factor 1mm.

#### 6.5.7.8	Bluetooth Assistance Data Request

#### –	BT-RequestAssistanceData

The IE BT-RequestAssistanceData is used by the target device to request BT assistance data from a location server.

-- ASN1START

BT-RequestAssistanceData-r18 ::= SEQUENCE {

requestedAD-r18			BIT STRING {	beacon-location		(0),

beacon-antConfig	(1),

beacon-advConfig}	(SIZE (1..8)),

...

}

-- ASN1STOP

BT-RequestAssistanceData field descriptions

requestedAD This field specifies the Bluetooth assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. The following assistance data types are included: - beacon-location: Bluetooth beacon location information - beacon-antConfig: Bluetooth beacon antenna configuration information for AoD estimation - beacon-advConfig: Bluetooth beacon advertisement and CTE configuration information

[…]

## 6.6	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

maxMBS-r14					INTEGER ::= 64

maxWLAN-AP-r13				INTEGER ::= 64

maxKnownAPs-r14				INTEGER ::= 2048

maxVisibleAPs-r14			INTEGER ::= 32

maxWLAN-AP-r14				INTEGER ::= 128

maxWLAN-DataSets-r14		INTEGER ::= 8

maxBT-Beacon-r13			INTEGER ::= 32

maxBT-BeaconAntElt-r18		INTEGER ::= 74

maxBT-DataSets-r18			INTEGER ::= 32

nrMaxBands-r16							INTEGER ::= 1024	-- Maximum number of supported bands in

-- UE capability.

nrMaxFreqLayers-r16						INTEGER ::= 4		-- Max freq layers

nrMaxFreqLayers-1-r16					INTEGER ::= 3

nrMaxNumDL-PRS-ResourcesPerSet-1-r16	INTEGER ::= 63

nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16	INTEGER ::= 7

nrMaxResourceIDs-r16					INTEGER ::= 64		-- Max Resource IDs

nrMaxResourceOffsetValue-1-r16			INTEGER ::= 511

nrMaxResourcesPerSet-r16				INTEGER ::= 64		-- Maximum resources for one set

nrMaxSetsPerTrpPerFreqLayer-r16			INTEGER ::= 2		-- Maximum resource sets for one TRP

nrMaxSetsPerTrpPerFreqLayer-1-r16		INTEGER ::= 1

nrMaxTRPs-r16							INTEGER ::= 256		-- Max TRPs per UE

nrMaxTRPsPerFreq-r16					INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxTRPsPerFreq-1-r16					INTEGER ::= 63

maxSimultaneousBands-r16				INTEGER ::= 4		-- Maximum number of simultaneously

-- measured bands

maxBandComb-r16							INTEGER ::= 1024

nrMaxConfiguredBands-r16				INTEGER ::= 16

maxNumOfRxTEGs-r17						INTEGER ::= 32

maxNumOfRxTEGs-1-r17					INTEGER ::= 31

maxNumOfTxTEGs-1-r17					INTEGER ::= 7

maxTxTEG-Sets-r17						INTEGER ::= 256		-- Maximum applicable number is 64

maxNumOfRxTxTEGs-1-r17					INTEGER ::= 255

maxNumOfTRP-TxTEGs-1-r17				INTEGER ::= 7

maxNumOfSRS-PosResources-r17            INTEGER ::= 64

maxNumOfSRS-PosResources-1-r17          INTEGER ::= 63

maxNumResourcesPerAngle-r17				INTEGER ::= 24

maxNumPrioResources-r17					INTEGER ::= 24

maxAddMeasTDOA-r17						INTEGER ::= 31

maxAddMeasAoD-r17						INTEGER ::= 23

maxAddMeasRTT-r17						INTEGER ::= 31

maxOD-DL-PRS-Configs-r17				INTEGER ::= 8

maxCellIDsPerArea-r17					INTEGER ::= 256

maxNrOfAreas-r17						INTEGER ::= 16

maxMeasInstances-r17					INTEGER ::= 32

-- ASN1STOP