3GPP TSG-RAN WG2 Meeting #123bis	R2-2311398

Xiamen, CN, Oct 9 - 13, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | draft | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LPP running CR for bandwidth aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2023-09-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of bandwidth aggregation based on RAN1 agreement and RAN1 parameter list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction support for bandwidth aggregation with the following changes: - Introduce finer report granularity within the measurement report for DL-TDOA and multi-RTT; - Introduce support of finer report granularity for DL-TDOA and multi-RTT; - Introduce support of NW pre-configured available on-demand PRS bandwidth aggregation configurations; - Introduce support of UE initiated on-demand PRS bandwidth aggregation configurations; - Introduce request of joint measurement based on aggregated PRS for DL-TDOA and multi-RTT; - Introduce indication of the two or three PFLs used for joint measurement for DL-TDOA and multi-RTT; - Introduce indication of the PRS resources sets across the two or three PFLs which can be used for joint measurement for DL-TDOA and multi-RTT; - Introduce indications of PRS resources sets used for joint measurements within the measurement report for DL-TDOA and multi-RTT; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bandwidth aggregation is not supported in NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3, 6.5.10.4, 6.5.10.5, 6.5.10.6, 6.5.12.4, 6.5.12.5, 6.5.12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

6	Information Element Abstract Syntax Definition

6.4.3	Common NR Positioning Information Elements

#### –	NR-AdditionalPathList

The IE NR-AdditionalPathList is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-RelativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-PathQuality.

-- ASN1START

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-AdditionalPathListExt-r17 ::= SEQUENCE (SIZE(1..8)) OF NR-AdditionalPath-r16

NR-AdditionalPath-r16 ::= SEQUENCE {

nr-RelativeTimeDifference-r16	CHOICE {

k0-r16					INTEGER(0..16351),

k1-r16					INTEGER(0..8176),

k2-r16					INTEGER(0..4088),

k3-r16					INTEGER(0..2044),

k4-r16					INTEGER(0..1022),

k5-r16					INTEGER(0..511),

...,

k-1-r18					INTEGER(0..32701),

k-2-r18					INTEGER(0..65401)

},

nr-PathQuality-r16				NR-TimingQuality-r16					OPTIONAL,

...,

[[

nr-DL-PRS-RSRPP-r17				INTEGER (0..126)						OPTIONAL

]]

}

-- ASN1STOP

NR-AdditionalPathList field descriptions

nr-RelativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing of the reference resource. The mapping of reported values and measured quantity value is defined in TS 38.133 [46] clause 10.1.23.3.3 and 10.1.25.3.3. A positive value indicates that the particular path is later in time than the detected path of the reference; a negative value indicates that the particular path is earlier in time than the detected path of the reference.

nr-PathQuality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

nr-DL-PRS-RSRPP This field specifies the DL PRS reference signal received path power (DL PRS-RSRPP) of the NR-AdditionalPath reported, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

NEXT CHANGE

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. the serving TRP.

NOTE 2:	The nr-DL-PRS-ReferenceInfo defines the "assistance data reference" TRP whose DL-PRS configuration is included in nr-DL-PRS-AssistanceDataList. The nr-DL-PRS-SFN0-Offset's and nr-DL-PRS-expectedRSTD's in nr-DL-PRS-AssistanceDataList are provided relative to the "assistance data reference" TRP.

NOTE 3:	The network signals a value of zero for the nr-DL-PRS-SFN0-Offset, nr-DL-PRS-expectedRSTD, and nr-DL-PRS-expectedRSTD-uncertainty of the "assistance data reference" TRP in nr-DL-PRS-AssistanceDataList.

NOTE 4:	For NR DL-TDOA positioning (see clause 6.5.10) the nr-DL-PRS-ReferenceInfo defines also the requested "RSTD reference".

For DL-PRS processing, the LPP layer may inform lower layers to start performing DL-PRS measurements and provide to lower layers the information about the location of DL-PRS, e.g. DL-PRS-PointA, DL-PRS Positioning occasion information.

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16			DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...,

[[

nr-DL-PRS-AggregationInfo-r18		NR-DL-PRS-AggregationInfo-r18		OPTIONAL	-- Need ON

]]

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...,

[[

prs-OnlyTP-r16				ENUMERATED { true }		OPTIONAL	-- Need ON

]],

[[

nr-DL-PRS-ExpectedAoD-or-AoA-r17

NR-DL-PRS-ExpectedAoD-or-AoA-r17	OPTIONAL	-- Need ON

]]

}

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16	INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...

}

NR-DL-PRS-ExpectedAoD-or-AoA-r17 ::= CHOICE {

expectedAoD-r17			SEQUENCE {

expectedDL-AzimuthAoD-r17		INTEGER (0..359),

expectedDL-AzimuthAoD-Unc-r17	INTEGER (0..60)	OPTIONAL, -- Need OP

expectedDL-ZenithAoD-r17		INTEGER (0..180),

expectedDL-ZenithAoD-Unc-r17	INTEGER	(0..30)	OPTIONAL  -- Need OP

},

expectedAoA-r17			SEQUENCE {

expectedDL-AzimuthAoA-r17		INTEGER (0..359),

expectedDL-AzimuthAoA-Unc-r17	INTEGER (0..60)	OPTIONAL, -- Need OP

expectedDL-ZenithAoA-r17		INTEGER (0..180),

expectedDL-ZenithAoA-Unc-r17	INTEGER	(0..30)	OPTIONAL  -- Need OP

}

}

NR-DL-PRS-AggregationInfo-r18 ::= SEQUENCE (SIZE (1..256) OF

NR-DL-PRS-AggregationIndexList-r18

NR-DL-PRS-AggregationIndexList-r18 ::= SEQUENCE (SIZE (2..3) OF NR-DL-PRS-AggregationElement-r18

NR-DL-PRS-AggregationElement-r18 ::= SEQUENCE {

nr-DL-PRS-FrequencyLayerIndex-r18	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-DL-PRS-TRP-Index-r18				INTEGER (0..nrMaxTRPsPerFreq-1-r16),

nr-DL-PRS-ResourceSetIndex-r18		INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16)

}

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.

nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.

nr-SSB-Config This field specifies the SSB configuration of the TRPs.

nr-DL-PRS-AggregationInfo This field specifies the DL-PRS Resource Sets across DL-PRS Positioning Frequency Layers available for DL-PRS bandwidth aggregation. The 2 or 3 DL-PRS Resource Sets indicated by IE NR-DL-PRS-AggregationIndexList are linked for bandwidth aggregation. - nr-DL-PRS-FrequencyLayerIndex: This field indicates the frequency layer provided in nr-DL-PRS-AssistanceDataList. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on. - nr-DL-PRS-TRP-Index: This field indicates the TRP/DL-PRS ID provided in nr-DL-PRS-AssistanceDataPerFreq. Value 0 corresponds to the first TRP/DL-PRS ID provided in nr-DL-PRS-AssistanceDataPerFreq, value 1 to the second TRP/DL-PRS ID in nr-DL-PRS-AssistanceDataPerFreq, and so on. - nr-DL-PRS-ResourceSetIndex: This field indicates the DL-PRS Resource Set in nr-DL-PRS-ResourceSetList in IE NR-DL-PRS-Info. Value 0 corresponds to the first DL-PRS Resource Set provided in nr-DL-PRS-ResourceSetList, value 1 to the second DL-PRS Resource Set in nr-DL-PRS-ResourceSetList. NOTE: The linked DL-PRS Resource Sets from two or three Positioning Frequency Layers are from the same TRP.

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.

nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

nr-PhysCellID This field specifies the physical cell identity of the TRP. When the field prs-OnlyTP is included, this field is not included.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, as defined in TS 38.331 [35]. When the field prs-OnlyTP is included, this field is not included.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. When the field prs-OnlyTP is included, this field is not included.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset corresponds to the number of full subframes counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP. NOTE: The location server sets the value in accordance with the defined search window for the target device using nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty.

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

nr-DL-PRS-Info This field specifies the PRS configuration of the TRP.

dl-PRS-SubcarrierSpacing This field specifies the subcarrier spacing of the DL-PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2. All DL-PRS Resources and DL-PRS Resource Sets in the same Positioning Frequency layer have the same value of dl-PRS-SubcarrierSpacing.

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-StartPRB.

dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.

dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N.

dl-PRS-CyclicPrefix This field specifies the Cyclic Prefix length of the DL-PRS Resource. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-CyclicPrefix.

prs-OnlyTP This field, if present, indicates that the NR-DL-PRS-AssistanceData is provided for a PRS-only TP. Whether the field is present or absent should be the same for all the NR-DL-PRS-AssistanceData of all the PRS transmitted under the same TP. The target device shall not assume that any other signals or physical channels are present for the TRP other than DL-PRS.

nr-DL-PRS-ExpectedAoD-or-AoA This field specifies the expected AoD or AoA in the Global Coordinate System (GCS) at the target device location together with uncertainty. - expectedDL-AzimuthAoD: This field specifies the expected azimuth angle of departure. Scale factor 1 degree; range 0 to 359 degrees. - expectedDL-AzimuthAoD-Unc: This field specifies the (single-sided) uncertainty of the expected azimuth angle of departure. If this field is absent, it indicates maximum uncertainty (60 degrees). Scale factor 1 degree; range 0 to 60 degrees. - expectedDL-ZenithAoD: This field specifies the expected elevation angle of departure. Scale factor 1 degree; range 0 to 180 degrees. - expectedDL-ZenithAoD-Unc: This field specifies the (single-sided) uncertainty of the expected elevation angle of departure. If this field is absent, it indicates maximum uncertainty (30 degrees). Scale factor 1 degree; range 0 to 30 degrees. - expectedDL-AzimuthAoA: This field specifies the expected azimuth angle of arrival. Scale factor 1 degree; range 0 to 359 degrees. - expectedDL-AzimuthAoA-Unc: This field specifies the (single-sided) uncertainty of the expected azimuth angle of arrival. If this field is absent, it indicates maximum uncertainty (60 degrees). Scale factor 1 degree; range 0 to 60 degrees. - expectedDL-ZenithAoA: This field specifies the expected elevation angle of arrival. Scale factor 1 degree; range 0 to 180 degrees. - expectedDL-ZenithAoA-Unc: This field specifies the (single-sided) uncertainty of the expected elevation angle of arrival. If this field is absent, it indicates maximum uncertainty (30 degrees). Scale factor 1 degree; range 0 to 30 degrees.

#### –	NR-DL-PRS-ResourceSetID-BWAggregation

The IE NR-DL-PRS-ResourceSetID-BWAggregation is used by the target device to indicate the PRS resource set IDs for the aggregated measurement which are used for measurements.

-- ASN1START

NR-DL-PRS-ResourceSetID-BWAggregation-r18  ::= SEQUENCE {

nr-DL-PRS-ResourceSetList-r18		SEQUENCE (SIZE (1.. nrMaxNumDL-PRS-ResourceSetsPerTRP-r18)) OF

NR-DL-PRS-ResourceSetID-r16

}

-- ASN1STOP

NEXT CHANGE

#### –	NR-On-Demand-DL-PRS-Configurations

The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations and/or PRS bandwidth aggregation which can be requested by the target device on-demand.

-- ASN1START

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE {

on-demand-dl-prs-configuration-list-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Configuration-r17,

... ,

[[

on-demand-dl-prs-aggregation-list-r18		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Aggregation-Info-r18

OPTIONAL

]]

}

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE {

dl-prs-configuration-id-r17					DL-PRS-Configuration-ID-r17,

nr-DL-PRS-PositioningFrequencyLayer-r17		NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-Info-r17							NR-DL-PRS-Info-r16,

...

}

DL-PRS-Configuration-ID-r17 ::= SEQUENCE {

nr-dl-prs-configuration-id-r17				INTEGER (1..maxOD-DL-PRS-Configs-r17),

...

}

On-Demand-DL-PRS-Aggregation-Info-r18 ::= SEQUENCE (SIZE (2..3)) OF DL-PRS-Configuration-ID-r17

-- ASN1STOP

NR-On-Demand-DL-PRS-Configurations field descriptions

dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.

nr-DL-PRS-PositioningFrequencyLayer This field, together with nr-DL-PRS-Info, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-PositioningFrequencyLayer are applicable: dl-PRS-ResourceBandwidth, dl-PRS-CombSizeN. The target device shall ignore the remaining fields in IE NR-DL-PRS-PositioningFrequencyLayer.

nr-DL-PRS-Info This field, together with nr-DL-PRS-PositioningFrequencyLayer, provides the On-demand DL-PRS Configuration information. Only the following fields in IE NR-DL-PRS-Info are applicable: DL-PRS periodicity in dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols, comb-size in dl-PRS-CombSizeN-AndReOffset, dl-PRS-QCL-Info. The target device shall ignore the remaining fields in IE NR-DL-PRS-Info.

on-demand-dl-prs-aggregation-list This field indicates the 2 or 3 DL-PRS-Configuration-ID's whose corresponding On-Demand-DL-PRS-Configuration's are available for DL-PRS aggregation.

NEXT CHANGE

#### –	NR-On-Demand-DL-PRS-Request

The IE NR-On-Demand-DL-PRS-Request is used by the target device to request on-demand DL-PRS and/or on-demand PRS bandwidth aggregation from a location server.

-- ASN1START

NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE {

dl-prs-StartTime-and-Duration-r17			DL-PRS-StartTime-and-Duration-r17		OPTIONAL,

nr-on-demand-DL-PRS-Information-r17			NR-On-Demand-DL-PRS-Information-r17		OPTIONAL,

dl-prs-configuration-id-PrefList-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF 
													DL-PRS-Configuration-ID-r17			OPTIONAL,

... ,

[[

dl-prs-aggregation-id-PrefList-r18			SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

INTEGER (1..maxOD-DL-PRS-Configs-r17)	OPTIONAL,

nr-on-demand-DL-PRS-Aggregation-ReqList-r18	SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF

NR-On-Demand-DL-PRS-Aggregation-ReqElement

OPTIONAL

]]

}

DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE {

dl-prs-start-time-r17		INTEGER (1..1024)						OPTIONAL,

dl-prs-duration-r17			SEQUENCE {

seconds-r17		INTEGER (0..59)		OPTIONAL,

minutes-r17		INTEGER (0..59)		OPTIONAL,

hours-r17		INTEGER (0..23)		OPTIONAL,

...

}										OPTIONAL,

...

}

NR-On-Demand-DL-PRS-Aggregation-ReqElement ::= SEQUENCE (SIZE (2..3)) OF 
												INTEGER (1..nrMaxFreqLayers-r16)		OPTIONAL

-- ASN1STOP

NR-On-Demand-DL-PRS-Request field descriptions

dl-prs-StartTime-and-Duration This field specifies the requested start time and duration for the on-demand DL-PRS and comprises the following subfields: - dl-prs-start-time specifies the desired start time for the requested DL-PRS. It indicates the time in seconds from the time the IE NR-On-Demand-DL-PRS-Request was received. - dl-prs-duration specifies the desired duration of the requested DL-PRS. The desired duration is the sum of the seconds, minutes, hours fields. If this field is included, at least one of the seconds, minutes, hours fields shall be present.

nr-on-demand-DL-PRS-Information This field specifies the on-demand DL-PRS configuration information requested by the target device. NOTE: If the network provided predefined on-demand DL-PRS configurations (NR-On-Demand-DL-PRS-Configurations), the target device can only request explicit parameters (nr-on-demand-DL-PRS-Information) within the scope of those configurations.

dl-prs-configuration-id-PrefList This field specifies the on-demand DL-PRS configuration associated with DL-PRS-Configuration-ID in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first DL-PRS-Configuration-ID in the list is the most preferred configuration, the second DL-PRS-Configuration-ID the second most preferred, etc.

dl-prs-aggregation-id-PrefList This field specifies the on-demand DL-PRS aggregated configuration associated with on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first integer value in the list is the most preferred aggregated configuration, the second integer value in the list is the second most preferred, etc. The integer value corresponds to the entry in the field on-demand-dl-prs-aggregation-list in IE NR-On-Demand-DL-PRS-Configurations.

nr-on-demand-DL-PRS-Aggregation-ReqList This field specifies the aggregated on-demand DL-PRS configuration information requested by the target device in the order of preference. The first NR-On-Demand-DL-PRS-Aggregation-ReqElement in the list is the most preferred aggregated configuration, the second element in the list is the second most preferred, etc. The integer value in NR-On-Demand-DL-PRS-Aggregation-ReqElement corresponds to the entry in the IE NR-On-Demand-DL-PRS-Information.

NEXT CHANGE

#### 6.5.10.4	NR DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

NOTE 2:	The "RSTD reference" TRP may or may not be the same as the "assistance data reference" TRP provided by nr-DL-PRS-ReferenceInfo in IE NR-DL-PRS-AssistanceData.

NOTE 3:	The target device includes a value of zero for the nr-RSTD and nr-RSTD-ResultDiff of the "RSTD reference" TRP in nr-DL-TDOA-MeasList.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]],

[[

nr-dl-PRS-Reference-BWAggregation-r18 NR-DL-PRS-ResourceSetID-BWAggregation-r18   OPTIONAL

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...,

k-1-r18						INTEGER (0..3940097),

k-2-r18						INTEGER (0..7880193)

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-DL-PRS-ResourceSetID-BWAggregation-r18 NR-DL-PRS-ResourceSetID-BWAggregation-r18   OPTIONAL

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...,

k-1-r18						INTEGER (0..16381),

k-2-r18						INTEGER (0..32761)

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UERxTEG | The field is optionally present, need OP, if the field nr-UE-Rx-TEG-ID is present; otherwise it is not present. |

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-DL-TDOA-SignalMeasurementInformation. If the nr-UE-Rx-TEG-ID is present and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the TOA and DL PRS-RSRP/RSRPP (if included) measurement is performed. The nr-SFN and nr-Slot in IE NR-TimeStamp correspond to the TRP provided in dl-PRS-ReferenceInfo as specified in TS 38.214 [45]. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-DL-TDOA-AdditionalMeasurements This field provides up to 3 additional RSTD measurements per pair of TRPs, with each measurement between a different pair of DL-PRS Resources or DL-PRS Resource Sets of the DL-PRS for those TRPs [45]. If this field is present, the field nr-DL-TDOA-AdditionalMeasurementsExt shall be absent.

nr-UE-Rx-TEG-ID This field provides the ID of the UE Rx TEG associated with the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. When different UE Rx TEGs for RSTD measurements are requested, the maximum number of reported RSTD measurements associated with different DL-PRS Resources per UE Rx TEG per target TRP is 4.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-DL-TDOA-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-DL-TDOA-MeasElement, provides TOA measurements of up to 4 DL-PRS Resources of a TRP with different UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 TOA measurement results with respect to different Rx TEGs. If this field is present, the field nr-DL-TDOA-AdditionalMeasurements shall be absent.

nr-RSTD-ResultDiff This field provides the additional DL RSTD measurement result relative to nr-RSTD. The RSTD value of this measurement is obtained by adding the value of this field to the value of the nr-RSTD field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-dl-PRS-Reference-BWAggregation This field provides the PRS resource set IDs for the aggregated measurement for the reference TRP which are used for RSRP/RSRPP and/or timing measurement results.

nr-DL-PRS-ResourceSetID-BWAggregation This field provides the PRS resource set IDs for the aggregated measurement which are used for RSRP/RSRPP and/or timing measurement results.

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

NEXT CHANGE

#### 6.5.10.5	NR DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1),

jointMeasurementsReq-r18 (2)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

timingReportingGranularityFactor-r18		INTEGER (6..7)					OPTIONAL, -- Need ON

nr-DL-PRS-JointMeasurementRequested-r18      NR-DL-PRS-JointMeasurementRequested-r18

OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTEG-Request This field, if present, indicates that the target device is requested to provide the nr-UE-Rx-TEG-ID in IE NR-DL-TDOA-SignalMeasurementInformation.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-TDOA-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in fields nr-AdditionalPathList or nr-AdditionalPathListExt.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-TDOA-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-TDOA-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-TDOA-ProvideLocationInformation.

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair This field specifies the maximum number of DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) and value (6..7) corresponds to (k-1..k-2) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTEG-Request should also be present.

nr-DL-PRS-JointMeasurementRequested This field indicates which two or three PFLs and the DL PRS resource sets in the two or three DL PFLs that are linked for DL PRS BW aggregation for the joint measurements.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

NEXT CHANGE

#### 6.5.10.6	NR DL-TDOA Capability Information

#### –	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

trpTEG-InfoSup	(3)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,								...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

finerReportingGranularitySupport-r18	ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

finerReportingGranularitySupport This field, if present, indicates that the target device supports finer measurement reporting granularity for RSTD measurement for k-1 and k-2 as in clause 6.5.12.4.

NEXT CHANGE

6.5.12.4	NR Multi-RTT Location Information Elements

–	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]],

[[

nr-DL-PRS-ResourceSetID-BWAggregation-r18 NR-DL-PRS-ResourceSetID-BWAggregation-r18   OPTIONAL

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

... ,

k-1-r18						INTEGER (0..3940097),

k-2-r18						INTEGER (0..7880193)

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

... ,

k-1-r18						INTEGER (0..16381),

k-2-r18						INTEGER (0..32761)

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Case2-3 | The field is mandatory present if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice case3. Otherwise it is not present. |
| TEGCase2-3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase1-2 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case1 and case2. Otherwise it is not present. |

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-NTA-Offset This field provides the NTAoffset used by the target device as specified in TS 38.133 [46], Table 7.1.2-2. Enumerated values nTA1, nTA2, nTA3, and nTA4 correspond to NTAoffset of 25600 Tc, 0 Tc, 39936 Tc, and 13792 Tc, respectively.

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. - nr-UE-Tx-TEG-ID specifies the ID of this UE Tx TEG. - carrierFreq specifies the frequency of the SRS for positioning resources. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG. For each UE Tx TEG, there may be up to 8 changes (different nr-TimeStamp) of the TEG-SRS association information provided in nr-SRS-TxTEG-Set, i.e., the maximum value for maxTxTEG-Sets is 64.

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case3 and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-TxTEG-TimingErrorMargin This field specifies the UE Tx TEG timing error margin value for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case2 or case3 and this field is absent, the receiver should consider the UE Tx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-RxTxTEG-TimingErrorMargin This field specifies the UE RxTx TEG timing error margin value for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case1 or case2 and this field is absent, the receiver should consider the UE RxTx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimeStamp This field specifies the time instance for which the measurement is performed.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-DL-PRS-ResourceSetID-BWAggregation This field provides the PRS resource set IDs for the aggregated measurement which are used for RSRP/RSRPP and/or timing measurement results.

nr-Multi-RTT-AdditionalMeasurements This field provides up to 3 additional UE Rx-Tx time difference measurements corresponding to a single configured SRS Resource or Resource Set for positioning. Each measurement corresponds to a single received DL-PRS Resource or DL-PRS Resource Set [45]. If this field is present, the field nr-Multi-RTT-AdditionalMeasurementsExt shall be absent.

nr-UE-RxTx-TEG-Info This field provides the ID(s) of the UE TEG associated with the nr-UE-RxTxTimeDiff or nr-UE-RxTxTimeDiffAdditional measurement. One of the following combinations of TEG IDs can be provided: - case1 provides the UE RxTx TEG ID; - case2 provides the UE RxTx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on; - case3 provides the UE Rx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-Multi-RTT-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-Multi-RTT-MeasElement, provides UE Rx-Tx time difference measurements of up to 4 DL-PRS Resources of a TRP with different UE RxTx or UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 measurement results with respect to different UE RxTx or UE Rx TEGs. If this field is present, the field nr-Multi-RTT-AdditionalMeasurements shall be absent.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result. The mapping of this field is defined as in TS 38.133 [46].

nr-UE-RxTxTimeDiffAdditional This field provides the additional UE Rx-Tx Difference measurement result relative to nr-UE-RxTxTimeDiff. The UE Rx-Tx Difference value of this measurement is obtained by adding the value of this field to the value of the nr-UE-RxTxTimeDiff field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

NEXT CHANGE

#### 6.5.12.5	NR Multi-RTT Location Information Request

#### –	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1),

jointMeasurementsReq-r18 (2)} (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]],

[[

nr-Multi-RTT-ReportConfig-r18		NR-Multi-RTT-ReportConfig-r18		OPTIONAL, -- Need ON

nr-DL-PRS-JointMeasurementRequested-r18      NR-DL-PRS-JointMeasurementRequested-r18

OPTIONAL  -- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16	INTEGER (0..5)					OPTIONAL  -- Need ON

}

NR-Multi-RTT-ReportConfig-r18 ::= SEQUENCE {

timingReportingGranularityFactor-r18	INTEGER (6..7)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiffMeasurementInfoRequest This field, if present, indicates that the target device is requested to report the DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) associated with the DL-PRS Resources(s) or the DL-PRS Resource Set(s) which are used in determining the UE Rx-Tx time difference measurements.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (0..5) corresponds to (k0..k5) and value (6..7) corresponds to (k-1..k-2) used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTxTEG-Request This field, if present, indicates that the target device is requested to provide the NR-UE-RxTx-TEG-Info in IE NR-Multi-RTT-SignalMeasurementInformation. Enumerated value 'case1' indicates that the target device is requested to provide the case1 choice in NR-UE-RxTx-TEG-Info, enumerated value 'case2' indicates that the target device is requested to provide the case2 choice in NR-UE-RxTx-TEG-Info, and so on.

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE RxTx TEGs and with the same UE Tx TEG. Enumerated value 'n0' indicates that the number N of different UE RxTx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE RxTx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE RxTx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTEGs should not be present.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTxTEGs should not be present.

nr-DL-PRS-JointMeasurementRequested This field indicates which two or three PFLs and the DL PRS resource sets in the two or three DL PFLs that are linked for DL PRS BW aggregation for the joint measurements.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-Multi-RTT-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in the field nr-AdditionalPathList or nr-AdditionalPathListExt.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-Multi-RTT-SignalMeasurementInstances in IE NR-Multi-RTT-ProvideLocationInformation.

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

NEXT CHANGE

#### 6.5.12.6	NR Multi-RTT Capability Information

#### –	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its NR Multi-RTT positioning capabilities to the location server.

-- ASN1START

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					ENUMERATED { supported }					OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,	nr-Multi-RTT-On-Demand-DL-PRS-Support-r17

NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-UE-RxTx-TEG-ID-ReportingSupport-r17	BIT STRING {	case1	(0),

case2	(1),

case3	(2)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType2-r17,

granularity-r17		LOS-NLOS-IndicatorGranularity2-r17,

...

}												OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)		OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

finerReportingGranularitySupport-r18	ENUMERATED { supported }				OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideCapabilities field descriptions

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-Multi-RTT-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-UE-RxTx-TEG-ID-ReportingSupport This field, if present, indicates that the target device supports nr-UE-RxTx-TEG-Info reporting in IE NR-Multi-RTT-SignalMeasurementInformation. This is represented by a bit string, with a one-value at the bit position means the particular case is supported; a zero-value means not supported: - bit 0 indicates that the target device supports the 'case1' choice in NR-UE-RxTx-TEG-Info. - bit 1 indicates that the target device supports the 'case2' choice in NR-UE-RxTx-TEG-Info. - bit 2 indicates that the target device supports the 'case3' choice in NR-UE-RxTx-TEG-Info.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-Multi-RTT-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

finerReportingGranularitySupport This field, if present, indicates that the target device supports finer measurement reporting granularity for UE Rx-Tx measurement for k-1 and k-2 as in clause 6.5.12.4.

END OF CHANGE