3GPP TSG-RAN WG2 Meeting #123-bis                                   	R2-2311406

Xiamen, China, Oct. 9-13, 2023

Agenda Item:	7.11.3

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Report of [AT123bis][608][eMBS] Shared processing (Qualcomm)

Document for:	Discussion/Decision

# Background

Report of submitted contributions under agenda item 7.11.3 in RAN2#123bis is available in R2-22311259. After a brief online session, following offline was assigned:

[AT123bis][608][eMBS] Shared processing (Qualcomm)

Scope: Discuss P3 and P4 from R2-2311259, i.e. clarify:

    - what carrierFreqMBS in the running CR refers to

    - what additional information is required for MII for shared processing compared to parameters already captured in the running CR

Intended outcome: Report in R2-2311406

Deadline:  Thursday 2023-10-12 11:00 (report uploaded for discussion during CB session)

This document is the report of the above offline discussion.

# Offline summary

Two discussion points from the contribution summary report were discussed offline. And following offline conclusions (shown in yellow highlight) were reached.

[Discussion point 1] Clarify further what carrierFreqMBS in the running CR refers to: whether that relates to broadcast gNB’s whole band where MBS service is provided, or only the CFR for broadcast; and further whether that is centre or the absolute start position, taking into account what information can be available in USD.

- FreqInfoMBS is the following, i.e. same as Rel17 understanding: If it is obtained from USD and SIB21

[Discussion point 2] Considering the outcome of discussion point 1, what additional information is required for MII for shared processing compared to parameters already captured in the running CR.

- Locationa&BW of CFR as well as pointA of non-serving cell (i.e info enough to point to the exact location of CFR), both optional.

Rapp observation: Since the unicast gNB will not know SIB1 of broadcast gNB, for the CFR locationAndBandwidthBroadcast-r17, only the absolute INTEGER (0..37949) would be applicable, but not the sameAsSib1ConfiguredLocationAndBW.

# Proposals

As discussed above, following is proposed as the summary of the offline discussion:

Proposal 1: For MII for shared processing, FreqInfoMBS in the running CR refers to the frequency information obtained from the USD or the SIB21 (i.e. same understanding as Rel-17).

Proposal 2: For MII for shared processing, signalling will support optionally reporting CFR location & BW (i.e. actual value of locationAndBandwidthBroadcast-r17 encoded as INTEGER (0..37949)) as well as point A of non-serving cell, i.e. information enough to point to the exact location of CFR, if available at the UE.

# References

[1] R2-2311259, [Pre123bis][601][eMBS] Summary of 7.11.3 Shared processing, Qualcomm