3GPP TSG-RAN WG2 Meeting #123bis	R2-2311538

Xiamen, China, October 9 – October 13 2023

Agenda item:	7.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Rapporteur summary [[AT123bis][503][feMob] subsequent CPAC security issues (Nokia)

Document for:	Discussion and Decision

# Introduction

This document captures the outcome of the following offline discussion.

[AT123bis][503][feMob] subsequent CPAC security issues (NOkia)

Scope: f2f offline, attempt further progress.

Intended outcome:

Deadline: CB acc to Meeting schedule.

# Discussion

Following are the agreements in RAN2 Main session related to security issues for S-CPAC.

Rel-18 Conditional-Reconfiguration Information element may include

- -	List of Group-ID (mapping to SN) and associated SK-counter values outside the candidate conditional configurations.

- -	The Group-ID parameter is included within each candidate conditional configuration(CondConfigAddMod) marked for subsequent CPAC.

R2 assumes that the UE need not include the selected SK-counter value in the RRC Reconfiguration Complete message as the selection of SK-counter for Inter-SN cell change follows the defined pattern according to SA3 solution. FFS if useful to still include this, to support some error/failure cases.

For Pcell-change /PSCell-change /SCG Release scenarios, if the SCPAC configuration is maintained, UE also maintains the unused SK-counter list.

We further discuss on the below open points in this offline discussion

- FFS related to scenarios for SK-counter mismatch. If such scenario exist what will be the UE and NW behaviour on the mismatch.

Modified Agreement related to SK-counter reporting based on the company views in the offline session.

UE include the selected SK-counter value in the MN RRC Reconfiguration Complete message when UE selects new SK-counter as part of S-CPAC execution.

- How to capture UE behaviour related to maintaining SK-counter status if SCPAC configuration is maintained after specific scenarios (SCG Release/PCell-Change/PSCell-Change).

Modified P4

For Pcell-change /PSCell-change /SCG Release scenarios, if the SCPAC configuration is maintained, UE also maintains the unused SK-counter list.

- Discussion on P5: Whether and How to capture RAN2 understanding in spec

RAN2 Understanding: The NW configuration ensures that the SK-counter lists assigned for SCPAC configurations and the SK-counter value assigned for CPAC configurations are uniquely different. No specification changes are needed in this regard.

- Discussion on P7

RAN2 to discuss the need for defining UE behaviour for the scenario where free SK-counter not available at the time of S-CPAC execution based on contributions in the next meeting.

No specification changes needed for UE behaviour for the Scenario where free SK-Counter not available at the time of execution. This scenario can be avoided by NW configuration.

# Conclusion

Based on the offline discussions, following are the modified agreements related to Security key change for SCPAC.

Modified P3 : UE include the selected SK-counter value in the MN RRC Reconfiguration Complete message when UE selects new SK-counter value as part of S-CPAC execution.

Modified P4: For Pcell-change /PSCell-change /SCG Release scenarios, if the SCPAC configuration is maintained, UE also maintains the unused SK-counter values.

Following are the conclusion related to P5 and P7.

RAN2 Understanding: The NW configuration ensures that The SK-counter lists assigned for SCPAC configurations and the SK-counter value assigned for CPAC configurations are uniquely different. No specification changes are needed in this regard.

No specification changes needed for UE behaviour for the Scenario where free SK-Counter not available at the time of execution. This scenario can be avoided by NW configuration.