3GPP TSG-RAN WG2 Meeting #123bis	R2-231xxxx

Xiamen, China, October 9th – 13th, 2023

Agenda: 	8.8

Source: 	Session Chair (Intel)

Title:	Report from IDC breakout session

Document for:	Approval

Organizational:

[AT123bis][700][IDC] Organizational Yi – IDC (Intel)

Scope:

- Share plans for the meetings and list/status of ongoing email discussions for the sessions.

- Share meeting notes and agreements for review and endorsement.

- [AT123bis][701][IDC]  Corrections on TS 38.331 and TS 38.300 Agreed in principle CR (Xiaomi)

Scope: To discuss the changes from R2-2310426, proposals and TPs from R2-2311046, R2-2310585, R2-2311007

Intended outcome: Report in R2-2311410 and Agreeable CRs in R2-2311411 (TS 38.331) and R2-2311412 (TS38.300) (Agreed in principle)

Deadline of company’ comments:  Wednesday 2023-10-11 2000

Deadline of comments on summary and the CR revision:  Thursday 2023-10-12 2000

## 7.10	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

Corrections. For smaller corrections please contact CR editor / Rapporteur directly.

R2-2310426	Correction to 38.300 running CR on IDC	vivo,xiaomi	draftCR	Rel-18	38.300	17.6.0	NR_IDC_enh-Core

To be treated in [AT123bis][701]

R2-2311046	Correction on the IDC Reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_IDC_enh-Core

To be treated in [AT123bis][701]

R2-2310585	Introduction of In-Device Co-existence (IDC) enhancements for NR	Xiaomi	draftCR	Rel-18	38.331	17.6.0	B	NR_IDC_enh-Core

To be treated in [AT123bis][701]

R2-2311007	Corrections for 38.331 Running CR for IDC Enhancements	Huawei, HiSilicon	discussion	Rel-18	NR_IDC_enh-Core

To be treated in [AT123bis][701]

- [AT123bis][701][IDC]  Corrections on TS 38.331 and TS 38.300 Agreed in principle CR (Xiaomi)

Scope: To discuss the changes from R2-2310426, proposals and TPs from R2-2311046, R2-2310585, R2-2311007

Intended outcome: Report in R2-2311410 and Agreeable CRs in R2-2311411 (TS 38.331) and R2-2311412 (TS38.300) (Agreed in principle)

Deadline of company’ comments:  Wednesday 2023-10-11 2000

Deadline of comments on summary and the CR revision:  Thursday 2023-10-12 2000

R2-2311410	Summary of [AT123bis][701][IDC] Corrections on TS 38.331 and TS 38.300 Agreed in principle CR (Xiaomi)	Xiaomi	discussion	Rel-18	NR_IDC_enh-Core

Proposal 1: No extra clarification is needed in the specification for the simultaneous configuration of Rel-16 FDM and Rel-18 FDM.

Proposal 2: The change in R2-2310426 is agreed and merged to the running stage-2 CR.

Proposal 3: No need to remove the “cell group” concept from the R16 IDC configuration related description.

Proposal 4: The ASN.1 structure changes of AffectedCarrierFreqRange-r18 and AffectedCarrierFreqRangeComb-r18 as proposed in R2-2311046 are agreed.

Proposal 5: Add need R for the absence case in the field description of “Cond FDM” of idc-TDM-AssistanceConfig-r18.

Proposal 6: The changes provided in R2-2310585, except for changing AffectedCarrierFreqRangeComb-r18 to AffectedCarrierFreqRange-r18, are agreed.

Proposal 7: Update the procedural text in section 5.7.4.2 and 5.7.4.3 to include the reporting of the MR-DC combination in affectedCarrierFreqCombList, as provided in R2-2311007.

Proposal 8: Update the field description for affectedCarrierFreqCombList to remove the EN-DC case.

Proposal 9: Update the procedural text in section 5.7.4.2 to align the idc-AssistanceConfig-r16 and idc-AssistanceConfig-r18 configuration related description, by using the text proposal from Xiaomi.

Proposal 10: Update the procedural text in section 5.7.4.3 by changing “frequency range” to “candidate frequency range”, as provided in R2-2311007.

Proposal 11: The field description for the idc-TDM-Assistance of the inter-node RRC message is updated as follows:

	This field is signalled upon MN not addressing IDC issue and contains IDC TDM assistance information reported by UE to MN for IDC problem.

Agreements:

1: No extra clarification is needed in the specification for the simultaneous configuration of Rel-16 FDM and Rel-18 FDM.

2: The change in R2-2310426 is agreed and merged to the running stage-2 CR.

3: No need to remove the “cell group” concept from the R16 IDC configuration related description.

4: The ASN.1 structure changes of AffectedCarrierFreqRange-r18 and AffectedCarrierFreqRangeComb-r18 as proposed in R2-2311046 are agreed.

5: Add need R for the absence case in the field description of “Cond FDM” of idc-TDM-AssistanceConfig-r18.

6: The changes provided in R2-2310585, except for changing AffectedCarrierFreqRangeComb-r18 to AffectedCarrierFreqRange-r18, are agreed.

7: Update the procedural text in section 5.7.4.2 and 5.7.4.3 to include the reporting of the MR-DC combination in affectedCarrierFreqCombList, as provided in R2-2311007.

8: Update the field description for affectedCarrierFreqCombList to remove the EN-DC case.

9: Update the procedural text in section 5.7.4.2 to align the idc-AssistanceConfig-r16 and idc-AssistanceConfig-r18 configuration related description, by using the text proposal from Xiaomi.

10: Update the procedural text in section 5.7.4.3 by changing “frequency range” to “candidate frequency range”, as provided in R2-2311007.

11: The field description for the idc-TDM-Assistance of the inter-node RRC message is updated as follows:

	This field is signalled upon MN not addressing IDC issue and contains IDC TDM assistance information reported by UE to MN for IDC problem.

R2-2311411	Introduction of In-Device Co-existence (IDC) enhancements for NR		Xiaomi	CR	Rel-18	38.331	17.4.0	4164	4	B	NR_IDC_enh-Core

Agreed in principle

R2-2311412	Introduction of In-Device Co-existence (IDC) enhancements for NR	Huawei, HiSilicon	CR	Rel-18	38.300	17.4.0	0680	4	B	NR_IDC_enh-Core

Agreed in principle