3GPP TSG RAN WG2 #124	R2-2312801

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.14.3

Source:	ZTE Corporation, Sanechips

Title:	Remaining issue on QoE measurement for NR-DC

Document for:	Discussion and Decision

Introduction

The contribution intends to discuss remaining issues on QoE support in NR-DC with consideration on RAN2 progress made last meeting and the email discussions[1].

Discussion

RVQoE reporting

Based on the email discussions, the remaining issues is on  whether the RVQoE report should be discarded and not stored and reported later if UE cannot send RVQoE report because the configured RVQoE specific SRB is not available. In our understanding RVQoE associated to SCG is for optimizing radio resource efficiency in SCG. If  the configured RVQoE specific SRB is not available, then for most the cases the RVQoE associated to SCG is useless. Furthermore, RAN2 has agreed that UE is not required to buffer the RVQoE report when UE cannot send RVQoE report because the configured RVQoE specific SRB is not available. Therefore, we think that the RVQoE report should be discarded when UE cannot send RVQoE report because the configured RVQoE specific SRB is not available.

Observation 1: RVQoE associated to SCG is mainly for optimizing radio configuration over SCG which is less  useful when  the configured RVQoE specific SRB is not available.

Proposal 1: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, the RVQoE report is discarded.

QoE reporting

RAN3: RAN3 doesn’t see any issue with this working assumption. In addition, RAN3 would also like to inform RAN2 about the latest agreements reached during RAN3#121bis, on NR-DC, and inter-RAT mobility, including: For NR-DC - Release all SN configured QoE measurements during SN release For Inter-RAT mobility - For HO from LTE/5GC to NR, there is no impacts to RAN3 - From NR to LTE, the source node decides which one of the QoE configurations to keep RAN3 thinks that the agreements on NR-DC and Inter-RAT mobility may require further RAN2 work, i.e., RAN2 needs to consider how to treat the unsent QoE report by UE during SN release, how UE release the other QoE measurements during the Inter-RAT handover.

Above LS[2] has been receive from RAN3 to inform RAN2 about their latest progress in QoE in NR-DC, where RTAN3 informs RAN2 on their decision to release all SN configured QoE measurements during SN release. Based on post email discussions in [Post123bis][618], most companies agree with RAN3’s new agreements, which means  there is no need to indicate to UE which SN configured QoE configurations should be released or kept Then, it leads to another remaining issues, which is on how to treat the unsent QoE report configured to be reported over SRB5 when SN is released. In our understanding, there could be two different cases that SN is released, one is proactively released by NW proactively  and the other is that the SN is released due to SCG failure. For the former case, if NW actively releases SN, NW can reconfigure SRB4 to send buffer reports before releasing by its implementation, which is inline with our previous agreements. As for other cases, where NW has not yet change the the reporting branch of SN  to SRB4, UE can continue buffer the QoE reports as in legacy until the configuration is released or the reporting leg is reconfigured or the SRB is available again. Therefore, we think that unsent QoE reports can be solved by reusing the legacy mechanism without additional enhancement.

Proposal 2: When SN is released, the unsent QoE reports mapped to unavailable SRB are buffered by UE until there is an SRB available for reporting, as in legacy. No enhancement is needed.

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: RVQoE associated to SCG is mainly for optimizing radio configuration over SCG which is less  useful when  the configured RVQoE specific SRB is not available.

Proposal 1: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, the RVQoE report is discarded.

Proposal 2: When SN is released, the unsent QoE reports mapped to unavailable SRB are buffered by UE until there is an SRB available for reporting, as in legacy. No enhancement is needed.

References

- RR2-2312704 [Post123bis][618][QoE] 37.340 CR update and open issues (Nokia), Oct, 2023

- R3-235807 Reply LS on Priority information and NR-DC, Oct, 2023

- R2-2311277 Report from session on MBS, QoE and LTE legacy