3GPP TSG-RAN WG2 Meeting #124	R2-2313159

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:	7.16.2.2

Source:	LG Electronics Inc.

Title:	AIML method Data Collection

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we discuss the data collection exchange scenario focusing on the CSI compression and CSI/beam prediction use case.

# 2. Discussion

## 2.1 Data Collection Exchange Scenario

Based on the RAN1 LS[1][2] and agreement, Table 1, Table 2, and Table 3 can serve as the starting point for input/output data related to the LCM purpose for each use case. Based on an analysis of when/which data exchange is required, RAN2 can discuss which existing methods is more suitable for which purpose/use case.

| CSI Compression | NW part | UE part | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = Target CSI |  | FFS | Relaxed | This row applies to Type 1, Type 2, and the first or second stage of described procedure of Type 3 separate training. |
|  | Input data = CSI Feedback |  | FFS | Relaxed | This is for dataset delivery for the second stage of described procedure of Type 3 separate training (either from Network side to UE side, or from UE side to Network side) and forward propagation information for Type 2 training. |
|  | Input data = Gradients for CSI Feedback |  | FFS | Relaxed | This is for backward propagation for Type 2 training |
| Inference | Input data = CSI Feedback | Input data = Internally available in UE | FFS | Time-critical | For NW part inference, L1 report similar to legacy CSI can be used |
| Monitoring | Input data = Target CSI, Calculated performance metric | Input data = Reconstructed CSI and/or | FFS | Near-real-time |  |

<Table 1. LCM input data related entity for CSI compression use case>

| CSI prediction | NW side | UE side | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = Target CSI | Input data = internally available in UE | FFS | Relaxed |  |
| Inference | No model operation | Input data = Internally available in UE Output data = Predicted CSI feedback | FFS | Time-critical | Can use L1 report similar to legacy CSI |
| Monitoring | Input data = ground truth (i.e., target CSI) corresponding to predicted CSI, Calculated performance metrics. | Input data = Performance metrics are available inside the UE Output data = Performance monitoring output | FFS | Near-real-time |  |

Table 2. LCM input data related entity for CSI Prediction use case>

| Beam prediction | NW side | UE side | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = L1-RSRP and/or beam-ID | Input data = internally available in UE (e.g., L1-RSRP and/or beam-ID) | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. | Relaxed |  |
| Inference | Input data = L1-RSRP, and Beam-IDs if needed for Set B | Input data = Internally available in UE Output data = Beam prediction results | For NW sided, Up to 10 bits, or up to 100 bits, or up to hundreds of bits. For UE sided, Small (10s of bits) | Time-critical | RAN1 has agreed to consider L1 signalling for this reporting |
| Monitoring | Input data = L1-RSRP(s) and/or beam-ID(s), Event occurrence and/or calculated performance metrics, L1-RSRP(s) and/or beam-ID(s) | Input data = Performance metrics are available inside the UE | For L1-RSRP(s) and/or beam-ID(s), Up to 10 bits, or up to 100 bits, or up to hundreds of bits. For Event occurrence and/or calculated performance metrics, Small(10s of bits) | Near-real-time |  |

<Table 3. LCM input data related entity for Beam prediction use case>

Among these, LCMs that require data collection and information exchange for each specific use case include:

- CSI compression:

  - Training:

    - For NW/UE part model: Target CSI, CSI feedback, and/or Gradients for CSI Feedback

    - Data set can vary depending on the training type, such as type 1/2/3

  - Inference:

    - For NW-part inference: CSI feedback from UE to NW

    - For UE-part inference: For input data, it is internally available in UE

  - Monitoring:

    - For NW-part monitoring: Target CSI and Calculated performance metrics from UE to NW

    - For UE-part monitoring: Reconstructed CSI from NW to UE

- CSI prediction:

  - Training:

    - For NW-side training: Target CSI from UE to NW

    - For UE-side training: For input data, it is internally available in UE

  - Inference:

    - For UE-side inference: Predicted CSI feedback from UE to NW (output data)

  - Monitoring:

    - For NW-side monitoring: ground truth (i.e., target CSI) corresponding to predicted CSI and Calculated performance metrics from UE to NW

    - For UE-side monitoring: For input data, Performance metrics are available inside the UE. For output data, Performance monitoring output from UE to NW (output data)

- Beam management:

  - Training:

    - For NW-side training: L1-RSRP and/or beam-ID from UE to NW

    - For UE-side training: For input data, it is internally available in UE

  - Inference:

    - For NW-side inference: L1-RSRP, and Beam-IDs if needed for Set B from UE to NW

    - For UE-side inference: For input data, it is internally available in UE. For output data, Beam prediction results from UE to NW (output data)

  - Monitoring:

    - NW-side monitoring: L1-RSRP(s) and/or beam-ID(s), Event occurrence and/or calculated performance metrics from UE to NW

    - For UE-side monitoring: For input data, Performance metrics are available inside the UE.

We propose to use Table 1, Table 2, and Table 3 as baselines for the use cases that require data collection and information exchange.

Proposal 1. To capture the Table1/2/3 that requires data collection and data exchange into the TR as shown in Annex 5

As per the RAN1 LS[2], the required latency for inference in CSI/beam use cases is Time-critical, which requires less than a few msecs. Considering latency and processing time in RRC taking up about10msec or more, the RRC messages is not suitable for data collection for model inference in CSI/beam use cases.

- In answering latency requirements, RAN1 used the following descriptions:

  - Relaxed (e.g., minutes, hours, days, or no latency requirement)

  - Near-real-time (e.g., several tens of msecs to a few seconds)

  - Time-critical (e.g., a few msecs)

Therefore, we propose to down-scope RAN2 discussion for data collection, by excluding model inference, which requires time-critical (a few milliseconds)

Proposal 2. To down-scope RAN2 discussion for data collection, by excluding model inference, which requires time-critical latency (a few milliseconds).

# 3. Conclusion

Proposal 1. To capture the Table1/2/3 that requires data collection and data exchange into the TR as shown in Annex 5

Proposal 2. To down-scope RAN2 discussion for data collection, by excluding model inference, which requires time-critical latency (a few milliseconds).

# 4. References

[1] R1-2308730 RAN2 LS reply on data collection

[2] R1-2310681 Reply LS on Data Collection Requirements and Assumptions

[3] R2-23XXXX – R2 Input to TR 38.843

# 5. Annex

#### 7.3.1.2	Data collection

Editor’s note (RAN2): There seem to be a need for further discussion in RAN2 to update, complete, and conclude on the content of this clause.

Data collection plays a crucial role in enabling the different use cases. Hence, the importance of defining the best approaches for collecting data to support UE-side and network-side model inference, monitoring, and training.

Table 7.3.1.2-1, Table 7.3.1.2-2, and Table 7.3.1.2-3 can serve as the starting point for input/output data related to the LCM purpose for each use case.

Table 7.3.1.2-1. LCM input data related entity for CSI compression use case

| CSI Compression | NW part | UE part | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = Target CSI |  | FFS | Relaxed | This row applies to Type 1, Type 2, and the first or second stage of described procedure of Type 3 separate training. |
|  | Input data = CSI Feedback |  | FFS | Relaxed | This is for dataset delivery for the second stage of described procedure of Type 3 separate training (either from Network side to UE side, or from UE side to Network side) and forward propagation information for Type 2 training. |
|  | Input data = Gradients for CSI Feedback |  | FFS | Relaxed | This is for backward propagation for Type 2 training |
| Inference | Input data = CSI Feedback | Input data = Internally available in UE | FFS | Time-critical | For NW part inference, L1 report similar to legacy CSI can be used |
| Monitoring | Input data = Target CSI, Calculated performance metric | Input data = Reconstructed CSI and/or | FFS | Near-real-time |  |

Table 7.3.1.2-2. LCM input data related entity for CSI prediction use case

| CSI prediction | NW side | UE side | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = Target CSI | Input data = internally available in UE | FFS | Relaxed |  |
| Inference | No model operation | Input data = Internally available in UE Output data = Predicted CSI feedback | FFS | Time-critical | Can use L1 report similar to legacy CSI |
| Monitoring | Input data = ground truth (i.e., target CSI) corresponding to predicted CSI, Calculated performance metrics. | Input data = Performance metrics are available inside the UE Output data = Performance monitoring output | FFS | Near-real-time |  |

Table 7.3.1.2-3. LCM input data related entity for Beam prediction use case

| Beam prediction | NW side | UE side | Typical data size | Typical latency | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | Input data = L1-RSRP and/or beam-ID | Input data = internally available in UE (e.g., L1-RSRP and/or beam-ID) | Up to 10 bits, or up to 100 bits, or up to hundreds of bits. | Relaxed |  |
| Inference | Input data = L1-RSRP, and Beam-IDs if needed for Set B | Input data = Internally available in UE Output data = Beam prediction results | For NW sided, Up to 10 bits, or up to 100 bits, or up to hundreds of bits. For UE sided, Small (10s of bits) | Time-critical | RAN1 has agreed to consider L1 signalling for this reporting |
| Monitoring | Input data = L1-RSRP(s) and/or beam-ID(s), Event occurrence and/or calculated performance metrics, L1-RSRP(s) and/or beam-ID(s) | Input data = Performance metrics are available inside the UE | For L1-RSRP(s) and/or beam-ID(s), Up to 10 bits, or up to 100 bits, or up to hundreds of bits. For Event occurrence and/or calculated performance metrics, Small(10s of bits) | Near-real-time |  |

Among these, LCMs that require data collection and information exchange for each specific use case include:

- CSI compression:

  - Training:

    - For NW/UE part model: Target CSI, CSI feedback, and/or Gradients for CSI Feedback

    - Data set can vary depending on the training type, such as type 1/2/3

  - Inference:

    - For NW-part inference: CSI feedback from UE to NW

    - For UE-part inference: For input data, it is internally available in UE

  - Monitoring:

    - For NW-part monitoring: Target CSI and Calculated performance metrics from UE to NW

    - For UE-part monitoring: Reconstructed CSI from NW to UE

- CSI prediction:

  - Training:

    - For NW-side training: Target CSI from UE to NW

    - For UE-side training: For input data, it is internally available in UE

  - Inference:

    - For UE-side inference: Predicted CSI feedback from UE to NW (output data)

  - Monitoring:

    - For NW-side monitoring: ground truth (i.e., target CSI) corresponding to predicted CSI and Calculated performance metrics from UE to NW

    - For UE-side monitoring: For input data, Performance metrics are available inside the UE. For output data, Performance monitoring output from UE to NW (output data)

- Beam management:

  - Training:

    - For NW-side training: L1-RSRP and/or beam-ID from UE to NW

    - For UE-side training: For input data, it is internally available in UE

  - Inference:

    - For NW-side inference: L1-RSRP, and Beam-IDs if needed for Set B from UE to NW

    - For UE-side inference: For input data, it is internally available in UE. For output data, Beam prediction results from UE to NW (output data)

  - Monitoring:

    - NW-side monitoring: L1-RSRP(s) and/or beam-ID(s), Event occurrence and/or calculated performance metrics from UE to NW

    - For UE-side monitoring: For input data, Performance metrics are available inside the UE.

Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to the gNB. As highlighted in Section 4.2, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting. Nonetheless, properties of the different methods listed in the Table can prove to be useful towards the analysis, irrespective of the RRC state for which these are designed or intended.

Table 7.3.1.2-4. Existing data collection methods identified.

| Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (It can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (It can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reportng | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - NW-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
****Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.