3GPP TSG-RAN WG2 Meeting #124	R2-2313205

Chicago, USA, 13 – 17 November 2023

Title:	[DRAFT] Reply LS on network assistant signalling for advanced receiver

Response to:	R4-2316980

Release:	Release 18

Work Item:	NR_demod_enh3-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG4

Cc:	TSG RAN WG1

Contact Person:

#### Name:	Andrew Lappalainen

E-mail Address:	andrew.lappalainen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN4 for the LS on network assistant signalling for advanced receivers.

RAN2 agrees to use RRC signalling to inform the UE supporting advanced receiver about the existence MU-MIMO DCI signalling.

Furthermore, RAN2 has made the following agreements about the network assistant signalling:

- Each of the network-signalled indications for advanced receiver are optional.

- By default, the UE treats the following assumptions as valid/true. Network assistant signalling is used to indicate if these assumptions are not valid/false.

  - The precoding and resource allocation of the co-scheduled UE(s) are the same in the PRG-level grid configured to the target UE when PRG=2 or 4.

  - The time domain resource assignment for PDSCH symbols of all co-scheduled UE(s), which have the same DM-RS sequence of the target UE, are the same as the target UE.

- By default, the UE considers that any one of the MCS tables {qam64, qam256, qam1024} could have highest modulation order among the MCS tables configured for the co-scheduled UE(s) with same DM-RS sequence as the target UE. Network assistant signalling is used to indicate explicitly to the UE which MCS table has highest modulation order.

Regarding the request to signal the following:

- Whether the DM-RS power boosting configurations (i.e., Number of DM-RS CDM groups without data, TS38.214 table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE

RAN2 believes the above condition has been made obsolete and therefore no longer needs to be signalled due to an agreement made at RAN1#114bis that a “UE may assume that CDM groups without data are not used for data transmission for any co-scheduled user in the same serving cell”. RAN2 would like to check if RAN4 has the same understanding.

2. Actions:

To RAN WG4.

ACTION: 	RAN2 respectfully asks RAN4 to confirm that it is no longer necessary to signal whether the DM-RS power boosting configurations (i.e., Number of DM-RS CDM groups without data, TS38.214 table 4.1-1) of all the co-scheduled UE(s), which has the same DM-RS sequence as the target UE, is the same as the target UE.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#125	from 2024-02-26	to 2024-03-01		Athens, GR

RAN2#125bis	from 2024-04-15	to 2024-04-19		China, CN