3GPP TSG-RAN WG2 Meeting #124		R2-2313352

Chicago, US, November 13th – 17th, 2023

Source:	vivo

Title:	Discussion on measurement gap enhancement

Agenda Item:	7.25.1.7

Document for:	Discussion

- Introduction

RAN2#123bis meeting has achieved below agreements:

R2-2310362 Discussion on further measurement gap enhancement MediaTek Inc. discussion NR_MG_enh2-Core Proposal 1: Remove below editor note in endorsed LTE running CR (R2-2306802) for MGE; and endorse the update LTE running CR in R2-2310393 (36.331) and R2-2310395 (36.306). • Editor Note: It is FFS whether UE indicating “no-gap-with-interruption” shall indicate TRUE for the corresponding NR band entry in the interRAT-NeedForGapsNR. - ZTE still has some concern with the ASN1 structure => Remove the editor’s note for now. => FFS if we need to specify something regarding what is expected if the UE doesn’t report this Rel-18 capability Draft CRs R2-2310393 Introduction of measurements without gap with interruption MediaTek Inc., Huawei, HiSilicon CR Rel-18 36.331 17.6.0 4929 4 B NR_MG_enh2-Core R2-2308766 => The CR is endorsed R2-2310395 Introduction of measurements without gap with interruption MediaTek Inc., Huawei, HiSilicon CR Rel-18 36.306 17.4.0 1870 4 B NR_MG_enh2-Core R2-2308767 => The CR is endorsed R2-2310397 Running CR for further measurement gap enhancements MediaTek Inc., Huawei, HiSilicon CR Rel-18 38.331 17.6.0 4063 4 B NR_MG_enh2-Core R2-2308768 => Combine periodicity and offset into a single field => the CR is postponed R2-2310403 Running CR for further measurement gap enhancements capabilities MediaTek Inc., Huawei, HiSilicon CR Rel-18 38.331 17.6.0 4286 1 B NR_MG_enh2-Core R2-2308769 => the CR is endorsed R2-2310404 Running CR for further measurement gap enhancements capabilities MediaTek Inc., Huawei, HiSilicon CR Rel-18 38.306 17.6.0 0906 4 B NR_MG_enh2-Core R2-2308770 => the CR is endorsed

In this contribution, we will discuss Rel-18 signalling for measurement gap enhancement.

- Discussion

2.1 Clarification of interruption ambiguity

FFS if we need to specify something regarding what is expected if the UE doesn’t report this Rel-18 capability

Regarding Rel-17 “NeedForGapNCSG” feature, RAN2 has agreed to clarify that value nogap-noncsg in the NeedForNCSG-InfoEUTRA-r17 or NeedForGapNCSG-InfoNR-r17 means no gap without interruption.

RAN2 #121bis-e: RAN2 understands that no need to extend the concept of “no-gap measurement with interruption” to Rel-17 NeedForGapNCSG reporting. RAN2 assumes "nogap-noncsg" in Rel-17 NeedForGapNCSG signalling implies "no gap and no interruptions". The Rel-18 indication is in addition to the legacy NeedForGaps information. The UE may report 3 different cases: - If gap is needed, the UE reports “gap” in Rel-16 field and empty field in corresponding R18 IE. - If gap is NOT needed and there is no interruption, the UE reports “no-gap” in Rel-16 field and “no-gap-no-interruption” in Rel-18 field - If gap is NOT needed but there is interruption, the UE reports “no-gap” in Rel-16 field and “no-gap-with-interruption” in Rel-18 field - If the NW does not request Rel-18 NeedForInterruptionInfoNR, the UE only reports NeedForGap in the legacy way. RAN2#123: RAN2 to clarify that value nogap-noncsg in the NeedForGapNCSG-InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications. (this is a rel-17 change, and separate CR is needed, can be provided next meeting).

Below is the summary of gap indication in Rel-16/17/18 according to [1][2][3]:

| Feature | Gap Indication |
| --- | --- |
| Rel-18 NeedForInterrupt | no-gap-with-interruption: interruption is needed no-gap-no-interruption: interruption is not needed If gap is needed, the UE reports “gap” in Rel-16 field and empty field in corresponding R18 IE. |
| Rel-17 NeedForGapNCSG | Gap: gap is required Ncsg: NCSG is needed. nogap-noncsg: neither a measurement gap nor a NCSG is needed. Interruption is not needed |
| Rel-16 NeedForGap | Gap: gap is required. no-gap: gap is not needed. RAN4 do not specify whether interruptions are allowed or not for “no-gap” feature in Rel-16. |

Then we check the interrupt ambiguity issue according to different UE/NW releases.

| Case# | UE Release | NW Release | Applied Feature |
| --- | --- | --- | --- |
| 1 | Rel-18 | Rel-18 | Rel-18 NeedForInterrupt, or Rel-17 NeedForGapNCSG, or Rel-16 NeedForGap. |
| 2 | Rel-17 | Rel-18 | Rel-17 NeedForGapNCSG，or Rel-16 NeedForGap. |
| 3 | Rel-18 | Rel-17 | Rel-17 NeedForGapNCSG，or Rel-16 NeedForGap. |
| 4 | Rel-17 | Rel-17 | Rel-17 NeedForGapNCSG，or Rel-16 NeedForGap. |
| 5 | Either UE or NW is Rel-16 |  | Rel-16 NeedForGap. |

Based on above RAN2 agreement and feature analysis, we observe that:

Observation 1: Interrupt behaviour is clear if NW requests Rel-17/18 reporting.

Then, the interrupt ambiguity issue mainly exists when:

- NW does not request  Rel-17/18 reporting;

- NW requests only Rel-16 reporting and UE reports “no gap”.

- Either UE or NW is Rel-16

We have to accept the ambiguity issue if either UE or NW is Rel-16. In other cases, there is no ambiguity issue if NW requests UE to reporting Rel-17/18 gap indications. i.e. Up to the Network to resolve the ambiguity issue.

Therefore, back to the FFS, we don’t think it’s needed to specify anything regarding what is expected if the UE doesn’t report this Rel-18 capability. It’s up to network implementation.

Hence, we propose that:

- No need to specify anything regarding what is expected if the UE doesn’t report the Rel-18 capability.

- Conclusion

Based on the discussion, we have the following observations and proposals:

Observation 1: Interrupt behaviour is clear if NW requests Rel-17/18 reporting

Proposal 1: No need to specify anything regarding what is expected if the UE doesn’t report the Rel-18 capability.

- References

- R2-2309459	(R4-2314457) Reply LS on measurements without gaps

- R2-2310668	Clarification on the meaning of nogap-noncsg, ZTE Corporation, Nokia, Sanechips

- R2-2310397	Running CR for further measurement gap enhancements