3GPP TSG-RAN2 Meeting #124	R2-2313355

# Chicago, USA, Nov 13-17 2023

Agenda Item:	7.8.5

Release:	Rel-18

Work Item:	NR_UAV-Core

Source: 	Xiaomi

Title: 	Support of PQI Values for DAA/BRID

Document for:	Discussion/Decision

# 1. Introduction

RAN2 have received LS in [1] with confirmation from SA2 they have defined new PQI Values to support A2X communication over the PC5 interface in CR[2].

# 2. Discussion

The A2X service PQI values are both GBR type in which case they can only be considered for delivery via unicast PC5 communication for DAA ony. Or as Non-GBR type, for which broadcast can be considered over the PC5 interface for both BRID and DAA.

Comparing below in figure 1 for the non-GBR type as examples, the newly defined A2X PQI Values (62 & 64) with the V2X service examples already defined in [3] Table 5.4.4-1, shown (PQI Values 55-57), we can see clearly that these A2X service QoS characteristics are consistent with previously defined PQI value QoS characteristics defined for the V2X service QoS requirements.

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 55 | Non-GBR | 3 | 10 ms | 10-4 | N/A | N/A | Cooperative lane change – higher degree of automation |
| 56 |  | 6 | 20 ms | 10-1 | N/A | N/A | Platooning informative exchange – low degree of automation; Platooning – information sharing with RSU |
| 57 |  | 5 | 25 ms | 10-1 | N/A | N/A | Cooperative lane change – lower degree of automation |
| 62 | Non-GBR | 4 | 100 ms | 10-2 | N/A | N/A | Broadcast Remote ID with lower latency requirement |
| 64 |  | 4 | 50 ms | 10-2 | N/A | N/A | Broadcast mode Detect and Avoid with lower latency requirement |

Figure 1 – comparison of V2X PQI values with A2X PQI values for non-GBR Resource types

SA2’s motivation in providing these PQI values for A2X services was,

“defining new PQI(PC5 5QI) value(s) is beneficial for BRID and DAA over NR PC5 so that the AS layer can distinguish A2X services from V2X services if differentiated handling is needed. For BRID and DAA over LTE PC5, it is considered that the existing QoS framework can be used.”

Currently in the Running TS38.300 CR [1] an open FFS as to whether to capture new PQI for BRID/DAA, we propose it is captured and propose an update to the section of TS38.300 in the following.

For LTE over PC5, SA2 already confirmed that the existing QoS framework can be used.

Proposal: The TS 38.300 stage 2 running CR is updated to capture the use of the A2X specific PQI Values as per the following.

# 2 References … [55] 3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol". [x] 3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2". ### 16.X.8 BRID and DAA Support via A2X Communication The Aerial UE supports A2X communication which is broadcasting of BRID and DAA messages using NR sidelink PC5 interface. DAA can also be provided via unicast transmissions over the NR sidelink PC5 interface based on configured A2X policy. BRID and DAA message transmission is supported in both in-coverage and out-of-coverage scenarios and relies only on UE autonomous resource selection for NR sidelink communication. BRID and DAA follow the QoS framework defined for NR sidelink with PQI values defined in [x] Table 6.2.4.1-1 with varying levels of QoS support for different performance of respective A2X service requirements. The NG-RAN can configure a separate SL Tx resource pool for BRID and DAA specifically, to ensure varying QoS requirements can be supported.

# 3. Conclusion

Based on the introduction of new PQI Values specifically in support of A2X services and the support of BRID and DAA over the NR sidelink PC5 interface,

Proposal: The TS 38.300 stage 2 running CR is updated to capture the use of the A2X specific PQI Values as follows.

TS 38.300 # 2 References … [55] 3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol". [x] 3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2". ### 16.X.8 BRID and DAA Support via A2X Communication The Aerial UE supports A2X communication which is broadcasting of BRID and DAA messages using NR sidelink PC5 interface. DAA can also be provided via unicast transmissions over the NR sidelink PC5 interface based on configured A2X policy. BRID and DAA message transmission is supported in both in-coverage and out-of-coverage scenarios and relies only on UE autonomous resource selection for NR sidelink communication. BRID and DAA follow the QoS framework defined for NR sidelink with PQI values defined in [x] Table 6.2.4.1-1 with varying levels of QoS support for different performance of respective A2X service requirements. The NG-RAN can configure a separate SL Tx resource pool for BRID and DAA specifically, to ensure varying QoS requirements can be supported.

# 4. References

[1]	R2-2311761 (S2-2311556) LS on New PQI values for A2X communication over PC5 reference point. SA2

[2]	S2-2311428 TS 23.256 CR#0109

[3]	3GPP TS 23.287 “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 18)”