3GPP TSG-RAN WG2 Meeting #124	        R2-2313441

Chicago, USA, Nov. 13th-17th, 2023

Agenda item:	    7.3.2

Source: 		III

Title: 	Discussion on Cell DTX/DRX NES

Document for:	Discussion

Introduction

In previous RAN2#121bis-e meeting [1]. Some issues were raised about SR process, as following:

Agreements - A periodic cell DTX/DRX configuration is explicitly signalled to the UEs. - A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling. - The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration. - As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released. - From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details. Ask about feasibility and reliability of using L1 signaling. Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration. Extract a few key benefits of dynamic signaling from email discussion and online discussions. - As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period - As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods - As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic - (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH. For the FFS case there may be some exceptions - The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). FFS how to deal with retransmissions

In previous RAN2#123 [2] meeting, the agreements of behaviour of Cell DTX/DRX parameter on the email discussion [POST122] [307] [3].As following.

Agreements: - Activation/deactivation is per serving cell. FFS if the configuration is per cell or per MAC entity 2 RAN2 will reuse the start timer formula of the onDurationTimer from UE C-DRX (including SlotOffset) to specify the start of cellDTX-onDurationTimer (and cellDRX-onDurationTimer) in 38.321. 3 The gNB should ensures that there is at least partial overlapping between UE C-DRX on-duration and cell DTX/DRX on-duration. It is up to network implementation to ensure the alignment. We will capture this in stage 2 specification. Understanding is that alignment means that the cell DTX/DRX and C-DRX periodicity should be multiple of each other. FFS if we anything needs to be specified in stage 3 (i.e. in IE description) 4 As a baseline legacy C-DRX reconfiguration is used to change UE C-DRX configuration once Cell DTX/DRX is activated/deactivated. 5 RAN2 specifies cellDTX-onDurationTimer (and cellDRX-onDurationTimer) to have the same value range as UE C-DRX on-duration timer. 6 RAN2 specifies cellDTX-Cycle (and cellDRX-Cycle) to have the same value range as UE C-DRX Long cycle. 7 Separate DTX and DRX configuration means that the features can be enabled separately (i.e. Cell DTX can be configured without Cell DRX) 8 On-duration and Cycle parameters are common between cell DTX and DRX, when both are configured. FFS if we have different start offset configuration for cell DTX and cell DRX 9 RAN2 will not introduce a MAC CE for cell DTX/DRX (de)activation. 10 Confirm working assumption, when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX. 11 We focus on the case where DTX in RRC can only be configured when C-DRX is configured. We will not optimize for the case where C-DRX is not configured.

In previous RAN2#123bis-e meeting, the agreement of Cell DTX/DRX configuration is maximum of two cell DTX/DRX patterns per MAC entity on offline [AT123bis][003][4].As following:

Agreements - Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions: - A maximum of two cell DTX/DRX patterns can be configured per MAC entity - The two configured patterns are aligned, - The start and slot offset are common for the two patterns. - one periodicity is an integer multiple of the other. - Working assumption: UE triggers RACH upon determining that an emergency call is initiated during the cell DTX/DRX non active period. We rely on the UE implementation to determine whether an emergency call is initiated. We will take time to check until next meeting to confirm the WA.

In this proposal, first we discuss the cell DTX/DRX mechanism about UE does not transmit SR overlapping with Cell DRX non-active periods. Second, two considerations are proposed for the decision of cell DTX/DRX patterns. Third, the retransmission behavior is discussed when UE C-DRX active time overlapping with Cell DTX/DRX non-active duration.

Discussion

SR process overlapping with cell DRX non-active period

In RAN2#121bis-e [1] meeting agreement, If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending. In TS 38.321[5], the SR process was specified as Fig 1.

Fig 1 SR occasions

When the UE will transmit data to gNB, it transmits an SR to gNB. This SR informs gNB that UE will transmit data, and triggers sr-ProhitbitTimer. This means SR should be transmitted in Cell DRX on duration. So one possible approach is to configure the PeriodicityaAndOffset value in SRresource configuration, as following:

- Periodicity should be the same or multiple of the serving Cell’s DRX periodicity, and Offset should be aligned with the serving Cell’s DRX periodicity.

Three methods were discussed for the UE keeping SR pending when SR process overlapping with Cell DRX non-active period.

- The UE retransmits SR even during Cell DRX non-active period, but SR_COUNTER is not incremented. Therefore, when SR_COUNT > sr-TransMax, UE is ensured that RACH is triggered during Cell DRX on duration in fig 2.

Fig 2 SR is transmitted during Cell DRX non-active period, UE is ensured that RACH is triggered during Cell DRX on duration.

- Extend the value of Sr-TransMAX to floor [periodicity/on Duration].

- The sr-ProhibitTimer value in SR configuration should be the same or multiple of the serving Cell’s DRX periodicity. This will forces retransmission of SR occurring in Cell DRX on duration.

Proposal 1: Three methods were proposed for the UE keeping SR pending when SR process overlapping with Cell DRX non-active period.

- Option 1: The UE retransmits SR even during Cell DRX non-active period, but SR_COUNTER is not incremented. Therefore, when SR_COUNT > sr-TransMax, UE is ensured that RACH is triggered during Cell DRX on duration in fig 2.

- Option 2: Extend the value of Sr-TransMAX to floor [periodicity/on Duration].

- Option 3: The sr-ProhibitTimer value in SR configuration should be the same or multiple of the serving Cell’s DRX periodicity. This will forces retransmission of SR occurring in Cell DRX on duration.

Cell DTX/DRX patterns

In RAN2#123-bis-e agreement - A maximum of two cell DTX/DRX patterns can be configured per MAC entity. Two consideration are proposed for the decision of Cell DTX/DRX patterns. First, whenever Cell DTX/DRX is in its on duration, all UEs should be also in their on duration as well (in Fig3). To meet this requirement, Cell DTX/DRX periodicity should be the least common multiple (LCM) of all UEs’ C-DRX periodicity. Thereby this will maximize the Cell DTX/DRX non-active period. Second, wherever any UE is in its on duration. The serving cell should be also in its on duration as well (in Fig.4). To meet this requirement, Cell DTX/DRX periodicity should be the greatest common divisor (GCD) of all UEs’ C-DRX periodicity. This will serve all UEs as legacy.

- Maximizes NES gain: The period of Cell DTX/DRX pattern should be the least common multiple (LCM) of UEs’ C-DRX periodicities. For example, UE_1 C-DRX periodicity is 8 ms and UE 2 C-DRX periodicity is 12 ms, highest common factor is 24ms.Therefore, Cell DTX/DRX configuration periodicity is 24ms.

Fig 3. Cell DTX/DRX periodicity is the LCM of all UEs’ C-DRX periodicity

Observation 1: Cell DTX/DRX periodicity should be the least common multiple (LCM) of all UEs’ C-DRX periodicity. This will maximize the Cell DTX/DRX non-active period.

- Cell DTX/DRX periodicity should be the greatest common divisor (GCD) of all UEs’ C-DRX periodicity. For example, UE_1 C-DRX periodicity is 8 ms and UE_2 C-DRX periodicity is 12 ms, highest common factor is 4ms.Therefore, Cell DTX/DRX configuration periodicity is 4ms.

Fig 4. Cell DTX/DRX periodicity is the GCD of all UEs’ C-DRX periodicity.

Observation 2: Cell DTX/DRX periodicity should be the greatest common divisor (GCD) of all UEs’ C-DRX periodicity. This will serve all UEs as legacy.

Proposal 2: The period list of all Cell DTX/DRX patterns should include LCM of all UEs’ C-DRX periodicity and GCD of all UEs’ C-DRX periodicity

UE C-DRX active time overlap with cell DTX/DRX non-active duration

As TS38.321, legacy UE C-DRX mechanism is as shown in fig 5. Some cases was discussed when the UE C-DRX active time (drx-inactivityTimer, drx-RetransmissionTimer) overlapping with Cell DTX/DRX non-active duration.

Fig 5 UE C-DRX mechanism

When UE C-DRX drx-RetransmissionTimer overlapping with Cell DTX/DRX non-active duration (as shown fig 6), the UE won’t receive retransmission from gNB during Cell DTX/DRX non-active duration. Two models were discuss for UE C-DRX drx-RetransmisssionTimer in overlap situation, as following:

- Model A: when UE C-DRX HARQ NACK feedback was received by gNB during Cell DTX/DRX on duration, gNB understand UE C-DRX retransmission time overlapping with Cell DTX/DRX non-active duration and gNB continues the retransmission in next UE C-DRX on-Duration/ Cell DTX/DRX on duration.

- Model B: when UE C-DRX HARQ NACK feedback was received by gNB during Cell DTX/DRX on duration, gNB understand UE C-DRX retransmission time overlapping with Cell DTX/DRX non-active duration and gNB cancels the retransmission and reschedules it in next UE C-DRX on-Duration as new transmission.

Fig 6 UE C-DRX drx-RetransmissionTimer overlapping with cell DTX/DRX non-active duration.

Proposal 3: For UE C-DRX drx-RetransmissionTimer overlapping with Cell DTX/DRX non-active duration, further discussion is needed for the following two models:

- Model A: gNB continues the retransmission in next UE C-DRX on-Duration/ Cell DTX/DRX on duration,

- Model B: gNB cancels the retransmission and reschedules it in next UE C-DRX on-Duration as new transmission.

So far, the definition of Cell DTX/DRX active time is not clear. Therefore, the relationship between Cell DTX/DRX on duration and Cell DTX active time needs to be specified. Other definitions can be discussed in later release, e.g.

- Cell DTX/DRX Active Time.

- Cell DTX/DRX-InactivityTimer.

- Cell DTX/DRX-RetransmissionTimerDL.

- Cell DTX/DRX-RetransmissionTimerUL.

Proposal 4: In order to reduce the scope of Rel-18, we consider only Cell DTX/DRX on duration as Cell DTX/DRX active time.

Conclusion

Proposal 1: Three methods were proposed for the UE keeping SR pending when SR process overlapping with Cell DRX non-active period.

- Option 1: The UE retransmits SR even during Cell DRX non-active period, but SR_COUNTER is not incremented. Therefore, when SR_COUNT > sr-TransMax, UE is ensured that RACH is triggered during Cell DRX on duration in fig 2.

- Option 2: Extend the value of Sr-TransMAX to floor [periodicity/on Duration].

- Option 3: The sr-ProhibitTimer value in SR configuration should be the same or multiple of the serving Cell’s DRX periodicity. This will forces retransmission of SR occurring in Cell DRX on duration.

Observation 1: Cell DTX/DRX periodicity should be the least common multiple (LCM) of all UEs’ C-DRX periodicity. This will maximize the Cell DTX/DRX non-active period.

Observation 2: Cell DTX/DRX periodicity should be the greatest common divisor (GCD) of all UEs’ C-DRX periodicity. This will serve all UEs as legacy.

Proposal 2: The period list of all Cell DTX/DRX patterns should include LCM of all UEs’ C-DRX periodicity and GCD of all UEs’ C-DRX periodicity

Proposal 3: For UE C-DRX drx-RetransmissionTimer overlapping with Cell DTX/DRX non-active duration, further discussion is needed for the following two models:

- Model A: gNB continues the retransmission in next UE C-DRX on-Duration/ Cell DTX/DRX on duration,

- Model B: gNB cancels the retransmission and reschedules it in next UE C-DRX on-Duration as new transmission.

Proposal 4: In order to reduce the scope of Rel-18, we consider only Cell DTX/DRX on duration as Cell DTX/DRX active time.

References

- R2-2304203, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning, April 17-26, 2023.

- R2-2308963, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning, August 21-25, 2023.

- R2-2307527, Outcome of [Post122][307][NES] DTX/DRX – alignment, single/multiple configurations, parameter values (Huawei), 21 – 25 August, 2023.

- R2-2311285, Summary of [AT123bis][003][NES] Offline discussion  (InterDigital), October 9th – 13th,2023.

- TS 38.321 V17.4.0, Medium Access Control (MAC) protocol specification, 2023-03.