3GPP TSG-RAN WG2 Meeting #124		R2-2313509

# Chicago, USA, 13 – 17 November 2023

Agenda item:	7.9.2

Source:	Samsung Electronics

Title:	SRAP design for U2U Sidelink Relay: remaining issues

Document for:	Discussion & Decision

# Introduction

This contribution discusses key outstanding issues with regards to SRAP design for L2 U2U relaying while taking into account recent RAN2 agreements on the topic.

# Modelling of SRAP entity and detailed SRAP entity description

The current running SRAP CR states: “On the U2N Remote UE, U2U Relay UE and U2U Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface.” We were proponents of having only one SRAP entity per U2U Relay UE, and we are supportive of the general direction of this sentence. We did however find some potential discrepancy between RRC and SRAP specs (possibly for legacy as well), and within the SRAP spec itself, and we address those in this section.

A Relay UE (both U2U and U2N) will establish multiple PC5 connections (so multiple separate RLC/MAC/PHY ‘stacks’ for communication to different Remote UEs). It is safe to assume that the parameters used to configure these multiple RLC/MAC/PHY stacks are different (depending on conditions of individual links, buffer status, traffic type etc.) – in other words separate RLC/MAC/PHY stacks are instantiated. It is then assumed that on top we have one SRAP entity per U2U Relay UE, as the SRAP entity will perform routing across all these individual RLC/MAC/PHY ‘stacks’. However the wording ‘at the PC5 interface’ in the SRAP spec apparently has (at least) two different meanings. In the above-quoted example, ‘PC5 interface’ in our understanding refers to a collection of PC5 links, whereas in the below example, it appears to refer to a specific PC5 link:

For U2U data packet, the receiving part on the SRAP entity of PC5 interface between the U2U Relay UE and the U2U Remote UE delivers SRAP Data PDUs to the transmitting part on the SRAP entity of PC5 interface between the U2U Relay UE and the peer U2U Remote UE. …

Observation 1. The term ‘PC5 interface’ is used in the SRAP spec to denote both a collection of PC5 links (e.g. the counterpart to the Uu interface in the U2N case), and also to denote a specific PC5 link (between the Relay UE and an individual Remote UE).

In addition, if we look at the following text from the RRC spec:

##### 5.8.9.7.2	PC5 Relay RLC channel addition/modification

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

In our understanding of the legacy (U2N case), an SRAP entity at the Relay UE is only established if there is no SRAP entity at the Relay UE already. In other words, when the first PC5 Relay RLC channel is added, an SRAP entity is established. Further additions of RLC channels on the same link or even other links, do not lead to addition of other SRAP entities. We further believe think this is confirmed by SRAP spec, where we have the following statement (for legacy): “On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface.” Since there is only one Uu link, we can assume there is only one SRAP entity at Uu interface. And then it makes sense that there should be only one entity across all PC5 links (although the wording ‘at the PC5 interface’ is unclear in our view, as per our Observation 1 above).

So we have the following two options:

- “No SRAP entity” refers to the Relay UE itself as a whole, and there is no discrepancy between RRC and SRAP specs (i.e. our understanding immediately above is correct), but SRAP spec itself contains inconsistency as per our Observation 1.

- “No SRAP entity” refers to the associated Remote UE, and there is therefore a potential discrepancy between RRC and SRAP, i.e. even for legacy (U2N), SRAP entity is established at the Relay UE per Remote UE according to RRC spec, whereas according to SRAP spec there is one entity per Relay UE.

Observation 2. There may be a lack of clarity (even for the legacy case) as to whether a single SRAP entity is established per Relay UE, or per PC5 link, as “no SRAP entity has been established” could refer to the associated Remote UE, or to the Relay UE itself.

Based on the above, we propose the following:

Proposal 1. RAN2 to confirm whether the meaning of ‘PC5 interface’ as used in the SRAP spec is to denote a collection of PC5 links (e.g. the counterpart to the Uu interface in the U2N case), or to denote a specific PC5 link (between the Relay UE and an individual Remote UE), and to align across the SRAP spec accordingly.

Proposal 2. RAN2 to confirm that there is only one (PC5) SRAP entity per U2N Relay UE and U2U Relay UE, and to align the RRC spec if needed by clarifying that “no SRAP entity has been established“ used in PC5 Relay RLC channel addition/modification refers to the Relay UE itself as a whole.

# Details of egress RLC channel determination at U2U Remote UE

For the legacy (U2N) case, we do not add an SRAP header for all data. In other words, there is special treatment of e.g. SRB0, and the procedures for DL and UL egress RLC channel determination are quite different. For the U2U case, we expect no special handling per RB type; therefore, we feel that the U2N DL procedure can be reused to cover the U2U case in both directions:

Proposal 3. U2N DL procedure for determining egress RLC channel can be reused to cover the U2U Remote UE case in both directions.

# BEARED ID derivation

In legacy case, BEARER ID was linked to srb-Identity/drb-Identity, which was configured by RRC. For U2U, SLRB index is a 9-bit number which identifies one of a number of RBs between 1 and a maximum number of 512.

For this U2U case, values 0, 1, 2 and 3 are already specified for SL SRBs. If RadioBearerConfigList (which follows legacy R16 SL communication) is configured, SL DRB ID will simply use this parameter. If the IE SLRB-Uu-ConfigIndex is configured to identify a SL DRB, slrb-PC5-ConfigIndex which is identical to slrb-Uu-ConfigIndex (except that slrb-Uu-ConfigIndex is assigned by gNB upon SLRB configuration derivation from gNB), is used between the two UEs. BEARER ID can then comprise 5 least-significant bits (LSBs) of slrb-PC5-ConfigIndex + 3.

For the SL DRBs, our proposal is therefore:

Proposal 4. When network-configured identifier is used for the SL DRBs, BEARER ID consist of 5 least-significant bits (LSBs) of slrb-PC5-ConfigIndex + 3.

# Local ID assignment

At RAN2#123-bis the following agreement was made:

WA: Carry L2 ID and Local ID in RRCReconfigurationSidelink message with the assumption that the association between User Info and L2 ID is done at ProSe layer.

Confirmation of the WA is pending a reply-LS from SA2; we are supportive of confirming the WA in RAN2.

The current WA implies that the local ID is per L2 ID, rather than per Remote UE. In cases where multiple L2 IDs are allocated to a single Remote UE, this allocation could lead to multiple SRAP IDs. However, we did also agree the following at RAN2#123-bis:

The UE ID assignment for U2U remote UEs is up to U2U relay UE implementation, i.e., no specification impact on how to assign the local ID is needed.

Therefore we propose the following:

Proposal 5. There is no need to specify whether SRAP ID is assigned per Remote UE, per L2 ID of said Remote UE, or per source/destination pair.

# Conclusion

On the topic of SRAP modelling, we made the following observations:

Observation 1. The term ‘PC5 interface’ is used in the SRAP spec to denote both a collection of PC5 links (e.g. the counterpart to the Uu interface in the U2N case), and also to denote a specific PC5 link (between the Relay UE and an individual Remote UE).

Observation 2. There may be a lack of clarity (even for the legacy case) as to whether a single SRAP entity is established per Relay UE, or per PC5 link, as “no SRAP entity has been established” could refer to the associated Remote UE, or to the Relay UE itself.

Based on the above, we proposed the following:

Proposal 1. RAN2 to confirm whether the meaning of ‘PC5 interface’ as used in the SRAP spec is to denote a collection of PC5 links (e.g. the counterpart to the Uu interface in the U2N case), or to denote a specific PC5 link (between the Relay UE and an individual Remote UE), and to align across the SRAP spec accordingly.

Proposal 2. RAN2 to confirm that there is only one (PC5) SRAP entity per U2N Relay UE and U2U Relay UE, and to align the RRC spec if needed by clarifying that “no SRAP entity has been established“ used in PC5 Relay RLC channel addition/modification refers to the Relay UE itself as a whole.

On the issue of egress RLC channel determination, our proposal was as follows:

Proposal 3. U2N DL procedure for determining egress RLC channel can be reused to cover the U2U Remote UE case in both directions.

For the BEARER ID determination for the SL DRB case, our proposal is:

Proposal 4. When network-configured identifier is used for the SL DRBs, BEARER ID consist of 5 least-significant bits (LSBs) of slrb-PC5-ConfigIndex + 3.

And finally, on the topic of SRAP ID assignment, we proposed:

Proposal 5. There is no need to specify whether SRAP ID is assigned per Remote UE, per L2 ID of said Remote UE, or per source/destination pair.